3GPP TSG-SA1 Meeting #88	S1-193140

# Reno, Nevada, United States, 18th Nov 2019 - 22nd Nov 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.866 | CR | 0005 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Aggregation for IoT device |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NOVAMINT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_REFEC |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This contribution proposes to new use cases related to improve battery efficiency by allowing aggregation for IoT devices. This use case was partly discussed at the SA1#87 meeting but it was for a broader scope (for all delay tolerant traffic - not only for IoT devices) and was not aggreed based on the fact it was necessary to completely define delay tolerant traffic and on assumptions on the complexity it may derive. This contribution limits this to IoT devices as it is crucial for IoT devices used as relay to be able to aggregate traffic for energy efficiency. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new use case & new requirements in consolidated functional requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 5G system may not provide a solution that is battery efficient for IoT devices |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New 5.x & 8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ********************* First Change *************************

## 5.X Battery efficiency enhancements for IoT use case

### 5.X.1 Description

IoT-related use cases pose unique challenges to the 5G system especially when the deployments expand to   areas where the cellular (5G) coverage is limited. At the same time, there is an expectation that large deployments can rely on delay-tolerance of some IoT communications to realize power savings both at the sensors and at all the communication devices. Delay-tolerant data in IoT allows for the periodic or semi-periodic intervals at which data is collected to be only loosely-coupled to the Core Network reporting intervals when the data is transmitted over multiple hops. This in turn allows for battery consumption optimizations for the cases in which the data collection intervals cannot be well synchronized. In the shipping container use case in clause 5.2 data is collected from sensors on the tracked product and within the containers by battery powered IoT UEs. This data is used for individual product and container tracking.  The battery powered IoT UE in each container is capable of relaying data from other containers in order to address the coverage challenges detailed in clause 5.2.

In addition to shipping, the maritime transportation company offers data collection services for oceanographic studies from sensors installed on the ship. Without the metal containers, the environmental sensors can communicate with the gateway which includes a 5G communication module communicating to terrestrial and satellite access networks. When the metal containers are on the ship, the relaying capabilities of the container UEs are used for the environmental sensor data in addition to the container tracking data.

All data collected is delay tolerant and can be forwarded at appropriate times to optimize a UE’s battery consumption. The relay UEs consider the delay tolerance of data to determine when bundling of the data transmissions towards the network can be employed for power saving purposes. While in some deployments this bundling of transmissions is achieved at application layer, in this deployment this is impractical as it requires explicit application-layer configuration of a relaying application. As an alternative to scheduling the bundling of the transmissions at the application level, being able to turn on/off bundling of transmissions enables IoT deployments to offer relaying services more dynamically and to a broader customer base.

### 5.X.2 Pre-conditions

Container-dedicated UEs are battery-powered, are capable of relaying the data from external sensors and may also have built-in sensors. The data to be relayed is delay tolerant and reported at various periodic intervals. The system supports connections for delay tolerant data with E2E latency of up to several days. The system also supports UEs to request delay tolerant communication on a per individual communication (e.g. 5G QoS flow).

### 5.X.3 Service Flows

Figure 5.X.3-1 illustrates container-dedicated relay UEs that transmit data received from various sensors, other relays or collected by their own sensors.

Figure 5.X.3-1: Shipping Container use case for delay tolerant data

Data from the sensors within each container is collected by the container-dedicated relay UEs and relayed to an upstream relay UE within the relay chains (depicted by red dashed lines). These relay chains are also used to transmit data collected by sensors embedded on the relay UE themselves.

In addition, data from oceanographic sensors installed on the ship is also relayed via relay chains (depicted by green lines) via the same container-dedicated relay UEs.

Relay-to-network UEs transmit data which is generated or received at various periodic interval sets (e.g. every two to three hours), for different purposes (tracking, oceanographic studies). The figure also depicts data along the relay chain path for each of these purposes. The relay UEs consider the delay-tolerance of the data to determine relaying intervals (e.g. once a day during maritime transportation, twice a day during land transportation) so that bundling or aggregating the data reduces overall transmit time and power.

The timing of the normal data transmissions is managed such that the transmissions are bundled or aggregated at the relay-to-network UE level, are eventually received by the Gateway and re-transmitted over satellite or terrestrial access networks to different application servers.

### 5.x.4	Post-conditions

All IoT devices have provided their data and the data of other IoT devices with minimal impacts on their battery and without impact on the service.

### 5.x.5	Existing features partly or fully covering the use case functionality

Editor’s Note:	The existing features that are not related to the relay feature to support this use case can be found in the TR 22.836 (Study on Asset Tracking Use Cases).

The 5G system shall support an IoT device acting as a relay for another IoT device.

### 5.x.6	Potential New Requirements needed to support the use case

[PR 5.x.6-1] The 5G system shall be able to support an IoT device used as UE-to-network relay UE to aggregate transmission of multiple IoT transmissions for energy efficiency purposes.

NOTE:	It is assumed the data that are aggregated are delay tolerant and it is the responsibility of the IoT devices owner to allow aggregation or not for its data traffic.

[PR 5.x.6-2] The 5G system shall support a mechanism to enable/disable aggregation of IoT transmissions at intermediate UE-to-network relay UEs.

[PR 5.x.6-3] The 5G system shall be able to provide a mechanism to identify that an IoT device is used as UE-to-network relay UE and is aggregating traffic.

[PR 5.x.6-4] The 5G system shall be able to identify all UE-to-network relay UEs that have been involved in an indirect network connection.

## ********************* End of First Change *************************

## ********************* Second Change *************************

## 8.1	Functional new requirements

### 8.1.1	General

[PR 8.1.1-1] The 5G system shall support the relaying of traffic between a remote UE and a gNB using one or more UE-to-Network Relay UEs.

[PR 8.1.1-2] The 5G system shall support different traffic flows of a remote UE to be relayed via different Indirect 3GPP Communication paths.

[PR 8.1.1-3] The 5G system shall enable the network operator to define the maximum number of hops supported when using UE-to-Network Relay UEs.

[PR 8.1.1-4] The 5G system shall be able to provide indication to a remote UE (alternatively, an authorized user) on the quality of currently available Indirect 3GPP Communication paths.

[PR 8.1.1-5] The 5G system shall be able to identify all UE-to-network relay UEs that have been involved in an indirect network connection.

### 8.1.2	Relay Selection

[PR 8.1.2-1] The 3GPP system shall support selection and reselection of a UE-to-Network Relay UE based on a combination of different criteria e.g.,

-	the characteristics of the traffic that is intended to be relayed (e.g. expected message frequency and required QoS),

-	the subscriptions of UE-to-Network Relay UEs and remote UE,

-	the capabilities/capacity/coverage when using the UE-to-Network Relay UE,

-	the QoS that is achievable by selecting the UE-to-Network Relay UE,

-	the power consumption required by UE-to-Network Relay UE and remote UE,

-	the pre-paired UE-to-Network Relay UE,

-	the 3GPP or non-3GPP access the UE-to-Network Relay UE uses to connect to the network,

-	the 3GPP network the UE-to-Network Relay UE connects to (either directly or indirectly),

-	the overall optimization of the power consumption/performance of the 3GPP system, or

-	battery capabilities and battery lifetime of the UE-to-Network Relay UE and the remote UE.

NOTE 1:	Reselection may be triggered by any dynamic change in the selection criteria, e.g. by the battery of a UE-to-Network Relay UE getting depleted, a new relay capable UE getting in range, a remote UEs requesting additional resources or higher QoS, etc.

### 8.1.3	Permission and Authorization

[PR 8.1.3-1] The 5G system shall enable the network operator to authorize a UE to use Indirect 3GPP Communication. The authorization shall be able to be restricted to using only UE-to-Network Relay UEs belonging to the same network operator. The authorization shall be able to be restricted to using only UE-to-Network Relay UEs belonging to the same application layer group.

[PR 8.1.3-2] The 5G system shall enable the network operator to authorize a UE to relay traffic as UE-to-Network Relay. The authorization shall be able to be restricted to relaying only for remote UEs belonging to the same network operator. The authorization shall be able to be restricted to relaying only for remote UEs belonging to the same application layer group.

[PR 8.1.3-3] The 5G system shall support a mechanism for an end user to provide/revoke permission to an authorized UE to act as a UE-to-Network Relay UE.

[PR 8.1.3-4] The 5G system shall provide a suitable API by which an authorized 3rd party shall be able to authorize (multiple) UEs under control of the 3rd party to act as a UE-to-Network Relay UE or remote UE.

[PR 8.1.3-5] The 5G system shall provide a suitable API by which an authorized 3rd party shall be able to enable/disable (multiple) UEs under control of the 3rd party to act as a UE-to-Network Relay UE or remote UE.

### 8.1.4	Services and Service Continuity

[PR 8.1.4-1] A 5G system shall support all types of data e.g., voice, data, multimedia, MCX for Indirect 3GPP communication.

[PR 8.1.4-2] The 5G system shall maintain service continuity of Indirect 3GPP Communication for a remote UE when the communication path to the network changes (i.e. change of one or more of the UE-to-Network Relay UEs, change of the gNB).

### 8.1.5	IoT devices and relays

[PR 8.1.5-1] The 5G system shall be able to support an IoT device used as UE-to-network relay UE to aggregate transmission of multiple IoT transmissions for energy efficiency purposes.

NOTE:	It is assumed the data that are aggregated are delay tolerant and it is the responsibility of the IoT devices owner to allow aggregation or not for its data traffic.

[PR 8.1.5-2] The 5G system shall support a mechanism to enable/disable aggregation of IoT transmissions at intermediate UE-to-network relay UEs.

[PR 8.1.5-3] The 5G system shall be able to provide a mechanism to identify that an IoT device is used as UE-to-network relay UE and is aggregating traffic.

## ********************* End of Second Change *************************