3GPP TSG-SA1 Meeting #88	S1-193144

# Reno, Nevada, United States, 18th Nov 2019 - 22nd Nov 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.836 | CR | 0002 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modification of KPI's |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NOVAMINT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5G_ATRAC |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | KPIs to be revised to be adapted to expectations |  |  |  |  |  |  |  |  |
| Summary of change: |  | Revision of KPIs for all use cases and final table |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No clear KPIs for next phases groups |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.7, 5.2.7, 5.3.7 & 6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ********************* First Change *************************

### 5.1.7	Potential Key Performances Requirements

The KPIs attached to this use case are listed below:

| Battery Life Expectancy (note 1) | Typical Message size | Maximum Message size | Typical Frequency (number of messages per day) | Typical Battery Capacity | Device density |  |
| --- | --- | --- | --- | --- | --- | --- |
| 7 years | 200 bytes | 2500 bytes | 12 | 41,76 Wh | 1,4 devices / m3 |  |
| NOTE 1: Battery life expectancy is to be assumed in all coverage conditions and is based on typical message size value and typical frequency NOTE 2: A large containership with a mix of 20 ft and 40 ft containers is assumed. NOTE 3: All the values in this table are example values and not strict requirements. |  |  |  |  |  |

## ********************* End of First Change *************************

## ********************* Second Change *************************

### 5.2.7	Potential Key Performances Requirements

The KPIs attached to this use case are listed below:

| Battery Life Expectancy (note 1) | Typical Message size | Maximum Message size | Typical Frequency (number of messages per day) | Typical Battery Capacity | Device density |
| --- | --- | --- | --- | --- | --- |
| 10 years | 200 bytes | 2500 bytes | 12 | 93,6 Wh | 0,3 devices / m2 |
| NOTE 1: Battery life expectancy is to be assumed in all coverage conditions and is based on typical message size value and typical frequency NOTE 2: All the values in this table are example values and not strict requirements. |  |  |  |  |  |

## ********************* End of Second Change *************************

## ********************* Third Change *************************

### 5.3.7	Potential Key Performances Requirements

The KPIs attached to this use case are listed below:

| Battery Life Expectancy (note 1) | Typical Message size | Maximum Message size | Typical Frequency (number of messages per day) | Typical Battery Capacity | Device density |  |
| --- | --- | --- | --- | --- | --- | --- |
| 7 years | 50 bytes | 300 bytes | 48 | 14,4 Wh | 4 devices/ m2 |  |
| NOTE 1: Battery life expectancy is to be assumed in all coverage conditions and is based on typical message size value and typical frequency NOTE 2: All the values in this table are example values and not strict requirements. |  |  |  |  |  |

## ********************* End of Third Change *************************

## ********************* Fourth Change *************************

## 6.2	Performances Requirements

|  | Scenario | Battery Life Expectancy (note 1) | Typical Message size | Maximum Message size | Typical Frequency (number of messages per day) | Typical Battery Capacity | Device density |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Containers (note 2) | 7 years | 200 bytes | 2500 bytes | 12 | 41,76 Wh | 1,4 devices / m3 |  |
| 2 | Wagons | 10 years | 200 bytes | 2500 bytes | 12 | 93,6 Wh | 0,3 devices / m2 |  |
| 3 | Pallets | 7 years | 50 bytes | 300 bytes | 48 | 14,4 Wh | 4 devices/ m2 |  |
| NOTE 1: Battery life expectancy is to be assumed in all coverage conditions and is based on typical message size value and typical frequency NOTE 2: A large containership with a mix of 20 ft and 40 ft containers is assumed. NOTE 3: All the values in this table are targeted values and not strict requirements. |  |  |  |  |  |  |  |

## ********************* End of Fourth Change *************************