3GPP TSG-SA WG1 Meeting #88 	S1-193168

Reno, Nevada, USA, 18 - 22 November 2019	(revision of S1-19xxxx)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.104 | CR | 0035 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Derivation of communication service availability and reliability from network performance metrics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCAV |  |  |  |  |  |  | Date: |  | 2019-11-08 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The relationship between communication service reliability and availability requirements and network-level reliability metrics needs to be described. An attempt to describe this relationship was made in clause 5.1 but it was based on the unrealistic assumption of uncorrelated packet errors. A more accurate model is required. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Table 5.1-1 and related text is removed. A new Annex C.x is introduced, which presents a translation function between communication service availability and reliability on the one hand, and per packet network level performance in the other hand. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be no reasonable way to translate communication service availability and reliability requirements into network-level reliability requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.1, new clause C.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Start of the 1st change --------------------------------------------------------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system".

[3]	IEC 61784-3: "Industrial communication networks – profiles – part 3: functional fieldbuses – general rules and profile definitions".

[4]	BZKI, "Aspects of dependability assessment in ZDKI", June 2017.

[5] 	BZKI, "Requirement Profiles in ZDKI", 2017.

[6]	IEC 61158: "Industrial communication networks – fieldbus specification", 2014.

[7]	IEC 61907, "Communication network dependability engineering".

[8]	Richard C. Dorf and Robert H. Bishop, "Modern Control Systems", Pearson, Harlow, 13th Edition, 2017.

[9]	Ernie Hayden, Michael Assante, and Tim Conway, "An Abbreviated History of Automation & Industrial Controls Systems and Cybersecurity", SANS Institute, https://ics.sans.org/media/An-Abbreviated-History-of-Automation-and-ICS-Cybersecurity.pdf {accessed: 2017-05-23}, 2014.

[10]	IEC 61512 "Batch control - Part 1: Models and terminology".

[11]	RESERVE project, Deliverable D1.3, ICT Requirements, 
http://www.re-serve.eu/files/reserve/Content/Deliverables/D1.3.pdf, September 2017.

[12]	RESERVE project, Deliverable D1.2, Energy System Requirements 
http://www.re-serve.eu/files/reserve/Content/Deliverables/D1.2.pdf, September 2017.

[13]	G. Garner, "Designing Last Mile Communications Infrastructures for Intelligent Utility Networks (Smart Grids)", IBM Australia Limited, 2010.

[14]	B. Al-Omar, B., A. R. Al-Ali, R. Ahmed, and T. Landolsi, "Role of Information and Communication Technologies in the Smart Grid", Journal of Emerging Trends in Computing and Information Sciences, Vol. 3, pp. 707-716, 2015.

[15]	H. Kagermann, W. Wahlster, and J. Helbig, "Recommendations for implementing the strategic initiative INDUSTRIE 4.0", Final report of the Industrie 4.0 working group, acatech – National Academy of Science and Engineering, Munich, April 2013.

[16]	IEC 62443-3-2: "Security for industrial automation and control systems - Part 3-2: Security risk assessment and system design", in progress.

[17]	IEC 62657-2: "Industrial communication networks - Wireless communication networks - Part 2: Coexistence management", 2017.

[18]	IEC 62657-1: "Industrial communication networks – Wireless communication networks – Part 1: Wireless communication requirements and spectrum considerations".

[19]	IEEE Std 802.1Q "Media Access Control (MAC) Bridges and Virtual Bridge Local Area Networks".

NOTE: IEEE Std 802.1Qbv-2015 "Enhancements for Scheduled Traffic" has been included into IEEE Std 802.1Q-2018.

[20] 	IEEE, Use Cases IEC/IEEE 60802, 2018.

[21] 	"IEEE Standard for Local and metropolitan area networks--Timing and Synchronization for Time-Sensitive Applications in Bridged Local Area Networks--Corrigendum 1: Technical and Editorial Corrections," IEEE Std 802.1AS-2011/Cor 1-2013 (Corrigendum to IEEE Std 802.1AS-2011), pp. 1-128, Sept 2013.

[22] 	"IEEE Standard for Local and metropolitan area networks--Timing and Synchronization for Time-Sensitive Applications," IEEE Std 802.1AS-Rev/D7.3, pp. 1-502, August 2018.

[23]	3GPP TS 22.289: "Mobile Communication System for Railways".

[24]	IEEE P802.1CS: “IEEE Draft Standard for Local and metropolitan area networks - Link-local Registration Protocol”

[25]	IEEE P802.1Qdd: “IEEE Draft Standard for Local and Metropolitan Area Networks--Bridges and Bridged Networks -- Amendment: Resource Allocation Protocol (RAP)”

[26]	IEC/IEEE 60802: "Time-Sensitive Networking Profile for Industrial Automation".

[27]	3GPP TS 23.501:  "System Architecture for the 5G System".

[28]	E. N. Gilbert, “Capacity of a Burst-Noise Channel,” Bell System Technical Journal, vol. 39, no. 5, pp. 1253–1265, 1960.

[29]	E.  O.  Elliott, “Estimates of Error Rates for Codes on Burst-Noise Channels,” Bell System Technical Journal, vol. 42, no. 5, pp. 1977–1997, 1963

----------------------------------------------------- End of the 1st change --------------------------------------------------------------

----------------------------------------------------- Start of the 2nd change --------------------------------------------------------------

## 5.1	Overview

There are two fundamental perspectives concerning dependable communication in 5G systems: the end-to-end perspective of the communication services and the network perspective (see Figure 5.1-1).

Figure 5.1-1: Network perspective of 5G system

The Communication Service in Figure 5.1-1 may be implemented between a UE on the one side and a network server on the other side, or between a UE on the one side and a UE on the other side.

In some cases, a local approach (e.g. network edge) is preferred for the communication service on the network side in order to reduce the latency or to keep sensitive data in a non-public network on the factory site.

The tables in Clauses 5.2 through 5.5 below provide sets of requirements where periodicity and determinism are critical to meeting cyber-physical control application needs in various vertical scenarios. While many use cases have similar KPI values in some cases, the important distinction is that in order to meet the needs of different verticals and different uses, the 5G system will need to be sufficiently flexible to allow deployment configurations that can meet the different sets of KPIs specific to each use.

Clock synchronisation is needed in many "vertical" use cases. The requirements and tables in Clause 5.6 provide specific criteria for managing time sensitive communications in an industrial environment.

High accuracy positioning is becoming essential for Factories of the Future. The reason for this is that tracking of mobile devices as well as mobile assets is becoming increasingly important in improving processes and increasing flexibility in industrial environments, Clause 5.7 provides positioning requirements for horizontal and vertical accuracy, availability, heading, latency and UE speed in an industrial use case scenario.

Communication Service Availability is considered an important service performance requirement for cyber-physical applications, especially for applications with deterministic traffic. Although the communication service availability cannot be directly measured by the network, it is dependent on the application survival time, end-to-end latency and reliability requirement for the 5G system. The interrelation between network, communication service and application is described in Annex C.3, while a translation function between network layer metrics and communication service availability and reliability is presented in Annex C.6.

----------------------------------------------------- End of the 2nd change --------------------------------------------------------------

----------------------------------------------------- Start of the 3rd change --------------------------------------------------------------

# C.x	Derivation of communication service availability and reliability from network performance metrics

This section presents a mapping function between communication service availability and reliability requirements and network performance metrics.

Throughout this section the term “packet loss” or “packet failure” is used to refer to an event in which a protocol data unit (PDU), e.g., an IP packet containing sensor updates, is not successfully delivered within a specified deadline to the target PDU layer (e.g., UPF). It is assumed that an application-level message fits in one packet on the network level.

## C.x.1 Overview

Communication Service performance metrics (i.e., availability and reliability) are considered as important service performance requirements for cyber physical applications, especially for the applications with deterministic traffic. In fact, all the requirements in Clauses 5.2-5.5 are defined on communication service level. Although the communication service availability and reliability cannot be directly measured by the network, they are the combination of survival time, availability, and reliability on network level. This clause describes a method resulting in closed form equations translating network layer metrics to communication service availability and reliability. With this method, the fulfilment of communication service requirements could be assessed by solely performing network layer measurements. Figure C.x.1-1 illustrates a functional overview of the method.

Figure C.x.1-1: Functional overview of the mapping between communication service performance and network performance metrics

## C.x.2 Terms

This clause defines supplementary terms used for the derivation of a mapping function between communication service performance and requirements on the one hand, and network level performance and requirements on the other hand.

Survival time (as described in Clause 3.1) is the time that an application consuming a communication service may continue without an anticipated message. For periodic traffic, survival time can be expressed as maximum number of lost packets (denoted here as Nsv, where Nsv=Survival Timecycle Time).

Communication service unavailability: percentage value of the amount of time the end-to-end communication service does not fulfil the agreed QoS, divided by the amount of time the system is expected to deliver the end-to-end service according to the specification in a specific area. Therefore,

Communication service unavailability=1- Communication service availability,

where communication service availability is as defined in Clause 3.1

(Per packet) Network availability: percentage value of the amount of time the network is able to deliver packets within the agreed delay budget, divided by the amount of time the system is expected to deliver the end-to-end service according to the specification in a specific area. Note that for the purpose of this analysis the network is considered unavailable after the first packet loss. Assuming periodic traffic, it can be shown that:

Network availability=1-p≈1-EPER

where p is the packet error probability which is the expected value of packet error rate (PER). The definitions of p and PER can be found in [27].

Network mean time between failures (MTBFN): mean time between packet failures (MTBF) which represents the mean value of how long the network is available before it becomes unavailable (on a per-packet basis). It is considered as an indicator for reliability.

Network mean time to repair (MTTRN): mean value of how long the network is unavailable (on a per-packet basis) before it becomes available again, or in other words, the average number of consecutive packet failures.

MTTRN, can be derived from p and network reliability (MTBFN) as:

MTTRN=MTBFN×p1-p

## C.x.3 Mapping function between communication service and network

To achieve the objective, and assuming periodic traffic, a Markov chain is applied, which extends the original Gilbert-Elliot Markov model [28, 29] to keep track of burst errors.  Figure C.x.3-1 illustrates, the space state is partitioned into Nsv+2 states. The first state, UN, represents the time that network is available on per-packet basis, i.e. it delivers packets as expected. While network is available, the first failure happens with the probability of MTBFN-1.The Nsv middle states keep track of Nsv consecutive failed packets during which the communication service is still available since it is within the specified survival time. It is assumed that transition probability between the Nsv middle states is constant. The far-right state, D, represents the time that communication service becomes unavailable which occurs after Nsv consecutive packet failure events.

Figure C.x.3-1: Markov chain for representation of burst error length

Since the proposed Markov chain is irreducible and aperiodic, it has a unique equilibrium distribution which can be derived by Markov properties using transition probabilities shown in Figure C.x.3-1. In this case, the steady state probability of UN and D represent the (per packet) network availability and communication service unavailability, respectively. In this case, the mean number of transitions per time unit to state D can be derived as

mean number of transitions to D per time unit=PrNsv×1-MTTRN-1,

where PrNsv denotes the steady-state probability of state Nsv. Accordingly, one over mean number of transitions per time unit is the mean time to have two consecutive transitions to state D, and therefore, multiplying this mean time to the communication service level availability results in the mean time period during which application is available. Hence, the reliability on communication service level can be calculated as

Communication service reliability=1-PrDmean number of transitions to D per time unit ,

where PrD is the steady-state probability of state D.

## C.x.4 Numerical Analysis

Figure C.x.4-1 and C.x.4-2 illustrate examples of the mapping between network parameters and communication service requirements when Nsv is 3 and 1 respectively. In Figure C.x.4-1-a and C.x.4-2-a, the unavailability (or 1 - availability) of the communication service is shown based on network level packet error rate and mean time to repair (MTTRN). The communication service availability requirements of 4 nines, 5 nines, 6 nines, and 9 nines are also drawn as horizontal lines. Note that the network mean time between failures (MTBFN) can also be calculated based on PER and MTTRN. The reliability of the communication service, which is derived based on network level MTTRN and PER, is presented in Figure C.x.4-1-b and C.x.4-2-b. The communication service reliability requirements of 1 week, 1 month, 1 year, and 10 years are also shown. It is observed that higher reliability and availability requirements put tighter requirements on the network level MTTRN. For instance, for Nsv=3 and PER of 10-6, the average number of consecutive packet failures (i.e. MTTRN) should be lower than 1.015 to be able to fulfil 10 years reliability requirement (refer to Figure C.x.4-1-b). However, if application survival time is only 1 cycle (Nsv=1), communication service reliability of 10 years would require network PER better than 10-8, as can be observed from Figure C.x.4-2-b.

b)

a)

Figure C.x.4-1: Impact of network level parameters on communication service availability and reliability when Nsv= 3

b)

a)

Figure C.x.4-2: Impact of network level parameters on communication service availability and reliability when Nsv= 1

----------------------------------------------------- End of the 3rd change --------------------------------------------------------------