3GPP TSG-SA WG1 Meeting #88 	S1-193169

Reno, Nevada, USA, 18 - 22 November 2019	(revision of S1-19xxxx)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.832 | CR | 0006 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Survival time vs message error rate |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_eCAV |  |  |  |  |  |  | Date: |  | 2019-11-08 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | If the survival time is greater than 0, the communication service availability is 100% even if every second message is incorrectly received or lost. This was however not the intention when survival time was defined. Therefore, a parameter specifying an overall limit on the number of allowed lost messages is needed. As a complement to the application requirements considered in 22.104, we propose to add “message error rate” as a characteristic parameter describing the requirements of cyber-physical control applications in vertical domains. The message error rate specifies an overall limit on the number of allowed lost messages even when the number of consecutive lost messages is fewer than what the survival time allows without application-level failure. As the message error rate may be significantly higher than the probability of application-level failure, the additional information that the message error rate provides can help to significantly increase the spectral efficiency of the communication system. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Message error rate is defined and the advantage of adding this 5G QoS characteristics is explained. In addition, the usage of packet and message is reconsidered to make sure that message is consistently used for application-level information blocks. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An unwanted behaviour of the 5G system will be implicitly allowed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Start of the 1st change --------------------------------------------------------------

## 4.2	Discussion on network performance requirements

The key performance requirements for cyber-physical control applications in vertical domains are specified in TS 22.104, including the new KPIs in addition to the usual KPIs (i.e., end-to-end latency, message size, service bit rate, and transfer interval):

Communication service availability - This KPI indicates if the communication system works as contracted ("available"/"unavailable" state). The communication system is in the "available" state as long as the availability criteria for transmitted messages are met. The service is unavailable if the messages received at the target are impaired and/or untimely (e.g. update time > stipulated maximum), resulting in survival time being exceeded.

Communication service reliability - Mean time between failures is one of the typical indicators for communication service reliability. This KPI states the mean value of how long the communication service is available before it becomes unavailable.

Survival time - The maximum survival time indicates the time period the communication service may not meet the application's requirement before there is a failure on the application layer, such that the communication service is deemed to be in an unavailable state.

Meanwhile the 5G QoS characteristics are specified in TS 23.501 to describe the packet forwarding treatment that a QoS Flow receives edge-to-edge between the UE and the UPF. The most relevant performance characteristics to the identified key performance requirements are:

Packet Delay Budget - The Packet Delay Budget (PDB) defines an upper bound for the time that a packet may be delayed between the UE and the UPF that terminates the N6 interface. For a certain 5QI the value of the PDB is the same in UL and DL. In the case of 3GPP access, the PDB is used to support the configuration of scheduling and link layer functions (e.g. the setting of scheduling priority weights and HARQ target operating points).

Packet Error Rate - The Packet Error Rate (PER) defines an upper bound for the rate of PDUs (e.g. IP packets) that have been processed by the sender of a link layer protocol (e.g. RLC in RAN of a 3GPP access) but that are not successfully delivered by the corresponding receiver to the upper layer (e.g. PDCP in RAN of a 3GPP access) within the packet delay budget. Thus, the PER defines an upper bound for a rate of packet losses. The purpose of the PER is to allow for appropriate link layer protocol configurations (e.g. RLC and HARQ in RAN of a 3GPP access). For every 5QI the value of the PER is the same in UL and DL.

Issues on network performance requirements that are addressed in this study are listed as follow:

-	What are the SA1 specified KPIs (for example ‘communication service reliability’) that are related to Packet Error Rate?

As a complement to ‘communication service reliability’ and ‘communication service availability’, message error rate can be defined:

Message error rate – percentage value of the number of application level messages incorrectly received or lost, divided by the total number of application level messages sent from a source to a destination.

Message error rate can be used to indicate the significance of individual message losses which for many of the industrial applications differs from the significance of losing several consecutive messages (message is ‘lost’ if it is not delivered intact within PDB). For example, loss of a single message may only slightly reduce the quality of experience of an industrial application (e.g., precision of a motion control application), while loss of several consecutive messages is considered as communication service unavailability potentially resulting in an emergency stop in the application. Message error rate is directly related to ‘communication service reliability’ and ‘communication service availability’ only in the special case where ‘failure’ of the communication system is specified to be loss of a single message (i.e. survival time is zero) for periodic deterministic traffic.

-	Are ‘communication service availability’ and ‘communication service reliability’ KPIs for 3GPP 5G system design, or parameters that are related to Service Level Agreement (i.e., relating to network deployment)?

‘Communication service availability’ and ‘communication service reliability’ can be considered for 3GPP 5G design as they indicate the significance of how message losses are distributed in time domain. In addition, message error rate is required for 3GPP 5G system design, since the probability of individual message loss may also impact the quality of experience of an industrial application. For example, if 5G system design considers avoidance of multiple consecutive message losses with higher priority than individual message losses, this may result in a system design that improves the quality of experience of the industrial applications. Both availability and reliability must always be specified with a context.

-	What is the meaning of ‘communication service reliability: mean time between failures’? What does the term ‘failure’ exactly mean in this KPI? Can it be used as a KPI for 3GPP 5G system design? Has network maintenance time been taken into account? What is the relationship between ‘communication service reliability: mean time between failures’ and network reliability (that is directly related to Packet Error Rate)?

‘Communication service reliability’ can be quantified as the mean time between application layer failures. Failure refers to the event when system becomes ‘unavailable’ considering the application specific requirements. For many of the industrial applications, the communication service is considered unavailable if ‘survival time’ is exceeded. For applications that have survival time equal to zero, any lost message triggers this unavailability, while for applications with non-zero survival time only two or more consecutive lost messages will trigger unavailability (depending on the agreed traffic periodicity and length of the survival time). The communication service is considered available again when it successfully delivers a packet within the delay constraints.

Communication service availability can be derived also from ‘communication service reliability’ which is quantified as ‘mean-time-to-fail’ (MTTF) and ‘communication service maintainability’ which can be quantified as mean-time-to-repair (MTTR);

‘communication service availability’ = MTTF / (MTTF + MTTR)

The ‘mean-time-to-repair’ refers to the average length of the unavailability periods in time.

-	Is there any new attribute required to specify 5G QoS characteristics to fulfil the requirements for cyber-physical control applications in vertical domains?

The performance requirements provided in 22.261 and 22.104 need to be complemented with a message error rate requirement to fully characterise the requirements for cyber-physical control applications in vertical domains.

- 	Is ‘survival time’ a key indicator to be considered for 3GPP 5G system design, or an application-specific parameter that cannot reliably and systematically be used to characterize the performance and/or properties of the communication service?

To efficiently fulfil requirements of cyber-physical control applications the message error rate, the communication service availability and the survival time should be included in 5G QoS characteristics. The message error rate specifies the allowed probability of individual message errors, while communication service availability combined with survival time specifies the allowed probability of communication service failure. The message error rate may be significantly higher than the probability of communication service failures, since the impact on the application of an individual message error is much lower. If these characteristics are not included, the 5G system needs to over-provision the PER targets, which may lead to significant reduction in system capacity and/or reduce the communication service reliability and availability (e.g., in the case when there is resource conflict between two URLLC service flows).

----------------------------------------------------- End of 1st change --------------------------------------------------------------