3GPP TSG-SA WG1 Meeting #88	S1-193377

Reno, Nevada, US, 18-22 November 2019    			          (revision of S1-193043)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.125 | CR | 0026 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KPIs for UAV services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom, Huawei, ZTE, CATT, InterDigital, Futurewei, Verizon, Intel, Airbus, Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | EAV |  |  |  |  |  |  | Date: |  | 2019-11-08 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | 1. Based on the discussion in the study report TR 22.829, EAV introduced some KPIs for UAVs. These KPIs are currently related to the networks for providing services for UAVs. These KPIs include the uplink and downlink data rate between UAVs and networks, as well as the corresponding time delay requirements, etc. 2. If UAV-related KPI requirements are added to 22.215, some references need to be added. 3. Merge the command control KPI CR to this document and keep the context consistent. Add the command control KPI related terms. 4. Add and modify the requirements, according to the comments of the aviation, which are mainly based on the input of GUTMA. |  |  |  |  |  |  |  |  |
| f |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add a new section for service-related KPI requirements with corresponding definition and references. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of the services performance KPIs in R17. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 5.1 , 7 (new) |  |  |  |  |  |  |  |  |
| 5.1 |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**********************FIRST CHANGE***************************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	FAA UTM Concept of Operations v1.0, Foundational Principles, Roles and Responsibilities, Use Cases and Operational Threads https://utm.arc.nasa.gov/docs/2018-UTM-ConOps-v1.0.pdf

[3]	FAA Remote Identification, https://www.faa.gov/uas/research_development/remote_id/

[4]	3GPP TS 22.261: "Service requirements for the 5G system; Stage".

**********************SECOND CHANGE**************************

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Command and Control (C2) Communication: the user plane link to deliver messages with information of command and control for UAV operation from a UAV controller or a UTM to a UAV.

Unmanned Aerial System (UAS): Composed of Unmanned Aerial Vehicle (UAV) and related functionality, including command and control (C2) links between the UAV and the control station, the UAV and the network, and for remote identification. An UAS may comprise of a UAV and a UAV controller.

Unmanned Aerial System Traffic Management (UTM): a set of functions and services for managing a range of autonomous vehicle operations.

UAV controller: The UAV controller of a UAS enables a drone pilot to control an UAV. BVLOS	Beyond Visual Line of Sight

UxNB: radio access node on-board UAV. It is a radio access node providing connectivity to UEs, which is carried in the air by an Unmanned Aerial Vehicle (UAV).

Above ground level (AGL): In the context of a UAV it is the UAV altitude referenced to ground level in the vicinity.

**********************THIRD CHANGE********************************

## 5      Requirements for Remote Identification of UAS

## 5.1	General

The 3GPP system should enable an MNO to augment the data sent to a UTM with the following: network-based positioning information of UAV and UAV controller.

NOTE: 	This augmentation may be trust-based (i.e. the MNO informs the UTM that the UAV position information is trusted) or it may be additional location information based on network information, such as  OTDOA, cell coordinates, synchronization source, etc.

NOTE: 	This requirement will not be applied to the case which the UAS and UTM has direct control communication connection without going through MNO.

The 3GPP network should be able to provide supplement location information of UAV and its controller to a UTM.

NOTE: 	This supplement may be trust-based (i.e. the MNO informs the UTM that the UAV position information is trusted) or it may be additional location information based on network information, such as OTDOA, cell coordinates, synchronization source, etc.

*******************FOURTH CHANGE********************************

# 7	Performance requirements

## 7.1	KPIs for services provided to the UAV applications

The 5G system shall be able to provide unmanned aerial vehicle with the service performance requirements reported in Table 7.1-1.

UAV originated QoS in the table refers to the QoS of Uplink data (e.g. from UAV to the network side). UAV terminated QoS is the QoS of downlink data (e.g. from the network side to UAV).

The data transmitted by the 5G system includes data collected by hardware devices installed on UAV such as cameras, e.g. pictures, videos and files. It is also possible to transmit some software calculation or statistical data, e.g. UAV management data. The service control data of transmitted by the 5G system may be based on application triggers, such as switch, rotation, promotion and demotion control of equipment on UAV. Various UAV applications may require different uplink and downlink QoS at the same time. The 5G system may simultaneously provide services to other users on the ground (e.g., the KPIs for rural and urban scenarios as defined in 7.1 of TS 22.261 [z]) in the same area without service degradation.

Table 7.1-1 KPIs for services provided to the UAV applications

| Use case | Services | Data rate | End to end Latency | Altitude AGL | service area (note 4) |
| --- | --- | --- | --- | --- | --- |
| 1 | 8K video live broadcast | 100Mbps UAV originated | 200ms | <100m | Urban, scenic area |
|  |  | 600Kbps UAV terminated | 20ms | <100m |  |
| 2 | Laser mapping/ HD patrol Note 7 | 120Mbps UAV originated Note 1 | 200ms | 30-300m | Urban, rural area, scenic area |
|  |  | 300Kbps UAV terminated | 20ms | 30-300m |  |
| 3 | 4*4K AI surveillance | 120Mbps UAV originated | 20ms | <200m | Urban, rural area |
|  |  | 50Mbps UAV terminated | 20ms | <200m |  |
| 4 | Remote UAV controller through HD video | >=25Mbps UAV originated (Note 3) | 100 ms | <300m | Urban, rural area |
|  |  | 300Kbps UAV terminated | 20ms | <300m |  |
| 5 | Real-Time Video | 0.06 Mbps w/o video UAV originated | 100 ms | - | Urban, rural, countryside |
| 6 | Video streaming | 4 Mbps for 720p video 9 Mbps for 1080p video UAV originated | 100 ms | - | Urban, rural, countryside |
| 7 | Periodic still photos | 1Mbps UAV originated; 1S; 0.1m; <120m; -; Urban, rural area. | 1s | <120m | Urban, rural, countryside |
| NOTE 1: The flight average speed is 60km/h. The KPI is referring to [w]. NOTE 2: The latency is the time of the 5G system provide higher accuracy location information of a UAV to a third party. NOTE 3: Referring to clause 5.2.2, the absolute flying speed of UAV in this service can be up to 160km/h. NOTE 4: The density of active UAV is 10/200km2. The maximum altitude is 300m. The flight average speed is 60km/h. |  |  |  |  |  |

## 7.2 KPIs for UAV command and control

UAVs may use a variety of flight command and control modes. Command and control (C2) communications refers to the two-way communication, which may include video, required to control the operation of the UAV itself. C2 messages may be communicated with the UAV controller, the UTM or both and may or may not be periodic. UAV controller and UTM communications may happen at essentially the same time with different required QoS. Any mission specific communication (e.g. HD video for area surveillance), if required, is additional. Different modes of control and their typical KPIs are listed in this clause below. The 5G system shall support UAV operation at altitudes of at least 120m / 400ft above ground level, e.g. the services should be provided and characterized up to 3000ft AGL.

Table 7.2-1 KPIs for command and control of UAV operation

| Control Mode | Function | Typical Message Interval | Max UAV ground speed | Typical message Size (note 1) | End to end Latency | Reliability (note 2) | Positive ACK (note 8) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Steer to waypoints (note 3) | UAV terminated C2 message | >=1s | 300km/h | 100byte | 1s | 99.9% | Required |
|  | UAV originated C2 message (note 4) | 1s |  | 84-140byte | 1s |  | 99.9% | Not Required |  |
| Direct stick steering (note 5) | UAV terminated C2 message | 40ms (note 6) | 60 km/h | 24byte | 40ms | 99.9% | Required |
|  | UAV originated C2 message (note 7) | 40ms |  | 84-140byte | 40ms | 99.9% |  |  | Not Required |
| Automatic flight on UTM (note 10) | UAV terminated C2 message | 1s | 300km/h | <10K byte | 5s (note 9) | 99.9% |  |  | Required |
|  | UAV originated C2 message | 1s (note 9) |  | 1500byte | 5s (note 9) | 99.9% |  |  | Required |
| Approaching Autonomous Navigation Infrastructure | UAV terminated C2 message | 500ms | 50km/h | 4kbyte | 10ms | 99% | Required |
|  | UAV originated C2 message | 500ms |  | 4kbyte | 140ms | 99.99% | Required |
|  | NOTE 1: Message size is at the application layer and excludes any headers and security related load. The numbers shown are typical as message size depends on the commands sent and is implementation specific. NOTE 2: Message reliability is defined as the probability of successful transmission within the required latency at the application layer while under network coverage. NOTE 3: Video is neither required nor expected to be used for steering in this mode. NOTE 4: It may be possible to transmit this message on an event driven basis e.g. approaching a geo fence. A status message may but is not required to be sent as a response to a control message. NOTE 5: A video feedback is required for this mode. The KPIs for video are defined in table 7.2-2. NOTE 6: UAVs on board controllers typically update at either 50Hz (20ms) or 25Hz (40ms). NOTE 7: A status message may but is not required to be sent as a response to a control message A 1Hz slow mode also exists. NOTE 8: Positive ACK is sent to the originator of the message (i.e. UAV controller and / or the UTM). The 5G system makes no assumption whether an appropriate ACK is sent by the application layer. NOTE 9: At the application level, the C2 communication between a UAV and UTM can be allowed to experience much longer traffic interruptions, e.g. timeouts of 30S on the uplink and 300S on the downlink. NOTE 10: This only represents periodic message exchange during a nominal mission in steady state. It does not represent unusual or aperiodic events such as conveying dynamic restrictions or a flight plan to the UAV on the downlink. |  |  |  |  |  |  |

Table 7.2-2 KPIs for video used to aid UAV control.

| Scenario (note 2) | Data rate | End to end Latency | Reliability (note 1) | Direction | Positive ACK required |
| --- | --- | --- | --- | --- | --- |
| VLOS (visual line of sight) | 2Mpbs at 480p, 30fps | 1s | 99.9% | Sent by UAV | Not Required |
| Non-VLOS | 4Mbps at 720p, 30fps | 140ms | 99.99% | Sent by UAV | Not Required |
| NOTE 1: Message reliability is defined as the probability of successful transmission within the required latency. NOTE 2: Maximum UAV speed is same as control mode of direct stick steering in 7.2-1 |  |  |  |  |  |

## 7.3    Positioning performance requirements

Table 7.3-1 below lists typical scenarios and the corresponding positioning requirements for horizontal and vertical accuracy, availability, heading, latency, and UE speed.

NOTE:	The column on "Corresponding Positioning Service Level in TS 22.261" maps the scenarios listed in Table 7.3-1 to the service levels defined in TS 22.261 [x].

Table 7.3-1: Positioning performance requirements

| Scenario | Accuracy (95 % confidence level) |  | Availability | Heading | Latency for position estimation of UE | UE Speed | Corresponding Positioning Service Level in TS 22.261 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Horizontal accuracy | Vertical accuracy |  |  |  |  |  |
| 8K video live broadcast | [0.5 m] | [1 m] | 99% |  | 1s | [<120 km/h] | 5 |
| Laser mapping/ HD patrol | [0.5 m] | [1 m] | 99% |  | 1s | [<120 km/h] | 5 |
| 4*4K AI surveillance | [0.1 m] |  |  |  |  | [<60 km/h] |  |
| Remote UAV controller through HD video | [0.5 m] | [1 m] | 99% |  | 1s | [<120 km/h] | 5 |
| Periodic still photos | [0.1 m] | [1 m] |  |  |  | [<60 km/h] |  |
| NOTE1: The positioning accuracy in this table is not related to navigation or safety. |  |  |  |  |  |  |  |

## 7.4  Other requirements

The 5G system shall support a mechanism to switch between C2 communication modes for UAS operation, e.g. from indirect C2 communication to direct C2 communication, and ensure the disconnect time is below the latency requirements.

The 3GPP system shall enable concurrent communications between the UAV-controller and UAV and between the UTM and the UAV that may require different KPIs.

The 3GPP system shall be capable of switching between the KPIs, as requested by the UAV-controller or the UTM, within [500ms].

**********************END OF CHANGE*************************