3GPP TSG-SA1 Meeting #90	S1-202046

Electronic Meeting, 18 - 22 May 2020	(revision of S1-20xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 21.905 | CR | <CR#> | rev | <Rev#> | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for UICC definition alignment to support Multi-USIM services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Thales |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2020/05/06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Multi USIM devices related service requirements in 3GPP TS 22.101 indicates that a UICC may be removable or not, and according to 3GPP TS 33.501, clause 6.1.1.1: “The USIM shall reside on a UICC. The UICC may be removable or non-removable.” Current definition only includes the removable UICC use case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the UICC definition to allow removable and non-removable UICC use cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The non-removable UICC use case is not taken into account in 3GPP specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 3	Terms and definitions

## 0-9

1.8V technology Smart Card: A Smart Card operating at 1.8V ± 10% and 3V ± 10%.

1.8V technology Terminal: A terminal operating the Smart Card - Terminal interface at 1.8V ± 10% and 3V ± 10%.

3GPP Generic User Profile (GUP): The 3GPP Generic User Profile is the collection of user related data which affects the way in which an individual user experiences services and which may be accessed in a standardised manner.

3GPP system: A telecommunication system conforming to 3GPP specifications, consisting of one or more 3GPP core networks, one or more 3GPP access networks (providing GSM/EDGE, UTRA, E-UTRA, or NR radio access), and/or non-3GPP access networks (such as WLAN), and User Equipment.

3GPP System core network: refers in this specification to an evolved GSM core network infrastructure.

3GPP System coverage: see coverage area.

3GPP System IC Card: An IC card (or 'smartcard') of defined electromechanical specification which contains at least one USIM.

3GPP System mobile termination: part of the 3GPP System Mobile Station which provides functions specific to the management of the radio interface (Um).

3GPP-WLAN Interworking: Used to generically refer to interworking between the 3GPP system and the WLAN family of standards.

3V technology Smart Card: A Smart Card operating at 3V± 10% and 5V ± 10%.

3V technology Terminal: A terminal operating the Smart Card - Terminal interface at 3V-± 10% and 5V ± 10%.

## A

A/Gb mode: mode of operation of the MS when connected to the Core Network via GERAN and the A and/or Gb interfaces.

[…]

## U

UE Service Capabilities: Capabilities that can be used either singly or in combination to deliver services to the user. The characteristic of UE Service Capabilities is that their logical function can be defined in a way that is independent of the implementation of the 3GPP System (although all UE Service Capabilities are of course constrained by the implementation of the 3GPP System). Examples: a data bearer of 144 kbps; a high quality speech teleservice; an IP teleservice; a capability to forward a speech call.

UICC: a physically secure device, an IC card (or 'smart card'), that may be inserted and removed from the terminal. It may contain one or more applications. One of the applications may be a USIM.

Universal Subscriber Identity Module (USIM): An application residing on the UICC used for accessing services provided by mobile networks, which the application is able to register on with the appropriate security.

Universal Terrestrial Radio Access Network (UTRAN): UTRAN is a conceptual term identifying that part of the network which consists of RNCs and Node Bs between Iu and Uu interfaces.

Usage Parameter Control (UPC): Set of actions taken by the network to monitor and control the offered traffic and the validity of the connection with respect to the traffic contract negotiated between the user and the network.

Uplink: An "uplink" is a unidirectional radio link for the transmission of signals from a UE  to a base station, from a Mobile Station to a mobile base station or from a mobile base station to a base station.

Uplink operating band: The part of the operating band designated for uplink.

Uplink Pilot Timeslot: Uplink part of the special subframe (for TDD operation)

Upper RF bandwidth edge: The frequency of the upper edge of the Base Station RF bandwidth, used as a frequency reference point for transmitter and receiver requirements

URA updating: URA updating is a family of procedures that updates the UTRAN registration area of a UE when a RRC connection exists and the position of the UE is known on URA level in the UTRAN.

User: An entity, not part of the 3GPP System , which uses 3GPP System services. Example: a person using a 3GPP System mobile station as a portable telephone.

User-network interface: The interface between the terminal equipment and a network termination at which interface the access protocols apply (source: ITU-T I.112).

User-user protocol: A protocol that is adopted between two or more users in order to ensure communication between them (source: ITU-T I.112).

User access or user network access: The means by which a user is connected to a telecommunication network in order to use the services and/or facilities of that network (source: ITU-T I.112).

User Equipment (UE): Allows a user access to network services. For the purpose of 3GPP specifications the interface between the UE and the network is the radio interface. A User Equipment can be subdivided into a number of domains, the domains being separated by reference points. Currently the User Equipment is subdivided into the UICC domain and the ME Domain. The ME Domain can further be subdivided into one or more Mobile Termination (MT) and Terminal Equipment (TE) components showing the connectivity between multiple functional groups.

In the context of Fixed Broadband Access to IMS, TISPAN defines the term UE in ETSI TR180 000 [5].

User Interface Profile: Contains information to present the personalised user interface within the capabilities of the terminal and serving network.

User Services Profile: Contains identification of subscriber services, their status and reference to service preferences.

UTRA Radio access mode: the selected UTRA radio access mode ie UTRA-FDD;UTRA-TDD.

UTRA-NTDD: Time Division Duplex UTRA access mode 1.28 Mcps option

UTRA-TDD:	Time Division Duplex UTRA Radio access mode (Includes UTRA-NTDD and UTRA-WTDD)

UTRA-WTDD: Time Division Duplex UTRA access mode 3.84 Mcps option

UTRAN access point: A conceptual point within the UTRAN performing radio transmission and reception. A UTRAN access point is associated with one specific cell, i.e. there exists one UTRAN access point for each cell. It is the UTRAN-side end point of a radio link.

UTRAN Registration Area: The UTRAN Registration Area is an area covered by a number of cells. The URA is only internally known in the UTRAN.

UTRAN Radio Network Temporary Identifier: The  U-RNTI is a unique UE identifier that consists of two parts, an SRNC identifier and a C-RNTI. U-RNTI is allocated to an UE having a RRC connection. It identifies the UE within UTRAN and is used as an UE identifier in cell update, URA update, RRC connection reestablishment and (UTRAN originated) paging messages and associated responses on the radio interface.

User Profile: Is the set of information necessary to provide a user with a consistent, personalised service environment, irrespective of the user's location or the terminal used (within the limitations of the terminal and the serving network).

Uu: The Radio interface between UTRAN and the User Equipment.

[…]