3GPP TSG-SA WG Meeting #90-e	S1-202085

Electronic Meeting, May. 18-22, 2020

Title:		Use Case for FS_EANS: Roaming Access to NS

Agenda Item:	4

Source:	Nokia, Nokia Shanghai Bell

Contact:	 Betsy Covell betsy.covell@nokia.com

Abstract: This contribution describes a potential use case for FS_EANS.

## 1. Introduction

In this contribution, a use case for EANS is described for a roaming UE when needed network slices are not all available from a single network.

## 2. Use case

### 2.1	Scenario condition

This use case involves a roaming UE which requires access to multiple slices available in the visited area.  Not all slices are offered by a single network in that area.

Following figure 1 shows the case where different network slices are available on different networks in the geographical location

Figure 1: roaming UE with service on network slices available on different networks

In this figure, basically, it is assumed that

- Location/Mobility:

  - UE A1 is roaming

- Subscription:

  - UE A1 has a subscription to use both slice M (for application App3) and slice N (for applications App1 and App2)

- Deployment:

  - In the local area, Network A offers slice N and Network B offers slice M

- Initial condition:

  - UE A1 is configured with a preference to use Network A in this area.

### 2.2	Discussion

Initially at time T1, UE A1 accesses Network A and makes use of applications App1 and App2 using slice N.

At time T2, UE A1 initiates use of App3 requiring slice M.  UE A1 accesses Network B to use slice M and is able to use App3.

When App3 ends, at time T3, UE A1 accesses Network A and has use of slice N for App1 and App2.

### 2.3	New requirements

Following new requirements can be derived from above use case.

- When a UE initiates an application that requires a network slice not offered by the serving network but available in the area from another network, the 3GPP system shall enable the UE to obtain service from the other network at least for the duration of the application. When the application completes, the 3GPP system shall enable the UE to return to the previous serving network e.g., based on preference.

## 3. Proposal

This is submitted for information to SA1#90e.