3GPP TSG-SA WG1 Meeting #90e 	S1-202088

Electronic Meeting, 18-22 May 2020	(revision of S1-20xxxx)

Title:	[Draft] Reply LS on Questions on onboarding requirements

Response to:	S2-2003216

Release:	Release 16

Work Item:	FS_eNPN

Source:	Qualcomm [to be changed to SA1]

To:	SA2

Cc: 	SA, SA3, CT1, CT6

Contact Person:

#### Name:	Jack Nasielski

E-mail Address:	jackn@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Description:

SA1 thanks SA2 for their reply regarding onboarding requirements. See below for further responses related to questions Q2 and Q3.

The 5G system shall support a secure mechanism for a network operator of an NPN to remotely provision the non-3GPP identities and credentials of a uniquely identifiable and verifiably secure IoT device.

Q2) SA2 would like to verify with SA1 whether the above-quoted requirement applies to PNI-NPN, which is the NPN “hosted by a PLMN” as described in TS 22.261 clause 6.25.1, or not, and what would be the corresponding use cases.

A2) SA1 requests clarification on the question from SA2, specifically, is SA2 asking if the above quoted question is related to primary or secondary authentication for the PNI-NPN.

SA2 is asking whether the non-3GPP identities and credentials (provided by authority of the PNI-NPN, e.g. enterprise, factory) for NSSAA or PDU session secondary authentication, can be remotely provisioned to UE which only has normal PLMN credentials and configuration.

=> Answer from SA1) there are no SA1 requirements on provisioning for NSSAA or secondary PDU session authentication for PNI-NPN.

-------------

Q3) If SA1 confirm the above-quoted requirement applies to PNI-NPN in Q2, SA2 have further Q3 as below.

For PNI-NPN, a UE may perform secondary PDU session authentication using 3rd party credentials, if the NPN is integrated in PLMN by means of dedicated DNNs, and/or a UE may perform Network specific slice authentication and authorisation (NSSAA) using 3rd party credentials if the NPN is integrated in PLMN by means of network slice. Given the authentication procedures already specified in TS 23.501, TS 24.501 and TS 33.501, SA2 would also like to ask whether provisioning for identities and credentials used for Network specific slice authentication and authorisation (NSSAA) and secondary PDU session authentication should be considered to be covered as part of NPN service requirement for onboarding and remote provisioning solution.

A3) SA1 requests clarification on the question from SA2, specifically, is SA2 asking whether 3rd party credentials may be used for secondary network slice authentication and authorization or

Is SA2 asking whether these 3rd party credentials for secondary authentication can be provisioned via the 3GPP system, or is SA2 asking something else.

SA2 is asking whether 3rd party identities and credentials (provided by authority of the PNI-NPN, e.g. enterprise, factory) used for NSSAA and PDU secondary authentication can be provisioned via the 3GPP system to UE, which only has normal PLMN credentials and configuration.

=> Answer from SA1) same answer as above.

2. Actions:

ACTION to SA2: 	SA1 kindly requests SA2 to take the above into account.

3. Date of Next TSG-SA WG1 Meetings:

SA1#91e	24 – 28 August 2020	e-meeting

SA1#92	16-20 November 2020	TBD, North America