3GPP TSG-SA WG1 Meeting #90e	S1-20xxxx

E-meeting, 18-22 May 2020

Source:	China Telecom, China Southern Power Grid, CEPRI-China, OMESH, Huawei, ZTE, CATT, Fudan, Spreadtrum

# Title:	Study on the 5G Smart Energy and Infrastructure

# Document for:	Approval

Agenda Item:	xx

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on the 5G Smart Energy and Infrastructure

Acronym: FS_5GSEI

Unique identifier:

Potential target Release: Rel-18.

- 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No |  |  |  |  |  |
| Don't know | X |  |  |  | X |

- 2	Classification of the Work Item and linked work items

- 2.1	Primary classification

This work item is a Study Item.

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

- 2.2	Parent Work Item

| Parent Work Items |  |
| --- | --- |
| Unique ID | Title |
| N/A | N/A |

- 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

- 3	Justification

A communication infrastructure is an essential part to the successes of the emerging smart energy.

A power grid consists of five phases: power generation, transmission, transformation, distribution, and consumption. These different phases require different data collection, working control and maintenance, so to smart grid communication infrastructure, it has to face different challenges:

- Clean & Friendly power generation

- Safety & High efficiency power transforming & transmission

- Flexible & Reliable power distribution

- interactive & On-demand power consumption

Referring the China State Grid white paper of “5g-network-sliceing-enableing-the-smart-grid”, and China South Grid white paper of “5G assists Smart Grid Application”, for example, in the distribution automation system, it is comprised of relay protection, telemetering, tele-indication, remote control and differential protection etc. working function, it requires 5G system to concurrently support multiple KPIs e.g. 2Mbps throughput, latency<10ms, clock sync<10us,latency jitter<50us/40km, reliability:99.999%, connection density is times of 10/km2, and also support to isolate this kind of service with other service from management working area named III and IV.  Another example is metrology automation & on-demand power supply, which can help the power supplier to predict electricity usage and schedule power supply efficiently.

In summary, it is considered beneficial for 3GPP to study enhanced 5G system support of different use cases and service requirements for smart grid.

In Rel-16 cyberCAV project, some use cases e.g electric-power distribution, centralized power generation have been studied in TR 22.804 and specific KPI and service function requirements have been specified in TS 22.104 and TS 22.261. In this study, it should be evaluate and analyse the GAP with these existed standards.

- 4	Objective

The aim of this work is to study use cases, potential new service requirements and KPI for 5G system to support smart grid, including:

- to support emerging Smart Grid services defined in other bodies, e.g. IEC.

- to support Smart Grid applications in specific phase or across multiple phases: e.g. on-demand power supply, distributed power supply system,  distribution automation, higher accuracy power load measurement and control, metrology automation, etc.

- gap analysis between potential new service requirements and existing requirements and functionalities for smart grid communications supported by 3GPP specs TS22.104/125/261 etc.

- 5	Expected Output and Time Scale

| New specifications [If Study Item, one TR is anticipated] |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Spec No. | Title | 1st rsp. WG | 2nd rsp. WG(s) | Presented for information at plenary# | Approved at plenary # | Comments |
| 22.xxx | Study on 5G Smart Energy and Infrastructure | SA1 |  | SA#93 | SA#94 |  |

- 6	Work item Rapporteur(s)

XIA Xu (xiaxu @chinatelecom.cn) (Rapporteur)

- 7	Work item leadership

SA1

- 8	Aspects that involve other WGs

Consideration of the grid security regulation and aspects.

- 9	Supporting Individual Members

Supporting IM name

China Telecom

OMESH Networks

Tencent

CATT

Telus

China Unicom

Sequans

Fudan University

China Mobile

Alibaba

Huawei

Huawei Device

Xiaomi

ZTE

China Southern Power Grid

CEPRI-China

Spreadtrum