3GPP TSG- SA WG1 Meeting #90-e	S1-202117

# Electronic Meeting, 18 - 25 May 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.125 | CR | 0029 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification to the definition of UAS, new requirements and numbering of requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Airbus |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2020-05-xx |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In response to the incoming LS S6-200544, clarifications, corrections and new requirements are proposed. In addition, numbering of non-numbered requirements and editorial corrections. |  |  |  |  |  |  |  |  |
| Summary of change: |  | First change: Definitions of UAS and UAV controller are clarified. Additionally one editorial correction. Second change: Adding missing requirement numbers. Third change: New requirements to clarify the relation and C2 communication needs between UAVC and UAV(s). Adding missing requirement numbers. Fourth change: Adding missing requirement numbers. Fifth change: Small correction in the figure (UAV-C Application). Removing bullets that were added as new requirements (third change). Clarification to the last bullet. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous definition of UAS causes issues for stage 2 groups (SA2, SA6). Tracking stage1 requirements in stage 2 groups is cumbersome when only ~50% of requirements in this TS are numbered. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 6.2, 6.3, 6.4, 6.5, 7.4, A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---First change---

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Above ground level (AGL): In the context of a UAV it is the UAV altitude referenced to ground level in the vicinity.

BVLOS	Beyond Visual Line of Sight

Command and Control (C2) Communication: the user plane link to deliver messages with information of command and control for UAV operation from a UAV controller or a UTM to a UAV.

Unmanned Aerial System (UAS): Composed of Unmanned Aerial Vehicle (UAV) and related functionality, including command and control (C2) links between the UAV and the control station, the UAV and the network, and for remote identification. An UAS may comprise of one or more UAV(s) and a UAV controller.

Unmanned Aerial System Traffic Management (UTM): a set of functions and services for managing a range of autonomous vehicle operations.

UAV controller: The UAV controller of a UAS enables a drone pilot to control UAV(s). BVLOS	Beyond Visual Line of Sight

UxNB: radio access node on-board UAV. It is a radio access node providing connectivity to UEs, which is carried in the air by an Unmanned Aerial Vehicle (UAV).

## ---End of the first change---

## ---Second change---

## 6.2	Network exposure for UAV services

[R-6.2-001] The 3GPP system shall provide means to allow a 3rd party to request and obtain real-time monitoring the status information (e.g., location of UAV, communication link status) of a UAV.

[R-6.2-002] Based on operator 's policy, the 3GPP system shall provide means to provide a 3rd party with the information regarding the service status for UAVs in a certain geographical area and/or at a certain time.​

NOTE: 	Service status is about the information of whether the communication service to the UAV can be provided with a certain QoS by the network.

## 6.3	Service restriction for UEs onboard of UAV

[R-6.3-001] The 3GPP network shall be able to support network-based 3D space positioning (e.g., with altitude 30~300m) of a UE onboard UAV.

[R-6.3-002] The 3GPP system shall be able to notify the authorized third party of potential stopping of connectivity service before the UE onboard of UAV enters an area (e.g., due to altitude) where the connectivity service is not authorized for the UE.

## 6.4	Requirements for UxNB

[R-6.4-001] The 5G system shall be able to support UxNBs to provide enhanced and more flexible radio coverage.

[R-6.4-002] The 3GPP system shall be able to provide suitable means to control the operation of the UxNBs (e.g. to start operation, stop operation, replace UxNB etc.).

[R-6.4-003] The 3GPP system shall be able to provide means to minimize power consumption of the UxNBs (e.g. optimizing operation parameter, optimized traffic delivery).

[R-6.4-004] The 3GPP system shall able to minimize interference e.g. caused by UxNBs changing their postions.

## ---End of the second change---

## ---Third change---

## 6.5	C2 communication

[R-6.5-001] The 3GPP system shall support C2 communication with required QoS for pre-defined C2 communication models (e.g. using direct ProSe Communication between UAV and the UAV controller, UTM-navigated C2 communication based on flight plan between UTM and the UAV).

[R-6.5-002] The 3GPP system shall support C2 communication with required QoS when switching between the C2 communication models.

[R-6.5-003] The 3GPP system shall support a mechanism for the UTM to request monitoring of the C2 communication with required QoS for pre-defined C2 communication models (e.g. using direct ProSe Communication between UAV and the UAV controller, UTM-navigated C2 communication between UTM and the UAV).

[R-6.5-004] A UAV controller connected via the 3GPP mobile network may control one or more UAV(s).

[R-6.5-005] A UAV controller connected via the 3GPP mobile network, controlling multiple UAVs, shall be able to send same data (using multicast) to all or to a group of UAVs that it is controlling.

[R-6.5-006] A UAV controller connected via the 3GPP mobile network, controlling multiple UAVs, shall be able to send different data simultaneously (using unicast connections) to each of the UAVs that it is controlling.

## ---End of the third change---

## ---Fourth change---

## 7.4	Other requirements

[R-7.4-001] The 5G system shall support a mechanism to switch between C2 communication modes for UAS operation, e.g. from indirect C2 communication to direct C2 communication, and ensure the disconnect time is below the latency requirements.

[R-7.4-002] The 3GPP system shall enable concurrent communications between the UAV-controller and UAV and between the UTM and the UAV that may require different KPIs.

[R-7.4-003] The 3GPP system shall be capable of switching between the KPIs, as requested by the UAV-controller or the UTM, within [500ms].

## ---End of the fourth change---

## ---Fifth change---

Annex A (informative):
UAS Reference Model

## A.1	UAS Reference Model in 3GPP ecosystem

Figure B.1-1: UAS model in 3GPP ecosystem.

In the UAS reference model:

-	UAVs are connected over cellular connectivity

-	a UAV may be controlled by a UAV controller connected via the 3GPP mobile network

-	a UAV may be controlled by a UAV controller not connected via the 3GPP mobile network, i.e. using transport not in 3GPP scope

-	the UAVs and the UAV controllers may exchange application data with a UTM

## ---End of the fifth change---