3GPP TSG-SA1 Meeting #90	S1-202129

Electronic Meeting, 18 - 22 May 2020	(revision of S1-20xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.261 | CR | 0447 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Complemetary requirements for consumed tethered VR headset services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Huawei Device |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNCIS |  |  |  |  |  | Date: |  |  | 2020-05-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There exists the following requirement gaps for the consumed tethered VR headset services considering the use case is a commercial case. - Gap1: ProSe is only available for public safety use case to the ProSe-enabled UE when the UE is not served by network - Gap 2: Pre-configuration with the permission to use ProSe is only available for the public safety use case - Gap 3: Network-controlled use of resources for ProSe is only available when the UE is served by network |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the requirements to enable a device not served by NG-RAN to perform communication over direct wireless link between devices. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirement for consumed tethered VR headset services is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X, 7.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------------------- 1st change --------------------------------------------------------------------

6.X	Proximity Services

6.X.1	Description

The following set of requirements complement the requirements listed in 3GPP TS 22.278 [5], clauses 7A.

6.X.2	Requirements

6.X.2.1	General

ProSe-enabled UEs can use ProSe for commercial when not served by NG-RAN.

The operator shall be able to pre-configure ProSe-enabled UEs (e.g., in the USIM or ME) not served by NG-RAN with the permission to use ProSe for commercial when using operator’s spectrum.

The 5G network shall be able to continuously control the use of NR-RAN resources for ProSe, as long as the ProSe is using operator's spectrum.

------------------------------------------------------------- end of 1st change -------------------------------------------------------------

------------------------------------------------------------- 2nd change --------------------------------------------------------------------

7.6	High data rate and low latency

7.6.1	AR/VR

Audio-visual interaction is characterised by a human being interacting with the environment or people, or controlling a UE, and relying on audio-visual feedback. In the use cases like VR and interactive conversation the latency requirements include the latencies at the application layer (e.g. codecs), which could be specified outside of 3GPP.

To support VR environments with low motion-to-photon capabilities, the 5G system shall support:

-	motion-to-photon latency in the range of 7-15ms while maintaining the required resolution of up to 8k giving user data rate of up to [1Gbit/s] and

-	motion-to-sound delay of [<20 ms].

NOTE: 	The motion-to-photon latency is defined as the latency between the physical movement of a user's head and the updated picture in the VR headset. The motion-to-sound latency is the latency between the physical movement of a user's head and updated sound waves from a head mounted speaker reaching their ears.

To support interactive task completion during voice conversations the 5G system shall support low-delay speech coding for interactive conversational services (100 ms, one way mouth-to-ear).

Due to the separate handling of the audio and video component, the 5G system will have to cater for the VR audio-video synchronisation in order to avoid having a negative impact on the user experience (i.e. viewers detecting lack of synchronization). To support VR environments the 5G system shall support audio-video synchronisation thresholds:

- in the range of [125 ms-5 ms] for audio delayed and

-	in the range of [45 ms-5 ms] for audio advanced.

When it comes to implementation of applications containing AR/VR components, the requirements on the 5G network could depend on architectural choices implementing these services. Note 3 in table 7.1-1 above gives an example on such dependences for a VR application in a 5G system. Table 7.6.1-1 below illustrates additional use cases and provides more corresponding requirements on the 5G system. The functional requirements related to communication over direct wireless link between UEs can be found in clause 6.X.

- Cloud/Edge/Split Rendering - Cloud/Edge/Split Rendering is characterised by the transition and exchange of the rendering data between the rendering server and device.

- Gaming or Training Data Exchanging – This use case is characterised by the exchange of the gaming or training service data between two 5G connected AR/VR devices.

- Consume VR content via tethered VR headset – This use case involves a tethered VR headset receiving VR content via a connected UE; this approach alleviates some of the computation complexity required at the VR headset, by allowing some or all decoding functionality to run locally at the connected UE. The requirements in the table below refer to the direct wireless link between the tethered VR headset and the corresponding connected UE.

Table 7.6.1-1 KPI Table for additional high data rate and low latency service

| Use Cases | Characteristic parameter (KPI) |  |  | Influence quantity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Max Allowed End-to-end latency | Service bit rate: user-experienced data rate | Reliability | # of UEs | UE Speed | Service Area (note 2) |
| Cloud/Edge/Split Rendering (note 1) | 5ms (i.e. UL+DL between UE and the interface to data network) (note 4) | 0.1-[1] Gbit/s supporting visual content (e.g. VR based or high definition video) with 4K, 8K resolution and up to120fps content. | 99.99% in uplink and 99.9% in downlink (note 4) | - | Stationary or Pedestrian | Countrywide |
| Gaming or Interactive Data Exchanging (note 3) | 10ms (note 4) | 0.1-[1] Gbit/s supporting visual content (e.g. VR based or high definition video) with 4K, 8K resolution and up to120fps content. | 99.99% (note 4) | ≤ [10] | Stationary or Pedestrian | 20 m x 10 m; in one vehicle (up to 120 km/h) and in one train (up to 500 km/h) |
| Consume VR content via tethered VR headset (note 6) | [5 -10] ms (note 5) | 0.1-[10] Gbit/s (note 5) | [99,99%] | - | Stationary or Pedestrian | - |
| NOTE 1: Unless otherwise specified, all communication via wireless link is between UEs and network node (UE to network node and/or network node to UE) rather than direct wireless links (UE to UE). NOTE 2: Length x width (x height). NOTE 3: Communication includes direct wireless links (UE to UE). NOTE 4: Latency and reliability KPIs can vary based on specific use case/architecture, e.g. for cloud/edge/split rendering, and may be represented by a range of values. NOTE 5: The decoding capability in the VR headset and the encoding/decoding complexity/time of the stream will set the required bit rate and latency over the direct wireless link between the tethered VR headset and its connected UE, bit rate from 100 Mbit/s to [10] Gbit/s and latency from 5 ms to 10 ms. NOTE 6: The performance requirement is valid for the direct wireless link between the tethered VR headset and its connected UE. |  |  |  |  |  |  |

-------------------------------------------------------------end of 2nd change -------------------------------------------------------------