3GPP TSG-SA1 Meeting #90	S1-202136

Electronic Meeting, 18 - 22 May 2020	(revision of S1-20xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 22.104 | CR | 0043 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update Time Synchronization Budget Requirement for 5G System |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Siemens |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCAV |  |  |  |  |  | Date: |  |  | 2020-05-18 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The time synchronization budget requirement for the 5G System has been for further study in TS 22.104. Realistic and useful use case scenarios for time synchronization in integrated 5G/TSN networks have been considered (S1-202134) in order to derive useful values for the time synchronization budget allocated to the 5G system. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Based on relevant use case scenarios for time synchronization in integrated 5G/TSN networks, different levels of the 5G time synchronization budget requirement are given in 5.6.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Time synchronization budget requirement for the 5G system is still for further study. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.1, 5.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF PROPOSED CHANGES

Start of Change 1

### 5.6.1	Clock synchronisation service level requirements

The 5G system shall support a mechanism to process and transmit IEEE1588v2 / Precision Time Protocol messages to support 3rd-party applications which use this protocol.

The 5G system shall support a mechanism to synchronise the user-specific time clock of UEs with a global clock.

The 5G system shall support a mechanism to synchronize the user-specific time clock of UEs with a working clock.

The 5G system shall support two types of synchronization clocks, the global time domain and the working clock domains.

The 5G system shall support networks with up to 32 working clock domains.

NOTE 1:	The domain number (synchronization domain identifier) is defined with one octet in IEEE 802.1AS [22]. This allows for 256 synchronization domains.

The 5G system shall support at least 2 simultaneous working clock domains on a UE.

The synchronicity budget for the 5G system within the global time domain shall not exceed the values given in Table 5.6.2-2.

NOTE 2:	The global time domain requires in general a precision of 1 µs between the sync master and any device of the clock domain. Some use cases require only a precision of ≤ 100 µs for the global time domain if a working clock domain with precision of ≤ 1 µs is available.

NOTE 3:	(void)

The synchronicity budget for the 5G system within a working clock domain shall not exceed the values given in Table 5.6.2-2.

NOTE 4: The working clock domains	require a precision of ≤ 1 µs between the sync master and any device of the clock domain.

NOTE 5:	Different working clock domains are independent and can have different precision.

The 5G system shall provide a media dependent interface for one or multiple 802.1AS sync domains [22].

The 5G system shall provide an interface to the 5G sync domain which can be used by applications to derive their working clock domain or global time domain (Reference Clock Model).

The 5G system shall provide an interface at the UE to determine and to configure the precision and time scale of the working clock domain.

### 5.6.2	Clock synchronisation service performance requirements

Table 5.6.2-1: Clock synchronization service performance requirements for the 5G System

| User-specific clock synchronicity accuracy level | Number of devices in one Communication group for clock synchronisation | Clock synchronicity requirement | Service area | Scenario |
| --- | --- | --- | --- | --- |
| 1 | Up to 300 UEs | <1 µs | ≤ 100 m x 100 m | - Motion control - Control-to-control communication for industrial controller |
| 2 | Up to 10 UEs | < 10 µs | ≤ 2500 m2 | - High data rate video streaming |
| 3 | Up to 100 UEs | <1 µs | < 20 km2 | - Smart Grid: synchronicity between PMUs |

Table 5.6.2-2: Synchronicity budget requirements for the 5G system

| Sync budget level | Integration scenario (one/two wireless links) | Clock synchronicity requirement | Synchronicity budget requirement for 5GS | Scenario |
| --- | --- | --- | --- | --- |
| 1 | one wireless link | ±1 µs | [750 ns] | smart grid - PMU |
| 2A | one wireless link | ±1 µs | 667 ns | two manufacturing robots (wireless/wired) – minimum |
| 2B | two wireless links | ±1 µs | 700 ns | two manufacturing robots (both wireless) |
| 3A | one wireless link | ±1 µs | 500 ns | two manufacturing robots (wireless/wired) – general |

End of Change 1

END OF PROPOSED CHANGES