3GPP TSG-SA1 Meeting #90	S1-202150

Electronic Meeting, 18 - 22 May 2020	(revision of S1-20xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.104 | CR | 0047 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clock synchronicity budget for the 5G system |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Nokia Shanghai Bell, Orange, Verizon, Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCAV |  |  |  |  |  | Date: |  |  | 2020-05-08 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The clock synchronicity budget allocated to the 5G system is a major open question in TS 22.104. Upon consideration of realistic deployment scenarios (see S1-20xxxx), values for the missing requirements on the synchronicity budget for the 5G system are proposed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Synchronicity budget requirements for the 5G system are provided in clause 5.6.1 and missing clock synchronization service performance requirements are provided in Table 5.6.2-1. The description of the clock synchronicity budget in Clause 5.6.0 is simplified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements on clock synchronicity budget for the 5G system will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.0, 5.6.1, 5.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Start of the 1st change --------------------------------------------------------------

### 5.6.0	Description

Clock synchronicity, or time synchronization precision, is defined between a sync master and a sync device.The requirement on the synchronicity budget for the 5G system is the time error contribution between clock input and output of the 5G system.

### 5.6.1	Clock synchronisation service level requirements

The 5G system shall support a mechanism to process and transmit IEEE1588v2 / Precision Time Protocol messages to support 3rd-party applications which use this protocol.

The 5G system shall support a mechanism to synchronise the user-specific time clock of UEs with a global clock.

The 5G system shall support a mechanism to synchronize the user-specific time clock of UEs with a working clock.

The 5G system shall support two types of synchronization clocks, the global time domain and the working clock domains.

The 5G system shall support networks with up to 128 working clock domains (with different synchronization domain identifiers / domain numbers), including for UEs connected through the 5G network.

NOTE 1:	The domain number (synchronization domain identifier) is defined with one octet in IEEE 802.1AS [22].

The 5G system shall be able to support up to four simultaneous synchronization domains on a UE.

NOTE 1A:	The four synchronization domains are used, for example, as two synchronization domains for global time and two working clock domains. One pair of global time and working clock is used as redundant synchronization domains for zero failover time.

The synchronicity budget for the 5G system within the global time domain shall not exceed 900 ns.

NOTE 2:	The global time domain requires in general a precision of 1 µs between the sync master and any device of the clock domain. Some use cases require only a precision of ≤ 100 µs for the global time domain if a working clock domain with precision of ≤ 1 µs is available.

NOTE 3:	(void)

The synchronicity budget for the 5G system within a working clock domain shall not exceed 900 ns.

NOTE 4:	The working clock domains 	require a precision of ≤ 1 µs between the sync master and any device of the clock domain.

NOTE 5:	Different working clock domains are independent and can have different precision.

The 5G system shall provide a media dependent interface for one or multiple 802.1AS sync domains [22].

The 5G system shall provide an interface to the 5G sync domain which can be used by applications to derive their working clock domain or global time domain (Reference Clock Model).

The 5G system shall provide an interface at the UE to determine and to configure the precision and time scale of the working clock domain.

The 5G system shall be able to support arbitrary placement of sync master functionality and sync device functionality in integrated 5G / non-3GPP TSN networks.

The 5G system shall be able to support clock synchronization through the 5G network if the sync master and the sync devices are served by different UEs. (Flow of clock synchronization messages is in either direction, UL and DL.)

The 5G system shall provide a suitable means to support the management of the merging and separation of working clock domains, that is interoperable with the corresponding mechanisms of TSN and IEEE 802.1AS.

### 5.6.2	Clock synchronisation service performance requirements

Table 5.6.2-1: Clock synchronization service performance requirements for 5G System

| User-specific clock synchronicity accuracy level | Number of devices in one Communication group for clock synchronisation | Synchronicity budget requirement (note) | Service area | Scenario |
| --- | --- | --- | --- | --- |
| 1 | Up to 300 UEs | ≤900 ns | ≤ 100 m x 100 m | - Motion control - Control-to-control communication for industrial controller |
| 2 | Up to 300 UEs | ≤900 ns | ≤ 1000 m x 100 m | - Control-to-control communication for industrial controller |
| 3 | Up to 10 UEs | < 10 µs | ≤ 2500 m2 | - High data rate video streaming |
| 3a | Up to 100 UEs | <1 µs | ≤10 km2 | - AVProd synchronisation and packet timing |
| 4 | Up to 100 UEs | <1 µs | < 20 km2 | - Smart Grid: synchronicity between PMUs |
| 5 | Up to 10 UEs | < 50 µs | 400 km | - Telesurgery and telediagnosis |
| NOTE: The clock synchronicity requirement refers to the clock synchronicity budget for the 5G system, as described in Clause 5.6.1. |  |  |  |  |

----------------------------------------------------- End of 1st change --------------------------------------------------------------