3GPP TSG-SA1 Meeting #90	S1-202157

Electronic Meeting, 18 - 22 May 2020	(revision of S1-20xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 22.832 | CR | 0028 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous editorial corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Siemens |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_eCAV |  |  |  |  |  | Date: |  |  | 2020-05-18 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There are several editorial slips in TR 22.832. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Miscellaneous editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | More miscellaneous editorial slips in TR 22.832. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.6, 5.18.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF PROPOSED CHANGES

Start of Change 1

### 5.2.6	Potential New Requirements needed to support the use case

[PR-5.2.6-001]	The 5G system shall be able to support up to four synchronization domains on a UE.

NOTE:	The four synchronization domains are used, for example, as two synchronization domains for global time and two working clock domains. One pair of global time and working clock is used as redundant synchronization domains for zero failover time.

[PR-5.2.6-002]	The 5G system shall be able to support clock synchronization through the 5G network if the sync master and the sync devices are in non-3GPP TSN networks connected to the 5G network through different UEs. (Flow of clock synchronization messages is both, UL and DL.)

Requirement [PR-5.2.6-002] highlights a specific integration scenario under the existing TSN requirements that requires the flow of clock synchronization messages to be both, UL and DL.

[PR-5.2.6-003]	The 5G system shall be able to support arbitrary placement of sync master functionality and sync device functionality in integrated 5G / non-3GPP TSN networks. (Flow of clock synchronization messages is both, UL and DL.)

End of Change 1

Start of Change 2

### 5.18.6	Potential New Requirements needed to support the use case

[PR-5.18.6-001] 5G System shall be able to support industrial wireless sensors according to the service performance requirements listed in Table 5.18.6-1.

Table 5.18.6-1: Industrial wireless sensors service performance requirements

|  | Characteristic parameter |  |  |  |  |  |  | Influence quantity |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use case | Communication service availability: target value in % | Communication service reliability: mean time between failure | End-to-end latency | Transfer interval | Bit rate [bits/s] | Battery lifetime [year] (note 1) | Direction | Message Size [byte] | Survival time | UE speed | UE density [UE / m²] | Range [m] (note 2) |
| 1 | 99,99 | ≥ 1 week | < 100 ms | 100 ms – 60 s | ≤ 1 M | ≥5 | Mobile originated | 20 (note 3) | 3 x transfer interval | Stationary | Up to 1 | <500 |
| 2 | 99,99 | ≥ 1 week | < 100 ms | ≤ 1 s | ≤ 200 k | ≥5 | Mobile originated | 25 k | 3 x transfer interval | Stationary | Up to 0,05 | <500 |
| 3 | 99,99 | ≥ 1 week | < 100 ms | ≤ 1 s | ≤ 2 M | ≥5 | Mobile originated | 250 k | 3 x transfer interval | Stationary | Up to 0,05 | <500 |
| NOTE 1: Industrial sensors can use a wide variety of batteries depending on the use case,but in general they are highly constrained in terms of battery size. NOTE 2: Distance between the gNB and the UE. NOTE 3: The application-level messages in this use case are typically transferred over Ethernet. For small messages, the minimum Ethernet frame size of 64 bytes applies and dictates the minimum size of the PDU sent over the air interface. |  |  |  |  |  |  |  |  |  |  |  |  |

Use case one

Sensors generating periodic measurements of a continuous value (e.g. temperature, pressure, flow rate sensors).

Use case two

Sensors generating waveform measurements (e.g. vibration sensors). Even though the waveform measurement is continuous, it is expected that this type of sensors will buffer and transmit the data periodically (e.g. every second) to save battery by enabling discontinuous transmission.

Use case three

Cameras (regular or thermal) for asset monitoring (e.g. for leakage detection). Even though the video recording is continuous, it is expected that this type of sensors will buffer and transmit the data periodically (e.g. every second) to save battery by enabling discontinuous transmission.

End of Change 2

END OF PROPOSED CHANGES