3GPP TSG-SA1 Meeting #90	S1-202177

Electronic Meeting, 18 - 22 May 2020	(revision of S1-202158)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 22.104 | CR | 0048 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous values for further study |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Siemens |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCAV |  |  |  |  |  | Date: |  |  | 2020-05-18 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There are still some values for further study at miscellaneous places in TS 22.104. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification of several miscellaneous ffs values and editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | More miscellaneous values left for further study and editorial slips. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, A.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF PROPOSED CHANGES

Start of Change 1

## 5.2	Periodic deterministic communication

Periodic deterministic communication is periodic with stringent requirements on timeliness and availability of the communication service. A transmission occurs every transfer interval. A description of periodic deterministic communication can be found in Clauses 4.3 and 4.4. Additional information on the underlying use cases of the sets of requirements in Table 5.2-1 can be found in Annex A. Further information on characteristic parameters and influence quantities used in Table 5.2-1 can be found in Annex C.

The 5G system shall be able to provide periodic deterministic communication with the service performance requirements reported in Table 5.2-1.

Process and asset monitoring using industrial wireless sensors is a special case of periodic deterministic communication with more relaxed requirements on timeliness and availability. These use cases put a slightly different set of requirements on the 5G system due to the specific constraints of industrial wireless sensors. These requirements are listed in Table 5.2-2 and additional information on the underlying use cases can be found in Annex A.

Table 5.2-1: Periodic deterministic communication service performance requirements

| Characteristic parameter |  |  |  | Influence quantity |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Communication service availability: target value (note 1) | Communication service reliability: mean time between failures | End-to-end latency: maximum (note 2) (note 12a) | Service bit rate: user experienced data rate (note 12a) | Message size [byte] (note 12a) | Transfer interval: target value (note 12a) | Survival time (note 12a) | UE speed (note 13) | # of UEs | Service area (note 3) | Remarks |  |
| 99,999 % to 99,99999 % | ~ 10 years | < transfer interval value | – | 50 | 500 μs | 500 μs | ≤ 75 km/h | ≤ 20 | 50 m x 10 m x 10 m | Motion control (A.2.2.1) |  |
| 99,9999 % to 99,999999 % | ~ 10 years | < transfer interval value | – | 40 | 1 ms | 1 ms | ≤ 75 km/h | ≤ 50 | 50 m x 10 m x 10 m | Motion control (A.2.2.1) |  |
| 99,9999 % to 99,999999 % | ~ 10 years | < transfer interval value | – | 20 | 2 ms | 2 ms | ≤ 75 km/h | ≤ 100 | 50 m x 10 m x 10 m | Motion control (A.2.2.1) |  |
| 99,9999 % | – | < 5 ms | 1 kbit/s (steady state) 1,5 Mbit/s (fault case) | < 1500 | < 60 s (steady state) ≥ 1 ms (fault case) | transfer interval | stationary | 20 | 30 km x 20 km | Electrical Distribution – Distributed automated switching for isolation and service restoration (A.4.4); (note 5) |  |
| 99,9999 % to 99,999999 % | ~ 10 years | < transfer interval value |  | 1 k | ≤ 10 ms | 10 ms | - | 5 to 10 | 100 m x 30 m x 10 m | Control-to-control in motion control (A.2.2.2); (note 9) |  |
| 99,9999 % to 99,999999 % | ~ 10 years | < transfer interval value (note 5) | 50 Mbit/s |  | ≤ 1 ms | 3 * transfer interval | stationary | 2 to 5 | 100 m x 30 m x 10 m | Wired-2-wireless 100 Mbit/s link replacement (A.2.2.4) |  |
| 99,9999 % to 99,999999 % | ~ 10 years | < transfer interval value (note 5) | 250 Mbit/s |  | ≤ 1 ms | 3 * transfer interval | stationary | 2 to 5 | 100 m x 30 m x 10 m | Wired-2-wireless 1 Gbit/s link replacement (A.2.2.4) |  |
| 99,9999 % to 99,999999 % | ~ 10 years | < transfer interval value |  | 1 k | ≤ 50 ms | 50 ms | - | 5 to 10 | 1000 m x 30 m x 10 m | Control-to-control in motion control (A.2.2.2); (note 9) |  |
| > 99,9999 % | ~ 10 years | < transfer interval value | – | 40 to 250 | 1 ms to 50 ms (note 6) (note 7) | transfer interval value | ≤ 50 km/h | ≤ 100 | ≤ 1 km2 | Mobile robots (A.2.2.3) |  |
| 99,9999 % to 99,999999 % | ~ 1 month | < transfer interval value | – | 40 to 250 | 4 ms to 8 ms (note 7) | transfer interval value | < 8 km/h (linear movement) | TBD | 50 m x 10 m x 4 m | Mobile control panels – remote control of e.g. assembly robots, milling machines (A.2.4.1); (note 9) |  |
| 99,999999 % | 1 day | <8 ms (note 14) | 250 kbit/s | 40 to 250 | 8 ms | 16 ms | quasi-static; up to 10 km/h | 2 or more | 30 m x 30 m | Mobile Operation Panel: Emergency stop (connectivity availability) (A.2.4.1A) |  |
| 99,99999 % | 1 day | <10 ms (note 14) | < 1 Mbit/s | <1024 | 10 ms | ~10 ms | quasi-static; up to 10 km/h | 2 or more | 30 m x 30 m | Mobile Operation Panel: Safety data stream (A.2.4.1A) |  |
| 99,999999 % | 1 day | 10-100 ms (note 14) | 10 kbit/s | 10-100 | 10-100 ms | transfer interval | stationary | 2 or more | 100-2000 m² | Mobile Operation Panel: Control to visualization (A.2.4.1A) |  |
| 99,999999 % | 1 day | <1 ms (note 14) | 12-16 Mbit/s | 10-100 | 1 ms | ~1 ms | stationary | 2 or more | 100 m² | Mobile Operation Panel: Motion control (A.2.4.1A) |  |
| 99,999999 % | 1 day | <2 ms (note 14) | 16 kbit/s (UL) 2 Mbit/s (DL) | 50 | 2 ms | ~2 ms | stationary | 2 or more | 100 m² | Mobile Operation Panel: Haptic feedback data stream (A.2.4.1A) |  |
| 99,9999 % to 99,999999 % | ~ 1 year | < transfer interval | – | 40 to 250 | < 12 ms (note 7) | 12 ms | < 8 km/h (linear movement) | TBD | typically 40 m x 60 m; maximum 200 m x 300 m | Mobile control panels -remote control of e.g. mobile cranes, mobile pumps, fixed portal cranes (A.2.4.1); (note 9) |  |
| 99,9999 % to 99,999999 % | ≥ 1 year | < transfer interval value | – | 20 | ≥ 10 ms (note 8) | 0 | typically stationary | typically 10 to 20 | typically ≤ 100 m x 100 m x 50 m | Process automation – closed loop control (A.2.3.1) |  |
| 99,999 % | TBD | ~ 50 ms | – | ~ 100 | ~ 50 ms | TBD | stationary | ≤ 100 000 | several km2 up to 100 000 km2 | Primary frequency control (A.4.2); (note 9) |  |
| 99,999 % | TBD | ~ 100 ms | – | ~ 100 | ~ 200 ms | TBD | stationary | ≤ 100 000 | several km2 up to 100 000 km2 | Distributed Voltage Control (A.4.3) (note 9) |  |
| > 99,9999 % | ~ 1 year | < transfer interval value | – | 15 k to 250 k | 10 ms to 100 ms (note 7) | transfer interval value | ≤ 50 km/h | ≤ 100 | ≤ 1 km2 | Mobile robots – video-operated remote control (A.2.2.3) |  |
| > 99,9999 % | ~ 1 year | < transfer interval value | – | 40 to 250 | 40 ms to 500 ms (note 7) | transfer interval value | ≤ 50 km/h | ≤ 100 | ≤ 1 km2 | Mobile robots (A.2.2.3) |  |
| 99,99 % | ≥ 1 week | < transfer interval value | – | 20 to 255 | 100 ms to 60 s (note 7) | ≥ 3 x transfer interval value | typically stationary | ≤ 10 000 to 100 000 | ≤ 10 km x 10 km x 50 m | Plant asset management (A.2.3.3) |  |
| > 99,999999% | >10 years | <2 ms | 2-16 Mbit/s | 250 to 2000 | 1 ms | transfer interval value | Stationary | 1 | < 100 m2 | Robotic Aided Surgery (A.6.2) |  |
| > 99,9999% | >1 year | <20 ms | 2-16 Mbit/s | 250 to 2000 | 1 ms | transfer interval value | Stationary | 2 per 1000 km2 | < 400 km (note 12) | Robotic Aided Surgery (A.6.2) |  |
| > 99,999% | >> 1 month (< 1 year) | <20 ms | 2-16 Mbit/s | 80 | 1 ms | transfer interval value | Stationary | 20 per 100 km2 | < 50 km (note 12) | Robotic Aided Diagnosis (A.6.3) |  |
| 99,9999 % to 99,999999 % | ~ 10 years | < 0,5 * transfer interval | 2,5 Mbit/s | 250 500 with localisation information | > 5 ms >2,5 ms >1,7 ms (note 10) | 0 transfer interval 2 * transfer interval (note 10) | ≤6 km/h (linear movement) | 2 to 8 | 10 m x 10 m x 5 m; 50 m x 5 m x 5 m (note 11 | Cooperative carrying – fragile work pieces; (ProSe communication) |  |
| 99,9999 % to 99,999999 % | ~ 10 years | < 0,5 * transfer interval | 2,5 Mbit/s | 250 500 with localisation information | > 5 ms >2,5 ms >1,7 ms (note 10) | 0 transfer interval 2 * transfer interval (note 10) | ≤12 km/h (linear movement) | 2 to 8 | 10 m x 10 m x 5 m; 50 m x 5 m x 5 m (note 11) | Cooperative carrying – elastic work pieces; (ProSe communication) |  |
| NOTE 1: One or more retransmissions of network layer packets may take place in order to satisfy the communication service availability requirement. NOTE 2: Unless otherwise specified, all communication includes 1 wireless link (UE to network node or network node to UE) rather than two wireless links (UE to UE). NOTE 3: Length x width (x height). NOTE 4: (void) NOTE 5: Communication includes two wireless links (UE to UE). NOTE 6: This covers different transfer intervals for different similar use cases with target values of 1 ms, 1 ms to 10 ms, and 10 ms to 50 ms. NOTE 7: The transfer interval deviates around its target value by < ± 25 %. NOTE 8: The transfer interval deviates around its target value by < ± 5 %. NOTE 9: Communication may include two wireless links (UE to UE). NOTE 10: The first value is the application requirement, the other values are the requirement with multiple transmission of the same information (two or tree times respectively). NOTE 11: Service Area for direct communication between UEs. The group of UEs with direct communication might move throughout the whole factory site (up to several km²). NOTE 12: Maximum straight-line distance between UEs. NOTE 12a: It applies to both UL and DL unless stated otherwise. NOTE 13: It applies to both linear movement and rotation unless stated otherwise. NOTE 14: The mobile operation panel is connected wirelessly to the 5G system. If the mobile robot/production line is also connected wirelessly to the 5G system, the communication includes two wireless links. |  |  |  |  |  |  |  |  |  |  |

Table 5.2-2: Communication service performance requirements for industrial wireless sensors

| Characteristic parameter |  |  |  |  |  | Influence quantity |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Communication service availability: target value | Communication service reliability: mean time between failure | End-to-end latency (note 6) | Transfer interval (note 1) (note 7) | Service bit rate: user experienced data rate (note 2) (note 7) | Battery lifetime [year] (note 3) | Message Size [byte] (note 7) | Survival time (note 7) | UE speed | UE density [UE / m²] | Range [m] (note 4) | Remarks |
| 99,99 % | ≥ 1 week | < 100 ms | 100 ms – 60 s | ≤ 1 Mbit/s | ≥5 | 20 (note 5) | 3 x transfer interval | Stationary | Up to 1 | <500 | Process monitoring, e.g. temperature sensor (A.2.3.2) |
| 99,99 % | ≥ 1 week | < 100 ms | ≤ 1 s | ≤ 200 kbit/s | ≥5 | 25 k | 3 x transfer interval | Stationary | Up to 0,05 | <500 | Asset monitoring, e.g. vibration sensor (A.2.3.2) |
| 99,99 % | ≥ 1 week | < 100 ms | ≤ 1 s | ≤ 2 Mbit/s | ≥5 | 250 k | 3 x transfer interval | Stationary | Up to 0,05 | <500 | Asset monitoring, e.g. thermal camera (A.2.3.2) |
| NOTE 1: The transfer interval deviates around its target value by < ± 25 %. NOTE 2: The traffic is predominatly mobile originated. NOTE 3: Industrial sensors can use a wide variety of batteries depending on theuse case, but in general they are highly constrained in terms of battery size. NOTE 4: Distance between the gNB and the UE. NOTE 5: The application-level messages in this use case are typically transferred over Ethernet. For small messages, the minimum Ethernet frame size of 64 bytes applies and dictates the minimum size of the PDU sent over the air interface. NOTE 6: It applies to both UL and DL unless stated otherwise. NOTE 7: It applies to UL. |  |  |  |  |  |  |  |  |  |  |  |

End of Change 1

Start of Change 2

## A.4.4	Distributed automated switching for isolation and service restoration

A power distribution grid fault is a stressful situation. There are self-healing solutions for automated switching, fault isolation and, service restoration. Furthermore, these solutions are ideally suited to handle outages that affect critical power consumers, such as industrial plants or data centres. Supply interruptions must be fixed within less than a second for critical power consumers. Automated solutions are able to restore power supply within a few hundred milliseconds.

Figure A.4.4-1: Depiction of a distribution ring and a failure (flash of lighting)

The FLISR (Fault Location, Isolation & Service Restoration) solution consists of switch controller devices which are especially designed for feeder automation applications that support the self-healing of power distribution grids with overhead lines. They serve as control units for reclosers and disconnectors in overhead line distribution grids.

The system is designed for using fully distributed, independent automated devices. The logic resides in each individual feeder automation controller located at the poles in the feeder level. Each feeder section has a controller device. Using peer-to-peer communication among the controller devices, the system operates autonomously without the need of a regional controller or control centre. However, all self-healing steps carried out will be reported immediately to the control centre to keep the grid status up-to-date. The controllers conduct self-healing of the distribution line in typically 500 ms by isolating the faults.

Peer-to-peer communication via IEC 61850 GOOSE (Generic Object Oriented Substation Event) messages provides data as fast as possible (Layer 2 multicast message). They are sent periodically (in steady state, with changing interval time in fault case) by each controller to several or all other controllers of the same feeder and are not acknowledged.

The data rate per controller is low in steady state, but GOOSE bursts with high data rate do occur, especially during fault situations. GOOSE messages are sent by several or all controller units of the feeder nearly at the same point in time during the fault location, isolation and service restoration procedure with a low end-to-end latency.

Table A.4.4-1: Service performance requirements for distributed automated switching for isolation and service restoration

| Use case # | Characteristic parameter |  |  |  | Influence quantity |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Communication service availability: target value in % | Communication service reliability: mean time between failures | End-to-end latency: maximum | Service bitrate: user experienced data rate | Message size [byte] | Transfer interval: target value | Survival time | UE speed | # of UEs | Service area (note 1) |
| 1 (note 2) | 99,9999 | – | < 5 ms | 1 kbit/s (steady state) 1,5 Mbit/s (fault case) | < 1500 | < 60 s (steady state) ≥ 1 ms (fault case) | transfer interval (one frame loss) | stationary | 20 | 30 km x 20 km |
| NOTE 1: Length x width NOTE 2: UE to UE communication (two wireless links) |  |  |  |  |  |  |  |  |  |  |

Use case one

GOOSE (a)periodic deterministic communication service supporting bursty message exchange for fault location, isolation, and service restoration.

End of Change 2

END OF PROPOSED CHANGES