3GPP TSG-SA WG1 Meeting # 90e	S1-202044r1

# Electronic Meeting, May, 18 – 22, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.261 | CR | 0441 | rev | - | Current version: | 16.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to use of AC 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIOT |  |  |  |  |  | Date: |  |  | 2020-05-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | AC10 is applied when UEs for IOT service needs to send exception data. However, current NOTE 6 is ambigous regarding when the UE can use AC 10. 1. In general, most of data that IoT devices generate belong to ‘delay tolerant’ data. And, only a few specific UE will sometimes generate data that can be categorized as ‘exception data’. Accordingly, only when a UE is allowed to use ‘exception data’, the UE can categorize some data into ‘exception data’ and other data into ‘delay-tolerant’ data. And, if a UE is not allowed to use ‘exception data’, the UE should not categorize any data as ‘exception data’. TS 24.501 v16.4.0 includes following statement: the UE is allowed to use exception data reporting (see the ExceptionDataReportingAllowed leaf of the NAS configuration MO in 3GPP TS 24.368 [17]) Thus, the UE is configured with whether it is allowed to use exception data or not. TS22.261 needs to be aligned to TS24.501 2. As described in 23.501, a UE can support both NB-IoT RAT and other RATs. Thus, the use of AC 10 should be restricted to when a UE is accessing NB-IoT RAT cell connected to 5GC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarfied that - only UE which is authorized to use ‘exception data’ can use AC 10. - AC 10 is used only in NB-IoT cell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AC 10 can be misused by a UE when it should not. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.22.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6.22.2.3	Access categories

Table 6.22.2.3-1: Access Categories

| Access Category number | Conditions related to UE | Type of access attempt |
| --- | --- | --- |
| 0 | All | MO signalling resulting from paging |
| 1 (NOTE 1) | UE is configured for delay tolerant service and subject to access control for Access Category 1, which is judged based on relation of UE’s HPLMN and the selected PLMN. | All except for Emergency, or MO exception data |
| 2 | All | Emergency |
| 3 | All except for the conditions in Access Category 1. | MO signalling on NAS level resulting from other than paging |
| 4 | All except for the conditions in Access Category 1. | MMTEL voice (NOTE 3) |
| 5 | All except for the conditions in Access Category 1. | MMTEL video |
| 6 | All except for the conditions in Access Category 1. | SMS |
| 7 | All except for the conditions in Access Category 1. | MO data that do not belong to any other Access Categories (NOTE 4) |
| 8 | All except for the conditions in Access Category 1 | MO signalling on RRC level resulting from other than paging |
| 9 | All except for the conditions in Access Category 1 | MO IMS registration related signalling (NOTE 5) |
| 10 (NOTE 6) | All | MO exception data |
| 11-31 |  | Reserved standardized Access Categories |
| 32-63 (NOTE 2) | All | Based on operator classification |
| NOTE 1: The barring parameter for Access Category 1 is accompanied with information that define whether Access Category applies to UEs within one of the following categories: a) UEs that are configured for delay tolerant service; b) UEs that are configured for delay tolerant service and are neither in their HPLMN nor in a PLMN that is equivalent to it; c) UEs that are configured for delay tolerant service and are neither in the PLMN listed as most preferred PLMN of the country where the UE is roaming in the operator-defined PLMN selector list on the SIM/USIM, nor in their HPLMN nor in a PLMN that is equivalent to their HPLMN. When a UE is configured for EAB, the UE is also configured for delay tolerant service. In case a UE is configured both for EAB and for EAB override, when upper layer indicates to override Access Category 1, then Access Category 1 is not applicable. NOTE 2: When there are an Access Category based on operator classification and a standardized Access Category to both of which an access attempt can be categorized, and the standardized Access Category is neither 0 nor 2, the UE applies the Access Category based on operator classification. When there are an Access Category based on operator classification and a standardized Access Category to both of which an access attempt can be categorized, and the standardized Access Category is 0 or 2, the UE applies the standardized Access Category. NOTE 3: Includes Real-Time Text (RTT). NOTE 4: Includes IMS Messaging. NOTE 5: Includes IMS registration related signalling, e.g. IMS initial registration, re-registration, and subscription refresh. NOTE 6: Applies to access of a NB-IoT-capable UE to a NB-IOT cell connected to 5GC when the UE is authorized to send exception data. |  |  |

Access Category 0 shall not be barred, irrespective of Access Identities.

NOTE:	The network can control the amount of access attempts relating to Access Category 0 by controlling whether to send paging or not.