3GPP TSG-SA1 Meeting #90	S1-202174

Electronic Meeting, 18 - 22 May 2020	(revision of S1-202135)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 22.104 | CR | 0042 | rev | 2 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update Time Synchronization Budget Requirement for 5G System |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Siemens |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCAV |  |  |  |  |  | Date: |  |  | 2020-05-18 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The time synchronization budget requirement for the 5G System has been for further study in TS 22.104. Realistic and useful use case scenarios for time synchronization in integrated 5G/TSN networks have been considered (S1-202173) in order to derive useful values for the time synchronization budget allocated to the 5G system. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Based on relevant use case scenarios for time synchronization in integrated 5G/TSN networks, a value for the 5G time synchronization budget requirement in Rel.17 is given in 5.6.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Time synchronization budget requirement for the 5G system is still for further study. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.1, 5.6.2, Annex X (new), X.1, X.2, X.2.1, X.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF PROPOSED CHANGES

Start of Change 1

### 5.6.1	Clock synchronisation service level requirements

The 5G system shall support a mechanism to process and transmit IEEE1588v2 / Precision Time Protocol messages to support 3rd-party applications which use this protocol.

The 5G system shall support a mechanism to synchronise the user-specific time clock of UEs with a global clock.

The 5G system shall support a mechanism to synchronize the user-specific time clock of UEs with a working clock.

The 5G system shall support two types of synchronization clocks, the global time domain and the working clock domains.

The 5G system shall support networks with up to 128 working clock domains (with different synchronization domain identifiers / domain numbers), including for UEs connected through the 5G network.

NOTE 1:	The domain number (synchronization domain identifier) is defined with one octet in IEEE 802.1AS [22].

The 5G system shall be able to support up to four simultaneous synchronization domains on a UE.

NOTE 1A:	The four synchronization domains are used, for example, as two synchronization domains for global time and two working clock domains. One pair of global time and working clock is used as redundant synchronization domains for zero failover time.

The synchronicity budget for the 5G system within the global time domain shall not exceed 900 ns.

NOTE 2:	The global time domain requires in general a precision of 1 µs between the sync master and any device of the clock domain. Some use cases require only a precision of ≤ 100 µs for the global time domain if a working clock domain with precision of ≤ 1 µs is available.

NOTE 3:	(void)

The synchronicity budget for the 5G system within a working clock domain shall not exceed 900 ns.

NOTE 4:	The working clock domains	require a precision of ≤ 1 µs between the sync master and any device of the clock domain.

NOTE 5:	Different working clock domains are independent and can have different precision.

NOTE 6:	The synchronicity budget for the 5G system is also applicable for two wireless links.

The 5G system shall provide a media dependent interface for one or multiple 802.1AS sync domains [22].

The 5G system shall provide an interface to the 5G sync domain which can be used by applications to derive their working clock domain or global time domain (Reference Clock Model).

The 5G system shall provide an interface at the UE to determine and to configure the precision and time scale of the working clock domain.

The 5G system shall be able to support arbitrary placement of sync master functionality and sync device functionality in integrated 5G / non-3GPP TSN networks.

The 5G system shall be able to support clock synchronization through the 5G network if the sync master and the sync devices are served by different UEs. (Flow of clock synchronization messages is in either direction, UL and DL.)

The 5G system shall provide a suitable means to support the management of the merging and separation of working clock domains, that is interoperable with the corresponding mechanisms of TSN and IEEE 802.1AS.

### 5.6.2	Clock synchronisation service performance requirements

Table 5.6.2-1: Clock synchronization service performance requirements for 5G System

| User-specific clock synchronicity accuracy level | Number of devices in one Communication group for clock synchronisation | Clock synchronicity requirement | 5G system clock synchronicity budget (note) | Service area | Scenario |
| --- | --- | --- | --- | --- | --- |
| 1 | Up to 300 UEs | <1 µs | < 900 ns | ≤ 100 m x 100 m | - Motion control - Control-to-control communication for industrial controller |
| 2 | Up to 300 UEs | <1 µs | < 900 ns | ≤ 1000 m x 100 m | - Control-to-control communication for industrial controller |
| 3 | Up to 10 UEs | <10 µs |  | ≤ 2500 m2 | - High data rate video streaming |
| 3a | Up to 100 UEs | <1 µs |  | ≤10 km2 | - AVProd synchronisation and packet timing |
| 4 | Up to 100 UEs | <1 µs | < 900 ns | < 20 km2 | - Smart Grid: synchronicity between PMUs |
| 5 | Up to 10 UEs | <50 µs |  | 400 km | - Telesurgery and telediagnosis |
| NOTE: The 5G system clock synchronicity budget is described in Clause 5.6.0. |  |  |  |  |  |

End of Change 1

Start of Change 2

Annex X (informative):	5GS Time Synchronization in integrated 5G/non-3GPP networks

## X.1	Introduction

The required clock synchronization accuracy is between the sync master and any sync device of the synchronization domain. In integrated 5G/non-3GPP networks, however, the path of the synchronization messages from the sync master to the sync device goes partially through the 5G system. Before and/or after the 5G system, there are portions that run through non-3GPP networks.

The 5GS time synchronization budget is the maximum absolute relative time error, the 5GS is allowed to contribute at most. This value should not be exceeded by the 5G system.

The sum of the 5GS time synchronization budget and the corresponding maximum time errors of the non-3GPP network parts need to stay within the required (overall) clock synchronization accuracy.

Clause X.2 contains several usage scenarios from different vertical domains, that use integrated 5G/non-3GPP networks and time synchronization. They provide some background information, how the usage of integrated 5G/non-3GPP networks with time synchronization in future is anticipated.

The network topology of the non-3GPP parts is important to derive the necessary maximum time error of the non-3GPP parts. The actual network structure and the topology constraints on the path of the synchronization messages depend on the area of concern, often on the characteristics of the specific application, mechanical constraints of the machinery, integration aspects, general network architecture, etc.

## X.2	Usage scenarios with time synchronization in integrated 5G/non-3GPP networks

### X.2.1	Manufacturing robots (industrial automation)

Figure X.2.1-1 shows a working clock domain with two manufacturing robots. Both manufacturing robots have a wireless connection to the 5G system and contain a 5G UE. There are two wireless links on the path of the synchronization messages. Both manufacturing robots need to be synchronized with the sync master with precision ±1 µs in order to work together properly.

Figure X.2.1-1: Use case scenario with two manufacturing robots (both wireless)

Inside the manufacturing robots, a wired non-3GPP communication network is used to connect the UE and the controller [C], sensors [S], and actuators [A]. This wired network has a line topology.

The line topology can have up to 16 hops from the UE to the furthest away network device.  The necessary number of hops depends on several things such as mechanical constraints and the placement of the UE.

Figure X.2.1-2 also shows a working clock domain with two manufacturing robots. However, the manufacturing robot on the right-hand side is connected through the wired non-3GPP factory network. There is one wireless link on the path of the synchronization messages.

Figure X.2.1-2: Use case scenario with two manufacturing robots (wireless/wired)

### X.2.2	Phasor Measurement Units (smart grid)

Phasor measurement units (PMUs) connect via 5G to a server in the smart grid backbone. The PMUs need a high synchronization precision between each other (±1 µs) in order to provide exact time stamping of voltage waves. These voltage waves are due to faults in a power line. The exact time stamping allows to compute the location of the fault in the power line.

The phasor measurment units (PMUs) might be equipped with a 5G UE that connects with the PMU through a switch (see Figure X.2.2-1). Due to the use of different time synchronization profiles in the 5G network and in the PMUs, an additional border clock is needed between the UE and the PMU. The border clock translates between the different PTP profiles, here into the power utility profile IEC 61850-9-3 [15].

Figure X.2.2-1: Use case scenario for PMUs in smart grid

End of Change 2

END OF PROPOSED CHANGES