3GPP TSG-SA1 Meeting #90	S1-202195

Electronic Meeting, 18 - 22 May 2020	(revision of S1-202151)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.832 | CR | 0025 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Time synchronization budget for the 5G system |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_eCAV |  |  |  |  |  | Date: |  |  | 2020-05-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The time synchronization budget for the 5G system is still open. Furthermore, some references need to be corrected or clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The open target value [x ns] in clauses 5.20.6 and 6.1 is replaced by an actual value. The “constraints on time synchronization” (Clause 5.20.1.3) are clarified to be specific to wired networks (IEC/IEEE 60802 does not consider wireless access) and references to 5G in this clause are removed. A note is also added that IEC/IEEE 60802 is still in draft status and hence is subject to change. The reference to an individual contribution to IEC/IEEE 60802 is removed and instead reference is made to the corresponding joint project of IEC/IEEE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be no concrete proposed value for the time synchronization budget for the 5G system. Unclear references to IEEE work in progress. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.20.1.2, 5.20.1.3, 5.20.6, 6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Start of the 1st change --------------------------------------------------------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains".

[3]	"IEEE Standard for Local and Metropolitan Area Networks--Timing and Synchronization for Time-Sensitive Applications," IEEE Std 802.1AS-Rev/D8.0, pp. 1-466, January 2019.

[4]	IEC 61158: "Industrial communication networks – fieldbus specification".

[5]	IEC 61907: "Communication network dependability engineering".

[6]	3GPP TS 22.186: "Enhancement of 3GPP support for V2X scenarios".

[7]	3GPP TS 22.261: "Service requirements for the 5G system".

[8]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[9]	IEEE 802.1Q-2018: "IEEE Standard for Local and Metropolitan Area Network--Bridges and Bridged Networks"

[10]	IEEE P802.1CS: "IEEE Draft Standard for Local and Metropolitan Area Networks -- Link-local Registration Protocol"

[11]	IEEE P802.1Qdd: "IEEE Draft Standard for Local and Metropolitan Area Networks -- Bridges and Bridged Networks -- Amendment: Resource Allocation Protocol (RAP)"

[12]	IEC/IEEE 60802: "Time-Sensitive Networking Profile for Industrial Automation", Joint Project of IEC SC65C/MT9 and IEEE 802 (https://1.ieee802.org/tsn/iec-ieee-60802/).

[13]	IEEE 802.3: "IEEE Standard for Ethernet" (http://www.ieee802.org/3/)"

[14]	Void

[15]		IEC/IEEE 61850-9-3: "Communication networks and systems for power utility automation – Part 9-3: Precision time protocol profile for power utility automation"

----------------------------------------------------- End of the 1st change --------------------------------------------------------------

----------------------------------------------------- Start of the 2nd change --------------------------------------------------------------

#### 5.20.1.2	Terminology

The term time synchronization or synchronicity budget of the 5G system corresponds to the maximum absolute relative time error of the 5G system.

Relative time error TER(A,B) is the difference between the time of clock A (TA) and the time of clock B (TB), i.e., TER(A,B) = TA – TB. (IEC/IEEE 60802 [12])

Maximum absolute relative time error max|TER| is the maximum of the absolute value of the relative time error. (IEC/IEEE 60802 [12])

Constant time error cTE is the time error or relative time error that is constant in time (i.e., static). (IEC/IEEE 60802 [12])

Dynamic time error dTE is the zero-mean, random component of the time error. (IEC/IEEE 60802 [12])

#### 5.20.1.3	Constraints on time synchronization in wired networks

Specific constraints on time synchronization in wired industrial networks have been identified in IEC/IEEE 60802 [12].

NOTE:	The IEC/IEEE 60802 profile document is a draft, which is subject to change. The latest draft at the time of writing is D1.1.

In time synchronization domains, the longest path between sync master and any sync device in the synchronization domain may be up to 64 hops. The required synchronization precision between the sync master and any sync device in the synchronization domain is ≤ 1 µs (maximum absolute relative time error max|TER| ≤ 1 µs (IEC/IEEE 60802 [12])).

Furthermore, constant time error and dynamic time error are distinguished in IEC/IEEE 60802 [12]. The limits of constant and dynamic time error are defined for each component of the path of the synchronization messages.

The time error limits are differently distributed in energy automation / smart grid.

The network time inaccuracy is required to be better than ±1 μs after crossing approximately 15 transparent clocks (50 ns time error each) or 3 boundary clocks (250 ns time error each) (IEC 61850-9-3 [15]).

The 5G network may be integrated with non-3GPP time synchonized networks also in energy automation. This may decrease the available time error budget for the 5G system.

----------------------------------------------------- End of the 2nd change --------------------------------------------------------------

----------------------------------------------------- Start of the 3rd change --------------------------------------------------------------

### 5.20.6	Potential New Requirements needed to support the use case

5G service performance requirements on the clock synchronization budget assigned to the 5G system in integrated TSN networks (5G network and non-3GPP networks, e.g. IEEE 802.1-based TSN networks):

[PR-5.20.6-001] The time synchronization budget assigned to the 5G system in integrated 5G TSN networks for the global time domain shall be at most 900 ns.

[PR-5.20.6-002] The time synchronization budget assigned to the 5G system in integrated 5G TSN networks for a working clock domain shall be at most 900 ns.

NOTE:	The clock synchronization precision between the sync master and any sync device in the synchronization domain is ≤ 1 µs in the above potential new requirements (in both, global time domain and working clock domain).

----------------------------------------------------- End of the 3rd change --------------------------------------------------------------

----------------------------------------------------- Start of the 4th change --------------------------------------------------------------

## 6.1	Industrial Ethernet integration

This clause contains merged and consolidated potential service requirements related to Industrial Ethernet integration, which includes time synchronization, different time domains, integration scenarios, and support for time-sensitive networking (TSN).

[MPR-22832-6.1-00a]	The 5G system shall be able to support four synchronization domains on a UE.

NOTE:	The four synchronization domains are used, for example, as two synchronization domains for global time and two working clock domains. One pair of global time and working clock is used as redundant synchronization domains for zero failover time.

[MPR-22832-6.1-00b]	The 5G system shall be able to support arbitrary placement of sync master functionality and sync device functionality in integrated 5G / non-3GPP TSN networks.

[MPR-22832-6.1-00c]	The 5G system shall be able to support clock synchronization through the 5G network if the sync master and the sync devices are served by different UEs. (Flow of clock synchronization messages is in either direction, UL and DL.)

[MPR-22832-6.1-00d]	The 5G System shall be able to support up to 128 synchronization domains (with different synchronization domain identifiers / domain numbers), including for UEs connected through the 5G network.

[MPR-22832-6.1-00e]	The 5G system shall provide a suitable means to support the management of the merging and separation of working clock domains, that is interoperable with the corresponding mechanisms of TSN and IEEE 802.1AS.

[MPR-22832-6.1-00f]	The 5G system shall support the fully distributed model for configuration of time-sensitive networking.

[MPR-22832-6.1-00g]	The 5G system shall support the fully distributed model for configuration of time-sensitive networking that is aligned with Multiple Stream Registration Protocol (MSRP, IEEE 802.1Q [9]), IEEE P802.1CS Link-local Registration Protocol (LRP) [10], and IEEE P802.1Qdd Resource Allocation Protocol (RAP) [11].

[MPR-22832-6.1-00h]	The 5G system shall support the user-network / network-network interface for the dynamic configuration of the fully distributed model for time-sensitive networking.

[MPR-22832-6.1-00i] The time synchronization budget assigned to the 5G system in integrated TSN networks for the global time domain shall be at most 900 ns.

[MPR-22832-6.1-00j] The time synchronization budget assigned to the 5G system in integrated TSN networks for a working clock domain shall be at most 900 ns.

NOTE:	The clock synchronization precision between the sync master and any sync device in the synchronization domain is ≤ 1 µs in the above potential new requirements (in both, global time domain and working clock domain).

Table 6.1-1: Relation between merged and new potential service requirements from use cases

| Merged Potential Service Requirement (clause 6) | Based on New Potential Service Requirements (clause 5) | Merged Potential Service Requirement (clause 6) | Based on New Potential Service Requirements (clause 5) |
| --- | --- | --- | --- |
| [MPR-22832-6.1-00a] | [PR-5.2.6-001] | [MPR-22832-6.1-00e] | [PR-5. 4.6-001] [PR-5. 4.6-002] |
| [MPR-22832-6.1-00b] | [PR-5.2.6-003] | [MPR-22832-6.1-00f] | [PR-5.15.6-001] |
| [MPR-22832-6.1-00c] | [PR-5.2.6-002] | [MPR-22832-6.1-00g] [MPR-22832-6.1-00i] [MPR-22832-6.1-00j] | [PR-5.15.6-002] [PR-5.20.6-001] [PR-5.20.6-002] |
| [MPR-22832-6.1-00d] | [PR-5.3.6-001] [PR-5.3.6-002] | [MPR-22832-6.1-00h] | [PR-5.15.6-003] |

----------------------------------------------------- End of the 4th change --------------------------------------------------------------