3GPP TSG-SA WG Meeting #91-e	S1-203043

Electronic Meeting, Aug. 24-28, 2020

Title:		DP on Network Names.

Agenda Item:	3

Source:	LG Electronics

Contact:	sungduck.chun@lge.com

Abstract: This contribution discusses issues on the Network Names

1. Introduction & Background

TS 22.261 includes following text:

#### 5.1.2.2 Legacy service support In principle, the 5G system shall support all EPS capabilities (e.g. from TSs 22.011, 22.101, 22.278, 22.185, 22.071, 22.115, 22.153, 22.173, 22.468),

TS 22.011 includes following text for PLMN selection.

#### 3.2.2.1 General In the following procedures the UE selects and attempts registration on PLMNs. In this TS, the term "PLMN Selection" defines a UE based procedure, whereby candidate PLMNs are chosen, one at a time, for attempted registration.

Observation: There is nothing specific There is no stage-1 requirement for ‘SNPN access mode’. Thus, HRNN does not apply.

TS 22.101 includes following text for PLMN Indication.

## A.3 Country/PLMN indication The country/PLMN indicator shows in which PLMN the UE is currently registered. This indicator is necessary so that the user knows when "roaming" is taking place and that the choice of PLMN is correct. Both the country and PLMN will be indicated. When more than one visited PLMN is available in a given area such information will be indicated.

TS 22.220 includes following text for HNB in section 5.4.2.

- A HNB Name present in the Operator CSG list shall take precedence over a HNB Name present in the Allowed CSG list, which in turn shall take precedence over the broadcasted HNB Name.

2. Discussion

2.1	PNI-NPN case

TS 22.220 specifies requirement for HNB/HeNB and CSG. As per the definitions in section 3.1 and general requirements in section 5.3.1, it is hard to conclude that all requirements in TS 22.220 apply to PNI-NPN. Also, there are discrepancy between TS 22.220 and what has been implemented in Rel-16 stage-3 for NPN.

Also, when SA1 developed Rel-16 stage-1 requirements for NPN, the requirements were not considered and evaluated against TS22.220

Observation: TS 22.220 is not stage-1 requirement for ‘PNI-NPN’.

2.2	SNPN case

Following NOTE is found in TS 22.261.

The 5G system shall support a mechanism for a UE to identify and select a non-public network. NOTE: Different network selection mechanisms may be used for physical vs virtual non-public networks.

Based on this NOTE, stage-2 defined ‘SNPN access mode’. However, regarding this SNPN access mode, there is no corresponding Stage-1 description and requirement.

Observation: There is no stage-1 requirement for ‘SNPN access mode’.

In fact, as the sections in TS 22.011 clearly states, the requirements are only for PLMN case. Because PLMN and SNPN are separate entities, the requirements in TS 22.101/011 does not apply for SNPN.

Conclusion: For NPN, no specific means for the delivery of NPN names are defined in stage-1.

3. Conclusion & Proposal

For Rel-16, it can be concluded that:

- There is no specific Stage-1 requirement that mandates the specific means for the delivery of NPN names.

- There is no specific Stage-1 requirement that specifies the relative priority among the means for the delivery of NPN names.

Thus, at least for Rel-16, potentially also including Rel-17, it is proposed to:

- Reply CT/CT1 that there is no stage-1 requirement for the delivery of HRNN, except already stage-3 agreed way using SIB broadcast.

For later releases, it is proposed:

- Later, if substantial motivation to add additional ways to deliver HRNN is found by downstream WGs, then SA1 will discuss which way is prioritized.