3GPP TSG-SA WG1 Meeting #91e	S1-203101

Online, 24 August – 02 September 2020

Title:	Use Case of Distributed Power Storage

Agenda Item:	7.9.1

Source:	China Electric Power Research Institute (CEPRI), ZTE

Contact:	xu.ling@zte.com.cn

yangdelong@epri.sgcc.com.cn,

Abstract: This contribution proposes a new use case of distributed power storage for FS_5GSEI

5.x	 Use case of Distributed Power Storage

5.x.1	Description

Distributed power includes various forms such as solar power, wind power, fuel cell and gas combined. It is generally distributed in the user site, or near locations to realize power generation, storage and supply. Distributed power generation has the characteristics of flexible location and decentralization, which adapts well to decentralized power demand, and has reduced the huge investment required for upgrading the transmission and distribution power grid. It also works as a backup for the large power grid to improve reliability of whole power supply. In storms, ice and snow weather, when the large power grid is severely damaged, distributed power sources can form islands or micro-grids on their own to provide emergency power to important users such as hospitals, transportation hubs, radio and television.

But the distributed power grid has brought new technical problems and challenges to the power grid operation. When the distributed power is connected to the large power grid, the large power grid will be changed from the original single power grid to a dual-power or even multi-power grid. The power flow on the distributed power grid becomes more complicated for that the user is becoming both the electricity user and the generator, and the current presents two-way flow and real-time dynamic changes. To improve the reliability, flexibility and efficiency of the distributed power grid, the communication system with a low latency, high reliability, massive connections and a high data rate is considered.

The distributed power grid which is comprised of residential, commercial and light storage users, requires to exchange information among the power management platform and the diverse distributed power storage devices. The information exchanged in the distributed power gird is not only to collect device status, but also to coordinate working flows of the distributed power storage equipment, to change the equivalent load characteristics, and to realize flexible power grid through load interaction etc..

The power storage device is plug-and-play and periodically collects the operating information, such as battery power, charge and discharge power, alarm information, etc., and transfers to the power storage cloud platform. The power storage cloud platform regularly maintains connections with the power storage devices and determines the online status and manages them. Further, it obtains the electricity, power, load and other information of the grid-connected users in the area from the power storage devices to support the decision-making of flexible interaction between the distributed power storage and the large power grid, or to analyze the user's electricity habits to guide the operation of distributed power storage.

5.x.2	Pre-Conditions

Distributed power system has the following functions: data acquisition and processing, active power adjustment, voltage and reactive power control, island detection, dispatch and coordination control, etc. It is mainly composed of distributed power storage management platform (DPSMP), the diverse distributed power device (DPD) and communication system.

The DPSMP is located in region center. And the diverse DPD can be deployed in buildings, indoors, outdoors, tunnels, ports and even moving vehicles which may be faced a poor communication signal condition or even out of network coverage.

The 5G system connects the DPDs with the DPSMP.

5.x.3	Service Flows

Every time after the DPD and the DPSMP establish a communication connection, the DPD reports its configuration data to DPSMP. For reliability, in general, the communications between DPSMP and DPD should be supported by multiple connections and backup each other.

The heartbeat data is always transmitted between the DSPMP and DPD to maintain a normal connection.

In the process of general service flow, there are three kinds of data exchange:

a. Command delivering: DSPMP sends control commands to the DSP. It always requires <10ms latency with 99.9% reliability communication service for control frequency etc.

b. Data reporting: equipment normal operation information, mainly including: energy storage battery, energy storage converter, AC and DC charging and discharging equipment and other current operating data. It always requires <1s latency.

c. Other data: For example, the DPD actively requests data, such as requesting the electricity price information from DPDMP, and the AC/DC charging and discharging facility sends relevant information about the current charging and discharging under abnormal conditions.

Based on the general service flow, in different scenarios, the data collection requirement is different. The following lists two typical scenarios in distributed energy storage service (Editor note: the data are from CEPRI).

Case1: data collection for power storage in rural area

In the typical scenario of power storage in rural area, it is always a large power storage in one location. Considering the data collection, taking 100Ah lithium iron phosphate batteries as an example, the integration of a 2MWh power storage container requires 6,250 batteries. So there are 6250 voltage and temperature collection points, and 780 current collection points, plus other 12-bit data e.g. alarms, internal auxiliary equipment such as air-conditioning, environmental monitoring and video. The collection data for the 2MWh power storage container is about 20,000 16-bit data and among them, the current related collection points is about 800, which need implement collection every millisecond and report every 10ms, other 13000 points need implement collection and report every second. Thus, there are two traffic models in the power storage station. One is for current flow with 40000 collection 16-bit data per millisecond, the others is 650000 collection 16-bit data per second.

Case2: data collection for distributed power storage in urban area

In urban area scenario, electric vehicle is one of the key DPDs. It is generally installed with about 7000 power batteries which requires 7000 voltages, 1000 voltages & temperature collection points at least. Further considering other collected data such as electronic control, charging and vehicle-grid interaction, for one electric vehicle, there is total about 10,000 16-bit data and among them, the current related collection data is more than 1000 16-bit data which need to be collected every millisecond and reported every 10ms, the others is more than 8000 16-bit data which need to be collected and reported every second.

5.x.4	Post-Conditions

In urban public building, community, industrial park, rural village, this kind of distributed energy storage system with the help of 5G system works well and can supply reliable power for users.

5.x.5	Existing features partly or fully covering the use case functionality

None

5.x.6	Potential New Requirements needed to support the use case

The 5G system shall be able to provide communication service with following performance.

|  |  | Transmission Frequency (Hz) | Message size (byte) | Experience data rate per storage location (bps) (note2) | Communication latency (ms) | Reliability | Storage node # /km2 (note3) | Active factor/km2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dense Urban | Virtual power storage: frequency modulation | 100 | UL:>20k | UL: >16M DL: >100k | DL:1 UL:1 | DL: >99.90% | >x*1k | 10% |
|  | Virtual power storage : other data collection (note1) | 1 | UL: 16k DL: >100k | UL: >128k DL: >100k | DL:<10 UL:<1000 | DL: >99.90% | >x*1k | 10% |
| Rural | Power storage station: frequency modulation | 100 | UL: 16k*50 | UL: > 640M DL: > 100k | DL:<1 UL:<1 | DL: >99.90% | >x*100 | 10% |
|  | Power storage station operation data collection: other data (note1) | 1 | UL: 26k*50 DL: >100k | UL: > 1.04M DL: > 100k | DL:<10 UL:<1000 | DL: >99.90% | >x*100 | 10% |
|  | Power storage station video | 1 | UL: 12.5M*50 DL: >100k | UL: >1G DL: >100k | DL:<10 UL:<1000 | DL: >99.90% | >x*100 | 10% |

NOTE1: In one power storage location, there are two traffic models for current and other data, and both need to be collected and reported.

NOTE2: this KPI is to require data rate in one power storage station which collects through multiple sensors and reports via multiple 5G connections at the same time.

NOTE3: this KPI is to require possible storage node number per KM2 which may deduce the traffic volume requirement through following formula:

(Current + other data + video) Experience data rate per storage location * (Storage node # /km2) * (Active factor/km2)

The 5G system shall be able to provide required communication service for distributed power storage where it is indoor, outdoor, underground, or in moving condition.

The 5G system shall be able to provide connection service for the plug-and-play power storage devices.

The 5G system shall be able to provide multiple connections for one power storage device.

The 5G system shall be able to support collecting current and other power storage data in one storage location at the same time.

The 5G system shall be able to provide security connection service according to different smart grid applications’ demands.