3GPP TSG-SA WG1 Meeting #91e	S1-203106

Online, 24 August – 02 September 2020

Title:	Use Case of Distributed Automation

Agenda Item:	7.9.1

Source:	ZTE, China Southern Power Grid

Contact:	xu.ling@zte.com.cn

Abstract: This contribution proposes a new use case of distributed automation for FS_5GSEI

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

[x]	China Southern Power Grid: "White Paper: 5G enables smart grid applications".

…

5.x	 Use case of distributed automation

5.x.1	Description

With the increased requirements for more reliable power supply, uninterrupted and continuous power supply, shorten the accident isolation time to milliseconds are required and support regional non-stop power services which make severe challenges for the main station in centralized distribution automation.

So intelligent distributed distribution automation has become one of the trends of the distribution automation grid development. Its characteristic lies in the distributed sinking of the processing logic of the original master station to the intelligent power distribution terminal, through the 5G communication between the terminals, intelligent judgment, analysis, fault location, fault isolation and non-fault area power supply restoration operations can be realized.

In this way, the fault handling process can be fully automated, the fault scope can be restricted and fault handling time of the distribution network can be decreased from minutes to milliseconds.

5.x.2	Pre-Conditions

Chinese Distributed Grid can be divided into urban and agricultural parts. In urban area, the power load is relatively concentrated, and the distributed grid working environment is better. But in the agricultural area, the power service range is very large, while the distributed grid has to face so many issues e.g. a large number of harmonic sources, three-phase unbalance, voltage flicker pollution.

The distributed automation grid system is mainly composed of distributed master station, distributed monitoring terminal and the communication system. The distribution master station is mainly used for information processing and human-computer interaction, and the distributed terminal is used for feeder status information collection and action execution; the communication system is to provide the communication link between the distribution master station and the distribution terminals.

5.x.3	Service Flows

- Data collection: The distributed terminals collect and report related status information to the distributed master station in real time.

- Fault detection and location: The distributed master station collects, processes, and stores the fault signal of the entire feeder loop, and judges the fault whether an instantaneous or permanent fault.

2a. When the distribution fault is an instantaneous fault, the feeder automatic processing process is not triggered.

2b. When the fault is a permanent fault, the master station will locate the feeder fault based on the signals of each power distribution terminal. The upstream power distribution terminal of the node will continue to send status information, while the downstream power distribution terminal will not report the fault signal. Therefore, in the corresponding fault node, only one switch should send out the fault signal. According to this feature, when a feeder fault occurs, the power distribution master station can locate fault point.

- Fault isolation: When the fault node is determined, according to the preconfigured action order, all switches around the faulty node will be notified and disconnected to realize effective identification and isolation of the faulty area.

- Fault restoration: For the restoration of power supply in power-loss areas, it is necessary to clarify the number and wiring of its tie switches, e.g. when a fault occurs in the cabinet of the one-in-two-out ring main unit, if two outgoing lines have isolated power-loss areas. The distributed master station requires the distribution terminals to report the number of tie lines and the number of contact switches in the power-loss area. Then, according to the preconfigured policy, the distributed master station sends a lot of commands to related distributed terminals to reconstruct the power distribution structure, and close the tie switch, to restore the power supply in the power-loss area.

- Restoration confirmation: After the power supply is restored in the power-loss area, the system needs the distribution terminals report status information to determine whether the restoration is well done.

5.x.4	Post-Conditions

It can identify and shorten the power feed accident isolation time to milliseconds and support regional non-stop power services.

5.x.5	Existing features partly or fully covering the use case functionality

None

5.x.6	Potential New Requirements needed to support the use case

The 5G system shall be able to provide communication service with following performance (note1) at the same time.

| User experienced Data rate (bps) | Latency (ms) | Latency jitter (us) | Synchronicity budget requirement (us) | Reliability % | Connection density | coverage | Isolation and safety |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2-10M | <10 | <50 (note2) | <10 | 99.999 | x*10/km2 |  | Belong to working I/II, need highest isolation with others |

NOTE1: refer [x] published by China Southern Power Grid.

NOTE2: the latency jitter is required for the communication link implemented between active and standby switch when their distance <40km.

The 5G system shall be able to provide multiple connections among the distributed master station and the distributed terminals to back up each other.

The 5G system shall be able to provide connection service wherever the power terminal is indoor, outdoor, low and medium altitude, or underground.

The 5G system shall be able to provide highest isolation level service for smart grid application.

Editor note: the definition of different isolation level for smart grid need to be studied.