3GPP TSG-SA WG1 Meeting #91e	S1-203153

Electronic Meeting, 24 August - 2 September 2020	(revision of S1-20xxxx)

Title:	Use Case on efficient routing for UE-to-UE communications via residential gateway

Agenda Item:	7.11.1

Source:	Huawei

Contact:	Yuan Wang (wangyuan5@huawei.com)

Abstract: This document proposes FS_Resident use case on efficient routing for UE-to-UE communications via residential gateway.

**************** All New Changes ****************

5.x	Use Case on efficient routing for UE-to-UE communications via residential gateway

5.x.1	Description

This use case assumes that multiple base stations were already deployed in individual rooms behind the residential gateway, to provide better coverage at home. This use case is to enable efficient routing for the communications between UEs via the residential gateway.

5.x.2	Pre-Conditions

Multiple indoor base stations are deployed in Alice’s home behind the residential gateway. In particular, one base station is deployed at the attic and another one is deployed at the basement.

A security sensor is deployed at the basement which is capable of detecting smoke or fire. The sensor is a 3GPP-capable UE which is connecting to the indoor base station in the basement.

5.x.3	Service Flows

Alice is working alone at home, staying in her office in the attic. Her smartphone is connecting to the indoor base station deployed in the attic.

Suddenly there is a fire in the basement.

The security sensor detects the fire and smoke, and then immediately sends an alarm to Alice’s smartphone.

The alarm message first reaches the residential gateway via the indoor base station in the basement, and then routed by the residential gateway to Alice smartphone via the indoor base station in the attic.

5.x.4	Post-Conditions

Alice reads the alarm message from her smartphone. After double-check the situations in the basement, Alice makes the call to the fire policy.

5.x.5	Existing features partly or fully covering the use case functionality

The 5G system shall support on-demand establishment of UE to UE, multicast, and broadcast private communication between members UEs of the same 5G LAN-VN. Multiple types of data communication shall be supported, at least IP and Ethernet.

5.x.6	Potential New Requirements needed to support the use case

[PR. 001] The 5G system shall be able to provide E2E QoS control for the communication path between the UE and the residential gateway via the indoor base station.

[PR. 002] The 5G system shall be able to support efficient routing for communication between UEs via the residential gateway.

************* End of First Change ***************