3GPP TSG-SA WG1 Meeting #91e 	S1-203167

Electronic Meeting, 24 Aug - 02 Sep Aug 2020	(revision of S1-20xxxx)

Title:	Use case of Smart Metering

Agenda Item:	7.9.1

Source:	China Telecom

Contact:	Heng Wang, wangh26@chinatelecom.cn

Abstract: This document proposes a use case and related potential requirements to be included in FS_5GSEI TR 22.867 version 0.0.0.

**************** First Change ******************

5.X	Use case of Smart Metering

5.x.1	Description

Instead of recording and sending the metering data from a traditional wired electricity meter unit, electricity metering collecting can be executed by an UE integrated smart meter unit, including UE integrated community facilities, smart household appliances, etc. Smart meter units can send real-time metering data to the server in the Power Enterprise through mobile networks. In this way, the Power Enterprise will reduce the cost of cable for data transiting and make the monitoring flexible that helps meet the benefits and bring the new chance.

Due to massive electricity meter boxes and household appliances keep online at the same time, the amount of connections the mobile network maintains simultaneously is so enormous that challenging to the mobile network capacity.

Furthermore, it helps people save energy and money with understanding their home’s energy use as the Power Enterprise gives the data and information need to take action. Also, it will notice what happening in your home comes in handy, e.g. apps installed in smart phone can push warning of problems before abnormal appliances get out of hand at or away from home.

5.x.2	Pre-conditions

UE A is a smart electricity meter unit, e.g. smart refrigerator. UE A is power on.

Tom is the owner of UE A, and already bound it in the Power Enterprise App installed in his smart phone.

Server A is a server deployed in the Power Enterprise.

5.x.3	Service Flows

- UE A report metering data to Server A periodically, the period granularity may be second, minute, etc.

- Tom opens his Power Enterprise App and checks the accumulating power usage, also further searches the usage in daily or hourly pattern from the history.

- Server A performs accurate billing based on the reported metering data and pushes Tom the bill.

- Tom opens his Power Enterprise App and pays the bill.

5.x.4	Post-conditions

Server A can analyze the data for abnormal power usage and alert Tom. Tom asks after-sales service and the engineer removes the hidden peril.

The Power Enterprise asks Tom to open the data and give Tom a discount for exchange, If Tom permits, the Power Enterprise will give the metering data to the manufacturer of UE A so that the manufacturer can optimize efficiency of the next generation smart refrigerator.

5.x.5	Existing features partly or fully covering the use case functionality

None.

5.x.6	Potential New Requirements needed to support the use case

The 5G system shall be able to provide communication service with following performance at the same time.

| Payload (Bytes) | Tx rate (Message/Sec) | Latency (ms) | Reliability (%) | Connection density (Connections/km2) | Coverage |
| --- | --- | --- | --- | --- | --- |
| < 64 | 100 | < 200 | 99.9 | 10000 | Indoor, outdoor, Low and medium-altitude Underground |

************* End of First Change ***************