3GPP TSG-SA WG1 Meeting # 91-e	S1-203298

# Electronic Meeting, Aug, 24 – Sep, 2, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.263 | CR | 0008 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Multicast Requirement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | AVPROD |  |  |  |  |  | Date: |  |  | 2020-08-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Section 6.3 includes requirements on the packet error rate for multicast traffic. Both table 6.3.1-1 and table 6.3.2-1 list multicast requirements. While table 6.3.1-1 includes column for ‘PER’, table 6.3.2-1 does not have requirement on PER. Thus, it is unclear how to interprete this inconsistency. When a multimedia stream is transmitted in a multicast manner over radio interface and when there are many UEs receiving the same multimedia stream, the requirement on packet error rate (PER) should define how many UEs are met. However, the table in 6.3.1-1 does not specify this. As a result, downstream WGs can not verify whether their solutions meet SA1 requirement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Delete the column for PER from the table 6.3.2-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear requirements remain in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 6.3	Multicast performance requirements

### 6.3.1	Audio and video production applications

Table 6.3.1-1: Performance requirements for low latency deterministic periodic traffic with multicast service.

| Profile | # of active UEs | # of UL streams | # of DL streams | UE Speed | Service Area | E2E latency (Note 1) | Transfer interval (Note 1) |  | Data rate UL | Data rate DL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Integrated audience services | 50000 | - | 30 multicast streams | 5 km/h | 1.5 km x 1.5 km | 7 ms DL | 3 ms |  | - | 200 kbit/s |
| Intercom system | 1000 | 240 (Note 4) | 30 multicast streams | 5 km/h | 1.5 km x 1.5 km | 7 ms DL 7 ms UL | 3 ms |  | 100 kbit/s | 100 kbit/s |
| NOTE 1: Transfer interval refers to periodicity of the packet transfers. It has to be constant during the whole operation. The value given in the table is a typical one, however other transfer intervals are possible as long as the end-to-end latency is ≤ (10 ms – Transfer interval). NOTE 2: Packet error rate is related to a packet size of (Transfer interval × data rate). Packets that do not conform with the end-to-end latency are also accounted as error. NOTE 3: The given requirement for a packet error rate assumes a uniform error distribution. The requirement for packet error rate is stricter if packet errors occur in bursts. NOTE 4: The UL stream originating from a UE may be the source of a DL multicast stream. |  |  |  |  |  |  |  |  |  |  |

### 6.3.2	Medical applications

Table 6.3.2-1: Performance requirements for low latency ultra-reliable imaging/video multicast traffic for medical applications

| Requirement | Characteristic parameter |  |  |  |  | Influence quantity |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Communication service availability: target value in % | Communication service reliability: Mean Time Between Failure | End-to-end latency: maximum | Bit rate | Direction | Message Size [byte] | Survival time | UE speed (km/h) | # of UEs connection | Service Area |
| UHD medical video over NPNs | >99.99999 | >1 day | <1 ms | < [50 Gbits/s] | DL | ~1500 - ~9000 (note 1) | ~8ms | stationary | <10 | 100 m2 |
| UHD medical video over PLMNs | >99.9999 | >1 day | < 250 ms | < [2 Gbit/s] | DL | ~1500 - ~9000 (note 1) | ~16 ms | stationary | <10 | 400 km |
| NOTE 1: MTU size of 1500 bytes is not generally suitable to gigabits connections as it induces many interruptions and loads on CPUs. On the other hand, Ethernet jumbo frames of up to 9000 bytes require all equipment on the forwarding path to support that size in order to avoid fragmentation. |  |  |  |  |  |  |  |  |  |  |