3GPP TSG-SA WG1 Meeting #91e 	S1-203415

Electronic Meeting, 24 Aug - 02 Sep Aug 2020	(revision of S1-203354)

Title:	Use Case of Smart Transportation Metro/Bus Validation

Agenda Item:	7.10.1

Source:	China Telecom, Xiaomi

Contact:	Heng Wang (wangh26@chinatelecom.com)

Abstract: This document proposes a use case and related potential requirements to be included in  FS_Ranging TR 22.855 version 0.0.0.

**************** First Change ******************

5.x	Use Case of Smart Transportation Metro/Bus Validation

#### 5.x.1	Description

Nowadays, heavy passenger flow shock the current public transportation system. However, passengers have to actively touch the billing device using public transportation card, which is so slow at rush time that it is easy to meet congestion. This problem can be effectively solved through the hands free metro/bus validation. It determines whether the user gets on or off the bus according to the relative positioning result, and then provides the information about getting on or off the bus/metro to a billing node. The billing node automatically deducts the corresponding fee from the user's account.

At rush time, crowd get close to an entrance simultaneously, and the Ranging module inside the entrance performs positioning process with lots of smart phones (e.g. 20 smart phones) at the same time. Since entrance machines are always built in a row closely, one smart phone may perform Ranging procedures with 2 or more entrances simultaneously while it appears within their effective ranging distances (e.g. ranging distance < 10m). Then, the smart phone shall begin data interaction with higher Ranging availability to its toward entrance once the two UEs fairly near (e.g. ranging distance < 2m) to each other.  In this way, smart phones can distinguish which entrance is the nearest one to enter without redundant data interactions to other entrances.

#### 5.x.2	Pre-Conditions

Tom downloads an app on his smart phone with Ranging module. The hands free metro/bus validation function is enabled on this APP, and the automatic payment function is also enabled.

The metro and bus system have Ranging module, which can do ranging with the smart phone.This Ranging module can use a combination of 3GPP technologies and non-3GPP technologies. This includes, but not limited to, GNSS (e.g. BeiDou, Galileo, GLONASS, and GPS), Terrestrial Beacon Systems (TBS), Bluetooth, WLAN, RFID, UWB and sensors.

#### 5.x.3	Service Flows

- Tom carries his smart phone near the metro entrance.

- After the metro entrance detects the smart phone, it implements relative positioning with the smart phone.

- When Tom gets close to the metro entrance from outside the station, if the relative positioning results reach a certain distance, the entrance is automatically opened.

- The metro entrance detects the change of position relative to the mobile phone and determines whether Tom enters the station. If Tom enters the station, continue the following procedure, or close the entrance in time.

- Tom has completed his trip. When Tom gets close to the metro exit from inside the station, if the relative positioning result reaches a certain distance, the exit is automatically opened.

- The metro exit detects the change of position relative to the mobile phone and determines whether Tom leaves the station. If Tom leaves the station, continue the following procedure, or close the exit in time.

#### 5.x.4	Post-Conditions

The billing node get his trip from the metro system, and deducts the corresponding fee from the Tom’s account.

- If the deduction is successful, the procedure ends.

- If the deduction is unsuccessful, the hands free metro/bus validation function will be suspended. Tom needs to manually complete the payment on the APP to re-enable the function.

#### 5.x.5	Existing features partly or fully covering the use case functionality

None.

#### 5.x.6	Potential New Requirements needed to support the use case

The 5G system shall be able to support for a UE to discover other UEs supporting Ranging.

The 5G system shall be able to support one UE to perform Ranging with [50]UEs simultaneously.

The 5G system shall be able to provide ranging service with following performance KPIs.

| Ranging distance (meters) | Ranging distance accuracy (cm) | Latency (ms) | Availability (%) | Relative UE velocity (km/h) |
| --- | --- | --- | --- | --- |
| [0.1] - [2] | < [10] | < [50] | [99] | < [3] |
| [2] - [10] | < [10] | < [50] | [95] | < [5] |

************* End of First Change ***************