3GPP TSG-SA WG1 Meeting #91e	S1-203102

Online, 24 Augest – 02 September 2020

Title:	 FS_Ranging Use case: Distance based Smart Home Device Control

Agenda Item:	7.10.1

Source:	Xiaomi

Contact:	Xiaowei Jiang (jiangxiaowei@xiaomi.com)

Abstract: This contribution propose a Smart Home device control use case for FS_Ranging TR22.855

**************** First Change ******************

5.x	 Distance based Smart Home Device Control Use Case

5.x.1	Description

Smart speaker has been playing a more and more important role in smart home. People tends to use smart speaker to control smart devices given its convenience, especially for simple interactions, such as turn on TV/light.

In a typical smart home scenario, there might be tens of smart devices, many of which may be with the same type. For example, people may have 5-10 smart lights, 2-3 smart TVs, etc.  As a consequence, user has to tell the smart speaker which device he or she intends to control. Often, it requires user to give a different name for the devices. To make things worse, user may have multiple smart speakers in different rooms to be able to pick up user’s voice, then the system may even not know which smart speaker to respond to user’s voice. Ranging capable devices can simplify the interaction.

5.x.2	Pre-Conditions

There are many smart Ranging enabled devices in Alice’s home, including:

- Smart TV-1, smart light A and B, smart speaker A are in bedroom

- Smart TV-2, smart light C, D and E, smart speaker B are in living room

Alice wears a smart Ranging enabled watch.

The smart home devices are capable of determining the range of Alice’s wearable device.

The smart speaker is capable of recognizing Alice’s voice and identify her.

5.x.3	Service Flows

Alice enters the door of the her house;

Alice’s smart watch discovers smart TV-1/TV-2, smart speaker A/B, smart light A/B/C/D;

Ranging is performed between the wearable device and the discovered smart speakers;

The nearest smart speaker is chosen as the responder speaker for Alice;

When Alice is moving in the house, responder speaker is updated based on the latest ranging results;

Alice enters the bedroom, and would like to turn on the nearest light in the bedroom;

Alice arouse the smart speaker and tell it to turn on the light;

The responder speaker responds, and  the light nearest to the user is switched on.

5.x.4	Post-Conditions

Alice’s smart home system always knows which smart speaker to respond to alice, and knows which smart device is closest to alice, and thus being able to respond to alice’s command  by choosing the nearest device if multiple of the same type devices exist and user doesn’t tell which one to control.

5.x.5	Existing features partly or fully covering the use case functionality

None

5.x.6	Potential New Requirements needed to support the use case

The 5G system shall be able to support Ranging enabled UEs to discover other Ranging enabled UEs.

The 5G system shall be able to support a ranging enabled UE to perform ranging with more than [20] UEs simultaneously and with latency < [50ms].

The 5G system shall be able to support ranging with [10cm] distance accuracy in LOS environment.

The 5G system shall be able to support ranging with service availability > [99%].

The 5G system shall be able to support ranging with ranging distance [0-20] meter.

************* End of First Change ***************