3GPP TSG-SA WG1 Meeting #91e 	S1-203169

Electronic Meeting, 24 Aug - 02 Sep Aug 2020	(revision of S1-20xxxx)

Title:	Use Case of Smart Transportation Metro/Bus Validation

Agenda Item:	7.10.1

Source:	China Telecom, Xiaomi

Contact:	Heng Wang (wangh26@chinatelecom.com)

Abstract: This document proposes a use case and related potential requirements to be included in  FS_Ranging TR 22.855 version 0.0.0.

**************** First Change ******************

5.x	Use Case of Smart Transportation Metro/Bus Validation

#### 5.x.1	Description

Nowadays, heavy passenger flow shock the current public transportation system. However, passengers have to actively touch the billing device using public transportation card, which is so slow at rush time that it is easy to meet congestion. This problem can be effectively solved through the hands free metro/bus validation. It determines whether the user gets on or off the bus according to the relative positioning result, and then provides the information about getting on or off the bus/metro to a billing node. The billing node automatically deducts the corresponding fee from the user's account.

#### At rush time, crowd get close to the entrance simultaneously, and the positioning module inside the entrance performs positioning process with lots of smart phones (e.g. 20 smart phones) at the same time. Even the transmitting information is simple (e.g. username and password encrypted in a small packet, typically 64 bytes), there is still a big challenge to the network processing a large amount of concurrent procedures (e.g. 200 procedures = 10 entrance working simultaneously × 20 smart phones connected to each entrance).

#### There can be two kind of areas for such process. One is connecting area (e.g. ranging distance < 10m) in which the two UE only performs ranging connecting procedures. The other is processing area (e.g. ranging distance < 2m) in which the two UE begin to interact each other for data transmission.

#### 5.x.2	Pre-Conditions

Tom downloads an app on his smart phone with positioning module. The hands free metro/bus validation function is enabled on this APP, and the automatic payment function is also enabled.

The metro and bus system have positioning module, which can do ranging with the smart phone.This positioning module can use a combination of 3GPP technologies and non-3GPP technologies. This includes, but not limited to, GNSS (e.g. BeiDou, Galileo, GLONASS, and GPS), Terrestrial Beacon Systems (TBS), Bluetooth, WLAN, RFID, UWB and sensors.

#### 5.x.3	Service Flows

- Tom carries his smart phone near the metro entrance.

- After the metro entrance detects the smart phone, it implements relative positioning with the smart phone.

- When Tom gets close to the metro entrance from outside the station, if the relative positioning results reach a certain distance, the entrance is automatically opened.

- The metro entrance detects the change of position relative to the mobile phone and determines whether Tom enters the station. If Tom enters the station, continue the following procedure, or close the entrance in time.

- Tom has completed his trip. When Tom gets close to the metro exit from inside the station, if the relative positioning result reaches a certain distance, the exit is automatically opened.

- The metro exit detects the change of position relative to the mobile phone and determines whether Tom leaves the station. If Tom leaves the station, continue the following procedure, or close the exit in time.

- The billing node get his trip from the metro system, and deducts the corresponding fee from the Tom’s account.

- If the deduction is successful, the procedure ends.

- If the deduction is unsuccessful, the hands free metro/bus validation function will be suspended. Tom needs to manually complete the payment on the APP to re-enable the function.

#### 5.x.4	Post-Conditions

None.

#### 5.x.5	Existing features partly or fully covering the use case functionality

None.

#### 5.x.6	Potential New Requirements needed to support the use case

The  5G system shall be able to authorize a measurement of distance between UEs.

The 5G system shall be able to authorize a measurement of distance between UEs when not served by NG-RAN.

The 5G system shall be able to support for a UE to discover other UEs supporting measurement of distance.

The 5G system shall be able to support [200] concurrent Ranging procedures.

The Ranging capable UE shall be able to perform Ranging with [20] UEs simultaneously.

The 5G system shall be able to provide communication service with following performance at the same time.

| Ranging distance (meters) | Distance accuracy (cm) | Payload (Bytes) | Max end to end latency (ms) | Availability (%) | Coverage | Moving speed (km/h) |
| --- | --- | --- | --- | --- | --- | --- |
| 0.1 - 2 | < 10 | 4-64 | < 50 | 99 | Indoor, outdoor, Low-altitude, Underground | < 5 |
| 2 - 10 | < 10 | 4-64 | < 50 | 95 | Indoor, outdoor, Low-altitude, Underground | < 5 |

************* End of First Change ***************