3GPP TSG-SA WG1 Meeting #92e	S1-210046

Electronic Meeting, 2 February – 4 March 2021

Title:	FS_5TRS Consolidation of potential requirements and KPIs

Agenda Item:	7.8.1

Source:	Nokia, Nokia Shanghai Bell

Contact:	Betsy Covell	betsy.covell@nokia.com

Abstract: This contribution provides the consolidated potential requirements and consolidated potential KPIs for the study.  The following changes from the content of the use cases have been made in the process of consolidation:

- [PR 5.3.6-1]  is modified to correct the grammar with the following deletion: The 5G system shall support a mechanism for a 3rd party application to mutually authenticate a 5G timing resiliency service.

- [PR 5.2.6.1-1]  is modified for clarity with the following insertion: The 5G system shall support a mechanism to monitor for timing service failure (e.g., detect QoS degradation).

- [PR 5.2.6.1-2] The 5G system shall be able to indicate to devices (e.g., UEs, application servers) using a timing service that they need to use an alternate source (e.g., use 5G system with internal holdover capability or an alternate source, e.g. atomic clock, Sync over Fiber, TBS).

and [PR 5.2.6.1-5] The 5G system shall be able to detect and indicate to devices (e.g., UEs, application servers) when GNSS reference timing signals are restored for network time synchronization.

are combined as follows:

[CPR 8.1.2-4] The 5G system shall be able to indicate to devices (e.g., UEs, application servers) using a timing service that there is a change in the timing source (e.g., to 5G system internal holdover capability, atomic clock, Sync over Fiber, TBS, GNSS).

- [PR 5. 1.6-2] is modified to allow detection of viability of timing signals from any timing source as follows: The 5G system shall be able to detect when reference timing signals (e.g., from GNSS or other timing source) are no longer viable for network time synchronization.

Proposed New Text:

# 7	Considerations

## 7.1	Potential security considerations

[CPR 7.1-1] The 5G system shall support a mechanism for a 3rd party application to mutually authenticate a 5G timing resiliency service.

[CPR 7.1-2] The 5G system shall support a mechanism for a 3rd party application server to verify authorization to use a 5G timing resiliency service.

[CPR 7.1-3] The 5G system shall support a mechanism to monitor and verify authenticity of the timing source.

[CPR 7.1-4] The 5G system shall support a mechanism to monitor and verify the integrity of the received clock signals.

## 7.2	Potential charging considerations

[CPR 7.2-1] The 5G system shall be able to collect charging information based on the timing source used for timing synchronization (e.g., the source in use, the viability level of the source at the time of a change, start and stop of source usage).

[CPR 7.2-2] The 5G system shall be able to collect charging information per UE for use of a timing service (e.g., start/stop time and source used by a UE, timing service used by UE, holdover capability of the service).

[CPR 7.2-3] The 5G system shall be able to collect charging information on 5G system timing resiliency service (e.g., service KPIs, holdover capability, number of UEs using the service).

[CPR 7.2-4] The 5G system shall be able to collect charging information per application server or 3rd party application server (e.g., timing service used, service KPIs, holdover capability, number of UEs using the service, timing source).

# 8	Consolidated potential requirements and KPIs

## 8.1	Consolidated potential requirements

### 8.1.1	General

[CPR 8.1.1-1] The 5G system shall be able to receive accurate timing signals from one or more independent timing source(s), which can offer a timing alternative to GNSS, e.g. TBS/MBS [7] [8]], Sync over Fiber [7].

[CPR 8.1.1-2] The 5G system shall be able to maintain accurate time synchronization as appropriate for the supported applications in the event of degradation or loss of GNSS timing signals.

[CPR 8.1.1-3] The 5G system shall support a mechanism to provide timing service to UEs and application servers with specific KPIs (e.g., accuracy, interval, coverage area).

[CPR 8.1.1-4] The 5G system shall support a mechanism to manage and correlate (e.g., compare, prioritize, correct) clock information from multiple time sources in the network.

[CPR 8.1.1-5] The 5G system shall support a mechanism to configure granularity of the timestamp for cases where UE/DS-TT performs the timestamping of the financial transaction.

### 8.1.2	Monitoring and Reporting

[CPR 8.1.2-1] The 5G system shall support a mechanism to monitor for timing service failure (e.g., detect QoS degradation).

[CPR 8.1.2-2] The 5G system shall be able to detect when reference timing signals (e.g., from GNSS or other timing source) are no longer viable for network time synchronization.

[CPR 8.1.2-3] The 5G system shall support a mechanism to monitor time distribution at time equipment within the network for reporting timestamp measurement uncertainty.

[CPR 8.1.2-4] The 5G system shall be able to indicate to devices (e.g., UEs, application servers) using a timing service that there is a change in the timing source (e.g., to 5G system internal holdover capability, atomic clock, Sync over Fiber, TBS, GNSS).

[CPR 8.1.2-5] The 5G system shall support mechanisms to calibrate and report time information for network entities involved in the timing distribution chain.

### 8.1.3	Service Exposure

[CPR 8.1.3-1] The 5G system shall support a mechanism for a 3rd party application server to request a timing service with specific KPIs (e.g., accuracy, interval, coverage area).

[CPR 8.1.3-2] The 5G system shall support a mechanism to report UTC traceability information to 3rd party applications.

### 8.1.4	Holdover

[CPR 8.1.4-1] The 5G system shall support a holdover capability (maintaining UTC required accuracy) of up to 24h.

[CPR 8.1.4-2] A time synchronization device connected to the 5G system shall support a holdover capability of >5s.

## 8.2	Consolidated potential KPIs

Table 8.2-1: Timing resiliency service performance requirements for 5G System

| Use-case | Holdover Time | Sync Target | Sync accuracy | Service area | Mobility | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| Power grid (5G network) | Up to 24 hour | UTC (note 1) | <250ns-1000ns [3], [9] (note2) | < 20 km2 | low | When 5G System provides direct PTP Grandmaster capability to sub-stations |
| Power grid (time synchronization device) | >5 s | UTC (note 1) | <250ns-1000ns [3], [9] (note2) | < 20 km2 | Low | When 5G sync modem is integrated into PTP grandmaster solution (with 24h holdover capability) at sub-stations) |
| Note 1: A different synchronization target is acceptable as long as the offset is preconfigured when an alternatively sourced time differs from GNSS. In this case, a 5G end device shall provide PPS output which can be used for measuring the difference. Note 2: Use case [New] in [9] illustrates the different accuracy measurements based on different configurations needed to support the underlying requirements from IEC [3]. The range is between 250 ns and 1000 ns. The actual requirement depends on the specific deployment. |  |  |  |  |  |  |

Table 8.2-2: Level of accuracy for members or participants of a trading venue [13, 14]

| Type of trading activity | Maximum divergence from UTC | Granularity of the timestamp (note 1) |
| --- | --- | --- |
| Activity using high frequency algorithmic trading technique | 100µs | ≤1µs |
| Activity on voice trading systems | 1s | ≤1s |
| Activity on request for quote systems where the response requires human intervention or where the system does not allow algorithmic trading | 1s | ≤1s |
| Activity of concluding negotiated transactions | 1s | ≤1s |
| Any other trading activity | 1ms | ≤1ms |
| Note 1: Only relevant for the case where the time synchronization service assists in configuring the required granularity for the timestamp (for direct use), otherwise it will be configured separately as part of the financial transaction timestamp process. |  |  |