3GPP TSG-SA WG1 Meeting #93e 	S1-210169

Electronic Meeting, 22 Febuary – 4 March 2020	(revision of S1-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.104 | CR | xxxx | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding energy efficiency use cases for positioning to the ANNEX A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Fraunhofer IIS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | EEHPP |  |  |  |  |  | Date: |  |  | 2021-xx-xx |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The need for low power highly accurate positioning is imminent in vertical domains. Therefore, example use cases with requirements for low power highly accurate positioning should be added to the informal Annex of 22.104. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add use cases with requirements for low power highly precise positioning to the Annex A (service performance requirements) of 22.104 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The energy efficiency apects for industrial IoT positioning scenarios are not addressed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the Change---

# A.7	Low Power Highly Accurate Positioning

## A.7.1	Overview

Low power highly accurate positioning is an integral part of a considerable number of industrial applications. The total energy needed for a specific operation time for such a low power highly precise positioning optimized IoT-device is a combination of energy for positioning (varies depending on the used positioning method), energy for communication/synchronization and a difficult to predict factor to take additional losses through e.g. security, power management, microcontroller and self-discharge of batteries into account.

Examples for low power highly precises positoaning are asset tracking in process automation, tracking of vehicles and tool tracking.

## A.7.2	Use cases for low power highyl accurate positioning

Following table gives an indication of the required operation time of the 5G enabled IoT device and duty cycle for the updated position information for specific use cases

Table A.7.2-1: Service performance requirements for low power highly precises positioning

| Use Cases for low power highly precises positioning sorted by application | Horizontal accuracy | Corresponding aervice level (22.261) | Positioning interval/ duty cycle | battery life time/ minimum operation time |
| --- | --- | --- | --- | --- |
| Process automation – plant asset management |  |  |  |  |
| Turbine Construction (palette tracking) | < 1 m | Service Level 3 | 5 seconds - 15 minutes | 18 month |
| Intralogistics (sequence container) | < 1 m | Service Level 3 | 1 second | 6 - 8 years |
| Asset Tracking | 2-3 m | Service Level 2 | < 4 seconds | > 6 month |
| Dolly Tracking (Outdoor) | 10 m | Service Level 1 | on request | 24 month |
| Flexible, modular assembly area in smart factories (for autonomous vehicles, only for monitoring purposes)) |  |  |  |  |
| Production line (vehicles in line, distance 1,5 meter) | 30 cm | Service Level 5 | 1 second | 6 -8 years (no strong limitation in battery size) |
| Flexible, modular assembly area in smart factories (for tracking of tools at the work-place location) |  |  |  |  |
| Tool Tracking | < 1 m | Service Level 3 | no indication | 1 work shift - 8 hour (up to 3 days, 1 month for inventory purposes) |
| Flexible, modular assembly area | < 1 m | Service Level 3 | 15-30 seconds | 6 - 12 month |
| Tool Assignment (assign tool to vehicle, left/right) | 30 cm | Service Level 5 | 250 ms | 18 month |

## ---End of the Change---