3GPP TSG-SA WG1 Meeting #93e	  S1-210171

Online, 22 February – 04 March 2021

Title:		Use Case of power distribution grid state estimation service

Agenda Item:	7.9.1

Source:	Ericsson

Contact:	Robert Farac (robert.farac@ericsson.com)

Abstract: This contribution proposes a new use case of power distribution grid state estimation service.

Acknowledgment

This contribution proposes a new use case of power distribution grid state estimation service. This contribution has been prepared in collaboration with RWTH University of Aachen Prof. Monti, Antonello. The use case has been implemented and tested in several European power distribution grids in EU project SOGNO (https://www.sogno-energy.eu/).

Discussion

-

Proposal

It is proposed to agree to add the following change to TR 22.867.

Begin First Change

2		References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	                       3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	SunWeiqing etc.  Generalized Energy Storage Control Strategies on User Side in Power Ancillary Service Market.  Automation of Electric Power System Vol.44 No.2 Jan.25, 2020

[3]	China Southern Power Grid, China Mobile, HuaWei: "White Paper: 5G enables smart grid applications".http://www.sgcio.com/technology/anli/2018/0716/86656.html

[4]  	GB/T 1936-2018 Guidelines of cyber security protection for electric power system supervision and control http://www.howtoabc.com/book8/14078-0.html, https://www.secrss.com/articles/24761

[5]	IEEE C37.2-2008 IEEE Standard Electrical Power System Device Function Numbers, Acronyms, and Contact Designations.

[6]	IEC TR 61850-90-1:2010, Communication Networks and Systems for Power Utility automation – Part 90-1: Use of IEC61850 for the communication between substations.

[7]	IEEE 1588-2019 – IEEE Standard for a precision clock synchronization protocol for networked measurement and control systems.

[8]	IEEE Guide for Application of Digital Line Current Differential Relays Using Digital Communication.

[9]	61850-9-3-2016 – IEC/IEEE International Standard - Communication Networks and Systems for Power Utility automation – Part 90-3: Precision time protocol profile for power utility

[10]                     3GPP TR 22.804: "Study on Communication for Automation in Vertical domains (CAV)".

[11]	Sendin, A., et. al., “Telecommunication Networks for the Smart Grid,” Artech House, 2016.

[12]	Goel, S., S. F. Bush, and D. Bakken, (eds.), IEEE Vision for Smart Grid Communications: 2030 and Beyond, New York: IEEE, 2013.

[13]	US Department of Energy, “Communications Requirements of Smart Grid Technologies, 2010”, accessed 14.08.20, http://energy.gov/sites/prod/files/gcprod/documents/Smart_Grid_Communications_Requirements_Report_10-05-2010.pdf

[14]	IT Process Wiki – The ITIL Wiki:. Content is available according to Creative Commons Attribution-NonCommercial-ShareAlike 3.0 Germany License. Access 14.10.20.

[15]	Incident Management: https://wiki.en.it-processmaps.com/index.php/Incident_Management Content is available according to Creative Commons Attribution-NonCommercial-ShareAlike 3.0 Germany License. Access 14.10.20.

[16]	Change Management: https://wiki.en.it-processmaps.com/index.php/Change_Management Content is available according to Creative Commons Attribution-NonCommercial-ShareAlike 3.0 Germany License. Access 14.10.20.

[17]	Service Asset and Configuration Management: https://wiki.en.it-processmaps.com/index.php/Service_Asset_and_Configuration_Management Content is available according to Creative Commons Attribution-NonCommercial-ShareAlike 3.0 Germany License. Access 14.10.20.

[18]	China Innovation Alliance for Power Distribution Internet of Things, “Technical Specifications of Smart Distribution Transformer Terminal”

[19]	Open PLC European Research Alliance (OPERA). https://cordis.europa.eu/project/id/026920

[20]	“Applications of ITU-T G.9960, ITU-T G.9961 transceivers for Smart Grid applications: Advanced metering infrastructure, energy management in the home and electric vehicles”, 06/2020. https://www.itu.int/dms_pub/itu-t/opb/tut/T-TUT-HOME-2010-PDF-E.pdf

[21]	M. Amin and A. Giacomoni, "Toward more secure, stronger and smarter electric power grids," IEEE PES’11, 2011.

[22]	Eric D. Knapp, Raj Samani, "Smart Grid Network Architecture", "Applied Cyber Security and the Smart Grid", 2013

[23]	5G DNA White Paper: "5GDN@Smart Grid White Paper: Requirements, Technologies, and Practices" https://www.5gdna.org/

[24]	IEEE Std 1344™-1995 (R2001), IEEE Standard for Synchrophasors for Power Systems

[25]                  	IEEE Std C37.118™-2011, IEEE Standard for Synchrophasors for Power Systems

[26]	IEEE Std 1588-2008, Precision Clock Synchronization Protocol for Networked Measurement and Control Systems

[27]	IEEE Std C37.238-2011, IEEE Standard Profile for Use of IEEE Std 1588™ Precision Time Protocol in Power System Applications.

[28]	IEEE Std C37.224-2013

  - [29]	IEC 61850-90-5:2012, use of IEC 61850 to transmit Synchrophasors information according to IEEE C37.118

[30]	IEEE Std C37.118.1a-2014: amendment 1 with modification of selected performance requirements.

[31]	IEEE Std C37.118.2-2011

[32] 	61850-90-3-2016 – IEC/IEEE International Standard - Communication Networks and Systems for Power Utility automation – Part 90-3: Precision time protocol profile for power utility

[33]	Budka, K, et. al., “Communication Networks for Smart Grids”, Springer Verlag, 2014.

[x1]	SOGNO project, Deliverable 4.6, "Description of the integration and testing of the solution including the 5G based advanced communication", https://www.sogno-energy.eu/global/images/cms/Deliverables/774613_deliverable_D4.6.pdf, June 2020.

End First Change

Begin Second Change

### 5.x	Use case of power distribution grid state estimation service

#### 5.x.1	Description

The State Estimation (SE) service [x1] supports the monitoring of the power grid. The goal of SE is to create insight into the operational state of the power network at any given instant in time, by processing the measurement information collected by the instrumentation deployed in the field. The SE service provides monitoring data which can be used by grid operators to assess the performance of their network and to detect possible anomalies in the grid operation. In addition, it can serve as an input for more complex management functions implemented by the Distribution System Operators (DSOs) to more reliably and efficiently operate. An example of a service which could use the SE monitoring data is the power control service [x1]. Other services which could use the SE monitoring data include, but are not limited to, network topology reconfiguration, Volt-VAR control and, in the future, demand side management and demand response services.

#### 5.x.2	Pre-conditions

The measurement devices are deployed in the field and capable to send the data to the DSO control centre (or whatever data concentrator).

#### 5.x.3	Service Flows

The SE service relies as input on the information of the electric grid topology, the electric line parameters and the real-time measurements provided by the instrumentation deployed on the field. Given this set of inputs, the SE service processes the received data to provide the most likely operating state of the network at the considered instant of time (the same time at which the measurements are collected). Finally, the results of the SE service are sent to the control centre (or whatever data concentrator) to give situational awareness to the DSO and, if needed, are made available for other control functions requiring this information. SE results are usually also stored in a database for possible a posteriori analysis.

For its operation, the SE service thus needs as input some important information of the grid characteristics, the deployed measurement infrastructure, as well as the real-time measurement data collected from the measurement units in the field. In addition, if the electric grid is reconfigurable (i.e. topology changes can be applied), the status of the switches and breakers should be also provided in input to the SE tool to make sure that the service is always considering the correct topology of the grid.

Beyond the grid data, which can be determined by the DSOs with a different degree of accuracy as the information of low voltage grids may be incomplete and inaccurate, the key point for enabling the SE service is the availability of measurement devices with real-time communication capabilities. Different from the transmission system, where a large coverage of devices is often available, the instrumentation penetration at the distribution level is usually poor with only few devices installed in specific points, like primary substations (high/medium voltage substations). As a matter of fact, the main power system requirement to enable the grid monitoring via SE is thus the deployment of devices providing the needed measurement data. Due to the large size of the distribution grids and obvious economic constraints, guaranteeing a full observability of the grid is mostly impractical and hardly achievable. The SE solution duly takes into account this aspect and aims at enabling the SE service using only few low-cost measurement devices deployed at strategic points of the grid (chosen by means of ad hoc meter placement strategies). Moreover, the conceived SE solutions are designed to maximize the accuracy performance even in presence of few measurement units in the field. However, it is worth noting that the accuracy of the estimation results strongly depends on the number of available measurements.

#### 5.x.3.1	Power system requirements of SE service

Needed components in the field: sensors and meters.

Information about the electrical grid: topology of the network; grid parameters (line impedance, transformer data, etc.); nominal power of connected loads and generators; location of the meters.

Type of measurements: voltage (both magnitude and phase angle), current (both magnitude and phase angle), active and reactive power are the measurements that can be used for SE purposes (it is not mandatory to have all the types of measurements reported).

Measurements number: the higher the number, the better the results.

Measurement accuracy: the higher the better; information on the uncertainty characteristics of both sensors and measurement units has to be known.

Measurement synchronization: highly recommended.

Additional data needed from the field: notification of switching events leading to a change in the network topology.

Real-time communication capability: yes, required.

Bi-directional communication: not required; the communication flow is mono-directional from the meters to the control centre.

Use of edge cloud. The concept of SE is based on a distributed architecture where both low voltage and medium voltage grids are divided in multiple, smaller areas. This is a solution to reduce the computational cost and execution time of the SE service, as the SE analyses for all areas may operate in parallel. One benefit of this solution is guaranteed scalability of the proposed architecture for SE. Given this framework, it is highly feasible that each local SE runs on a dedicated edge cloud.

#### 5.x.4	Post-conditions

The operating state of the network provided to DSO at a given instant of time.

#### 5.x.5	Existing features partly or fully covering the use case functionality

Table X shows communications requirements for the State Estimation service.

Table X Communications requirements of SE service

| Connection density | Latency | Message size | Sampling rate | Data rate (per measurement device) | Communications service reliability: mean time between failure | Communication service availability: target value | Security |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Up to few 10’s per km2 (in urban areas) | 100’s ms to few seconds | <1 KByte | <50 messages per second | <100 Kbps | ~ 1 day (by small number of measurements) | 99.99% (communications downtime of 4 minutes per month) | Critical |

#### 5.x.6	Potential New Requirements needed to support the use case

None

End Second Change