3GPP TSG-SA WG1 Meeting #93e	  S1-210185

Online, 22 February – 04 March 2021

Title:		Edge cloud driven data acquisition (edgePMU)

Agenda Item:	7.9.1

Source:	Ericsson

Contact: 	Robert Farac (robert.farac@ericsson.com)

Abstract:

Acknowledgment

This contribution proposes a new use case of edge cloud driven data acquisition (edgePMU). This contribution has been prepared in collaboration with RWTH University of Aachen Prof. Monti, Antonello. The use case is being implemented and will be tested in a lab trial and field trials in edgeFLEX project [https://www.edgeflex-h2020.eu/].

Discussion

Proposal

It is proposed to agree to add the following change to TR 22.867.

Begin First Change

2		References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	                       3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	SunWeiqing etc.  Generalized Energy Storage Control Strategies on User Side in Power Ancillary Service Market.  Automation of Electric Power System Vol.44 No.2 Jan.25, 2020

[3]	China Southern Power Grid, China Mobile, HuaWei: "White Paper: 5G enables smart grid applications".http://www.sgcio.com/technology/anli/2018/0716/86656.html

[4]  	GB/T 1936-2018 Guidelines of cyber security protection for electric power system supervision and control http://www.howtoabc.com/book8/14078-0.html, https://www.secrss.com/articles/24761

[5]	IEEE C37.2-2008 IEEE Standard Electrical Power System Device Function Numbers, Acronyms, and Contact Designations.

[6]	IEC TR 61850-90-1:2010, Communication Networks and Systems for Power Utility automation – Part 90-1: Use of IEC61850 for the communication between substations.

[7]	IEEE 1588-2019 – IEEE Standard for a precision clock synchronization protocol for networked measurement and control systems.

[8]	IEEE Guide for Application of Digital Line Current Differential Relays Using Digital Communication.

[9]	61850-9-3-2016 – IEC/IEEE International Standard - Communication Networks and Systems for Power Utility automation – Part 90-3: Precision time protocol profile for power utility

[10]                     3GPP TR 22.804: "Study on Communication for Automation in Vertical domains (CAV)".

[11]	Sendin, A., et. al., “Telecommunication Networks for the Smart Grid,” Artech House, 2016.

[12]	Goel, S., S. F. Bush, and D. Bakken, (eds.), IEEE Vision for Smart Grid Communications: 2030 and Beyond, New York: IEEE, 2013.

[13]	US Department of Energy, “Communications Requirements of Smart Grid Technologies, 2010”, accessed 14.08.20, http://energy.gov/sites/prod/files/gcprod/documents/Smart_Grid_Communications_Requirements_Report_10-05-2010.pdf

[14]	IT Process Wiki – The ITIL Wiki:. Content is available according to Creative Commons Attribution-NonCommercial-ShareAlike 3.0 Germany License. Access 14.10.20.

[15]	Incident Management: https://wiki.en.it-processmaps.com/index.php/Incident_Management Content is available according to Creative Commons Attribution-NonCommercial-ShareAlike 3.0 Germany License. Access 14.10.20.

[16]	Change Management: https://wiki.en.it-processmaps.com/index.php/Change_Management Content is available according to Creative Commons Attribution-NonCommercial-ShareAlike 3.0 Germany License. Access 14.10.20.

[17]	Service Asset and Configuration Management: https://wiki.en.it-processmaps.com/index.php/Service_Asset_and_Configuration_Management Content is available according to Creative Commons Attribution-NonCommercial-ShareAlike 3.0 Germany License. Access 14.10.20.

[18]	China Innovation Alliance for Power Distribution Internet of Things, “Technical Specifications of Smart Distribution Transformer Terminal”

[19]	Open PLC European Research Alliance (OPERA). https://cordis.europa.eu/project/id/026920

[20]	“Applications of ITU-T G.9960, ITU-T G.9961 transceivers for Smart Grid applications: Advanced metering infrastructure, energy management in the home and electric vehicles”, 06/2020. https://www.itu.int/dms_pub/itu-t/opb/tut/T-TUT-HOME-2010-PDF-E.pdf

[21]	M. Amin and A. Giacomoni, "Toward more secure, stronger and smarter electric power grids," IEEE PES’11, 2011.

[22]	Eric D. Knapp, Raj Samani, "Smart Grid Network Architecture", "Applied Cyber Security and the Smart Grid", 2013

[23]	5G DNA White Paper: "5GDN@Smart Grid White Paper: Requirements, Technologies, and Practices" https://www.5gdna.org/

[24]	IEEE Std 1344™-1995 (R2001), IEEE Standard for Synchrophasors for Power Systems

[25]                  	IEEE Std C37.118™-2011, IEEE Standard for Synchrophasors for Power Systems

[26]	IEEE Std 1588-2008, Precision Clock Synchronization Protocol for Networked Measurement and Control Systems

[27]	IEEE Std C37.238-2011, IEEE Standard Profile for Use of IEEE Std 1588™ Precision Time Protocol in Power System Applications.

[28]	IEEE Std C37.224-2013

  - [29]	IEC 61850-90-5:2012, use of IEC 61850 to transmit Synchrophasors information according to IEEE C37.118

[30]	IEEE Std C37.118.1a-2014: amendment 1 with modification of selected performance requirements.

[31]	IEEE Std C37.118.2-2011

[32] 	61850-90-3-2016 – IEC/IEEE International Standard - Communication Networks and Systems for Power Utility automation – Part 90-3: Precision time protocol profile for power utility

[33]	Budka, K, et. al., “Communication Networks for Smart Grids”, Springer Verlag, 2014.

[x1]	SOGNO project, Deliverable 2.2, "Description of initial interfaces & services for grid awareness", https://www.sogno-energy.eu/global/images/cms/Deliverables/774613_SOGNO_D2.2.pdf, December 2018.

[x2]	edgeFLEX project, Deliverable 1.2, “Dynamic-phasor driven voltage control concept for current VPPs in large scale deployment deliverable”, [to be published on “https://www.edgeflex-h2020.eu/progress/work-packages.html” on 31.03.2021].

[x3]	edgeFLEX project, Deliverable 4.1, “Description of edgeFLEX platform design”, [to be published on “https://www.edgeflex-h2020.eu/progress/work-packages.html” on 31.03.2021].

End First Change

Begin Second Change

### 5.x		Edge cloud driven data acquisition (edgePMU)

#### 5.x.1	Description

In the power grid, there are devices deployed to measure physical quantities from the grid, such quantities are e.g. electrical quantities such as amplitudes of voltage and current, phase, frequency or rate of change of frequency (ROCOF). One device that can provide such measurements is called a Phasor Measurement Unit (PMU). This particular device provides the above-mentioned values synchronized to a global time clock derived, for example from GPS. Such synchronized values can be used as an input to power distribution grid services, such as state estimation service [x1]. Another possible service that relies on measurements of voltage, current and phase is a voltage control service [x2].

As PMUs are very expensive devices, they are placed only at key points of interest. However, it is possible to use cheaper field devices that collect sampled measurement data and then process them in the edge cloud hosted PMU software, virtualizing the processing functionality of a traditional PMU, thus enabling the concept of the edgePMU [x3]. Due to the lower processing requirements of the field devices, their cost is reduced, and they can also be deployed in the distribution grid providing power system operators with a more precise knowledge of the state of their grids. The edgePMU concept separates the data acquisition from the data processing by exploiting the computational capabilities offered by distributed 5G edge clouds.

The edgePMU, with its modular approach, tackles the growing need for low-cost measurement devices in distribution networks from a new perspective. It utilizes the scalability of cloud infrastructure and decreases the specialization needed in the data acquisition device, by providing flexible cloud solutions for different data use cases. Furthermore, the overall deployment is simplified from a communication point of view since there is no need for special communications cabling and a network infrastructure managed by distribution grid operators. This approach simplifies the adoption of PMUs in distribution networks by helping distribution grid operators gradually deploy their measurement architectures. In fact, the cloud-based approach offers great flexibility in the development of monitoring and automation functionalities, by gradually stacking services and processing modules. It also provides scalability opportunities, thanks to the cloud technology-based computational infrastructure. The increased computational power offered by 5G edge cloud compared to that of low-cost single board computers enables more computationally challenging algorithms to be deployed.

Here are the key features of the edgePMU concept [x3]:

- High rate raw data transmission between the data acquisition unit and the edge cloud using 5G wireless connectivity

- The ability to deploy services or other applications on edge cloud infrastructure instead of having to deploy them in a traditional PMU with its high processing power

- A single field device can be used as a data acquisition unit for different services hosted in the edge cloud

- Services can be deployed, modified and upgraded on demand without the need to physically visit the sites

- The cloud-based software is not limited to only providing phasor measurements, but could also be used to calculate other metrics for power distribution grid services

#### 5.x.2	Pre-conditions

Coverage by 5G communications networks of regions in which measurement devices and edge cloud are to be located is required. The voltage or current measurement sensors and their associated secure data acquisition units have to be deployed in the power grid.

The secure data acquisition unit is connected wirelessly by the 5G network to the edge cloud hosted services.

The service of phasor calculation is deployed in the edge cloud infrastructure.

#### 5.x.3	Service Flows

Figure 0-1: Service flow diagram for edgePMU use case

- The data acquisition unit is collecting data, such as voltage and current values, from the power grid.

- This data is then sent via secure 5G communication to the relevant service hosted on the edge cloud.

- After the data has been collected in the edge cloud, the service processes the data. A small buffer is needed for post-processing with the service hosted on the edge cloud.

- In the case of the phasor calculation service, the estimated phasors can be forwarded to further applications or actors, either hosted by the edge cloud or external to it.

#### 5.x.4	Post-conditions

The relevant power distribution grid service is successfully deployed in the edge cloud and running smoothly.

The voltage or current measurements are collected by the data acquisition units and transmitted to the service hosted on the edge cloud.

The service deployed in the edge cloud provides upstream services, such as state estimation or voltage control, with the input data they need.

The resulting service output data, hosted on the edge cloud, is stored permanently or forwarded to upstream services.

#### 5.x.5	Existing features partly or fully covering the use case functionality

Existing LTE wireless connectivity, if configured to provide the required latency, could support the transmission of field device measurement data to services hosted on clouds hosted on distribution system operator owned servers.

Latency: Lower than 2/Fs, where Fs is the output reporting rate of the edge cloud service. Example: 60 phasors per second at 60 Hz means Fs = 60 and 2/60=0.033 means the overall latency budget is less than 33 ms. Typical networks aim for 50 or 60 phasors, depending on the geo location. Higher reporting rates are also allowed by the standard.

| Latency | Communication service reliability: mean time between failure | Security |
| --- | --- | --- |
| Typically of the order of magnitude of 33 - 40 ms | > 1 minute | High |

#### 5.x.6	Potential New Requirements needed to support the use case

Message sampling rate

From at least 1,000 messages per second