3GPP TSG-SA WG1 Meeting #93e 	S1-210227

E-Meeting    22 Feb - 04 Mar 2021	(revision of S1-21xxxx)

Title:	Updates of Use case 5.4 for on demand PALS services

Agenda Item:	7.13.1

Source:	Intel

Contact:	Ellen Liao, ellen.c.liao at Intel.com

Abstract: this PCR provides updates to use case 5.4 related to on demand PALS services.

Proposal:

This PCR proposed to update the following aspects:

- Resolve editor notes related to PALS service and PALS network as well as visiting/roaming terms based on related contributions in this meeting

- Clarify the service flows to address the scenario for new service requirements

  - for user authentication and authorization

  - for UE configured with PALS services

  - for enabling support for specific 5G services in PALS network

  - for using home operator’s service hosting environment

*** 1st Change ***

## 5.4	Use Case for UEs using on demand services via hosting network

### 5.4.1	Description

This use case assumes that network operators can build short term relationship using application layer approach, e.g. using smart contracts based blockchain technologies, for offering on demand services provided by different service providers via a hosting network.

The application layer approaches require 5G network to expose network capabilities for the PALS service. For example, for the automatic e-agreement mechanism deployed by SP-A for a PALS service at a specific occasion (time and location), which allows PALS services to be shared among service providers. In addition, the PALS e-agreement can also allow the hosting network to accommodate on-demand services provided by different service providers.

Based on the PALS e-agreement, the hosting network can be configured with PALS service at a specific time and location for its PALS service subscribers (other network operator), e.g. PALS service policies of time, location, network-A access parameters, including spectrum, access technologies (3GPP or non-3GPP), network slice, charging policies, and subscriber’s network policies for authentication, and routing. In the meantime, the SP-A and SP-B can further offer on-demand services via hosting network.

The 5G network provides suitable mechanisms to allow establishing automatic roaming (e)-agreements for a specific occasion (time and location) and building temporary relationship among PALS network operator (NPN or PLMN) and other service operators including network operators or third-party content service providers. The on-demand service configuration can be provisioned in the hosting network based on the e-agreement. For example, SP-C can subscribe PALS service and SP-B’s on demand service based on e-agreement for their UEs and configures their UEs with PALS service. When a UE configured with PALS service connects to hosting network, the SP-A can present available on-demand services provided by different service providers for UEs configured with PALS services to select any one of them. When using SP-B’s on demand service, the SP-C’s UEs can be charged by their home network.

Editor’s note: the application approach for automatic e-agreement is an example that provides some insights for what the suitable APIs would be required for PALS service enabler in 5GS.

### 5.4.2	Pre-conditions

At the Monster theme park on an island hosting three days adventures for visitors, the park has SP-A network coverage for a hosting network-A providing access to localized service (PALS) which has been subscribed by SP-B and SP-C for their UEs based on  e-agreement established by SP-A for sharing this PALS service. In the meantime, SP-B’s network is not available on the island and SP-C’s network is very spotty and only available at some zones.

For the on-demand dedicated services, SP-A deploys on demand services on its owned Service Hosting Environment-A for streaming live video and immersive media in different monsters’ zones. SP-B also provides on demand services on its Service Hosting Environment-B connected to the hosting network for superhero interactive game via hosting network-A. SP-D is a content service provider which offers on demand services on its application platform-D for streaming movies. The SP-C subscribes SP-A’s and SP-B’s on demand services for their UEs attending adventures on the island. In addition, the SP-C rents the resource from the SP-A for providing on demand services for interactive Monster hunting games on its owned Service Hosting Environment-C connected to the hosting network.

When the scheduled event is started, the network-A starts to provision PALS services as well as the on-demand services provided by different service providers.

Lexi and Timmy visit their grandfather and attend this annual event. Their UEs, UE-C1 and UE-C2, are both with SP-C’s subscriptions. The SP-C subscribed SP-A’s PALS services configures their UEs with information of PALS network identity for selecting PALS network and credentials for accessing PALS services via PALS newtork.

### 5.4.3	Service Flows

Use Case A:  use of home network service and on demand services when both networks are available

Step1: When visiting the genetic engineering center where both SP-A and SP-C networks are available, Lexi’s UE-C1 and Timmy’s UE-C2 configured with PALS service can select and connect to hosting network-A and its SP-C’s network simultaneously. The hosting network-A authenticates the UE-C1 and UE-C2 and both UEs use their configured user credentials for accessing to PALS services via PALS network.

Step2: UE-C1 and UE-C2 present available on demand dedicated services to Lexi and Jimmy.

Step3: Jimmy selects the superhero interactive game provided by SP-B and chooses to be the Spiderman playing with holograms of live monsters at the park (which enables specific network-A 5G services). In the meantime, Lexi selects one immersive media provided by SP-A to watch the live monsters running around the park. The hosting network-A gets PALS service authorization from service providers and creates the service session for the authorized UEs.

Step4: Both Lexi and Jimmy can share live videos with their friends on social media using home network services via SP-C network.

Use Case B:  use of local SP-A’s on demand services and SP-D’s on demand services via hosting network

Step1: after one day’s fun adventure at the park, Lexi lying on the bed at the cabin where her UE-C1 finds only SP-A is available and connects to the hosting network.

Step2: Lexi scrolls the on demand services menu and determines to select an on demand movie, Jurassic Park (1993), provided by SP-D.

Step2: in the meantime, she selects another on demand services provided by SP-A to monitor live monstors’ night life in Zone X.

Step3: The hosting network-A establishes required IP connections for two on demand services, and routes the UE-C1’s traffic locally at SP-A’s hosting network and to SP-B’s network, respectively.

### 5.4.4	Post-conditions

Use Case A: Lexi and Timmy enjoy the adventure and good network services using their UE-C1 and UE-C2 for SP-A and SP-B on demand service via hosting network, respectively. In the meantime, their UEs can connect SP-C networks and use home network services. Since SP-C subscribes to SP-A’s and SP-B’s on demand service, the on-demand services charge will be in the next month statement from their SP-C’s home network operator.

Use Case B: Lexi needs to pay for the on-demand service provided by SP-D using online payment method before enjoying the movie. Lexi is happy to continue fun experience at night using her UE-C1 for on demand services provided by SP-A and SP-D via network-A, respectively.

### 5.4.5	Existing features partly or fully covering the use case functionality

The existing service requirements for roaming users assume that roaming policies are in place for UEs to use their home network services or on demand services provided by other service providers.

### 5.4.6	Potential New Requirements needed to support the use case

[PR.5.4.6-001]: The 5G network shall provide suitable APIs to allow a trusted third party application  (other service providers) to provision their on-demand services policies (e.g. network slice, required IP connectivity, QoS) and routing policies for the application of the on-demand services via the PALS network.

[PR.5.4.6-002]: The 5G network shall be able to allow a roaming UE configured with PALS service to connect and manually select on-demand services, which are provided by the PALS network operator, home network operator, or other service providers and routed via the PALS network.

[PR.5.4.6-003]: The 5G network shall be able to provide required IP connection, e.g. network slice, QoS, for  a roaming UE configured with PALS service to simultaneously use its home network services, and on-demand services.

[PR.5.4.6-004]:  A roaming UE configured with PALS service shall be able to select and connect to the PALS network for simultaneously using home network service and on-demand services when only PALS network is available.

[PR.5.4.6-005]: The UE configured with PALS service shall be able to simultaneously connect to its home network for home network service and connect to PALS network for on-demand services when both home network and PALS network are available.

[PR.5.4.6-006]: The 5G network shall be able to collect charging records for PALS service and on-demand services and provide the charging records to UEs’ home operators based on service and charging policies provided by the on-demand service providers.

[PR.5.4.6-007]: Based on operator policies and service agreements, the PALS network shall enable specific 5G services support required by the PALS services for the authorized UEs.

[PR.5.4.6-008]: The 5G network shall be able to provide user credentials for roaming UEs and authenticate/authorize the users using the user credentials when the roaming UEs request for accessing PALS services via PALS network.

[PR.5.4.6-009]: A UE configured with PALS services shall be able to select and connect to PALS network based on configured PALS network identity and UE’s credential; The authenticated UE shall be able to access selected PALS services via PALS network based on configured user credentials.

*** End of Change ***