3GPP TSG-SA WG1 Meeting #93e 	S1-210236

E-Meeting    22 Feb - 04 Mar 2021	(revision of S1-21xxxx)

Title:	Enable support for user centric identifiers and authentication in Residential

Agenda Item:	7.11.1

Source:	Intel

Contact:	Ellen Liao, ellen.c.liao at Intel.com

Abstract: this PCR proposed to enable support of user centric identifiers and authentication for (off-the-shelf) PRAS in Residential environment.

### Proposal:

This PCR proposed to update use case and add service requirements to enable support of the user centric identifiers and authentication in the Residential/PIN studies.

This PCR proposes clarification to address editor’s note of SR002.

In the TS22.101 clause 26a, the 5G network operator can act as an identity provider for users to be identified could be:

- an individual human user, using a UE with a certain subscription, or

- an application running on or connecting via a UE, or

- a device (“thing”) behind a gateway UE.

These users are associated to the 3GPP devices (UE or gateway UE) which are subscribers of the 5G network. The 5G network can trust and identify a user based on the authenticated 3GPP device that is associated to the user.

The following service requirement in TS22.101 clause 26a provide the principle for user centric identifiers and authentication by the 3GPP system:

The 3GPP System shall support operators to act as User Identity provider and to authenticate users for accessing operator and non-operator deployed (i.e. external non-3GPP) services

As such, in the context residential environment, this PCR proposed that the 5G network can trust and identify a user based on the authenticated “3GPP device” that is associated to the user for supporting UEs to access operator deployed services. For an operator’s eRG (a 3GPP device), it is proposed that the (off-the-shelf) PRAS can be a user for being managed by the 5G network operator. The trust between (off-the-shelf) PRAS and eRG can be established when the (off-the-shelf) PRAS and eRG are both authenticated by the 5G network operator, as shown in Figure X.

This PCR proposed to enable support of the user centric identifiers and authentication in Residential network.

Figure X: user centric identifiers and authentication in Residential environment or PIN

*** 1st Change ***

### 5.7 	Identification and Provisioning evolved residential gateways and Premises Radio Access Stations

### 5.7.1	Description

The use case illustrates how an evolved residential gateway or a Premises Radio Access Station can be connected and automatically provisioned to the operator's 5G network.

### 5.7.2	Pre-conditions

Ali has a home connectivity service subscription and a 5G subscription on his mobile phone with Network Operator Vanilla.

Case A:

Ali purchased an evolved residential gateway and one (or more) Premises Radio Access Stations off-the-shelf, i.e. these devices were not provided by the operator.

NOTE:	it is assumed the evolved residential gateway is a fixed wireless access (FWA) gateway

Case B:

Ali purchased two Premises Radio Access Stations off-the-shelf, i.e. these devices were not provided by the operator and connected them to operator’s evolved residential gateway (one is placed in the first floor and another one is placed in the second floor).

Ali is at home and has his mobile phone nearby.

### 5.7.3	Service Flows

1.	Ali switches both the evolved residential gateway and Premises Radio Access Station on for the first time.

Case A: (off-the-shelf) Premises Radio Access Stations connects to (off-the-shelf) eRG

2a.	Ali logs on to his account on Vanilla's portal to add both devices to his Vanilla account and to enable the devices in his home.

3a.	When both devices are turned on and connected to Vanilla’s 5G core network, Vanilla's 5G core network provisions both the evolved residential gateway and Premises Radio Access Station. Once this process is complete, both the evolved residential gateway and Premises Radio Access Station are successfully connected to Vanilla's core network and are now fully operational.

4a.	Ali can now connect his TV to the Premises Radio Access Station in order to stream a move.

Case B: (off-the-shelf) Premises Radio Access Stations connects to operator’s eRG

2b.	Ali logs on to his account on Vanilla's portal to add both Premises Radio Access Stations to his Vanilla account to enable the devices in his home.

3b. Vanilla's 5G core network creates user identities for both devices and stores user profiles of evolved residential gateway and Premises Radio Access Station.

3c.	When both devices are turned on and connected to Vanilla’s 5G core network, Vanilla's 5G core network authenticates the user identities of Premises Radio Access Stations and then provisions Premises Radio Access Stations. Once this process is complete, Premises Radio Access Station are successfully connected to Vanilla's core network and are now fully operational.

4.	Ali can now connect his UE to the Premises Radio Access Station and speak to the phone when walking around the house with good 5G service coverage.

### 5.7.4	Post-conditions

Case A:

The evolved residential gateway and Premises Radio Access Station have been provisioned automatically with minimum user interaction.

Case B:

Both Premises Radio Access Stations have been authenticated and provisioned automatically with minimum user interaction.

### 5.7.5	Existing features partly or fully covering the use case functionality

There are existing features that support remote provisioning, such as via support of (un-)trusted non-3GPP access 3GPP TS 23.501 [4].

Remote provisioning of H(e)NBs is defined in 3GPP TS 22.220 [3], which includes verification and configuration of H(e)NB identity, initial OA&M provisioning.

If an evolved residential gateway is considered as a UE, remote provisioning is already defined.

### 5.7.6	Potential New Requirements needed to support the use case

[PCR 5.7.6-001] The 5G system shall provide mechanisms for the network operator or a trusted third party (e.g. a homeowner) to perform remote provisioning of evolved residential gateways, which includes verification and configuration of evolved residential gateway identity and initial OA&M provisioning.

NOTE 1:	it is assumed the evolved residential gateway is a fixed wireless access (FWA) gateway and is not a UE

[PCR 5.7.6-002] The 5G system shall provide mechanisms for the network operator or a trusted third party (e.g. a homeowner) to perform remote provisioning of Premises Radio Access Stations, which includes verification and configuration of Premises Radio Access Station identity and initial OA&M provisioning.

NOTE 2:	it is assumed the Premises Radio Access Station is not a UE

Editor's Note: whether a distinction for 'off-the-shelf' eRG / PRAS is need is FFS. If needed, then a suitable term for 'off-the-shelf' should be defined

[PCR 5.7.6-003] The 5G network operator shall be able to act as an identity provider for users that are (off-the-self) Premises Radio Access Stations connected to operator’s eRG.

[PCR 5.7.6-004] The 5G system shall provide mechanisms for the network operator to authenticate the user identity of the (off-the-self) Premises Radio Access Station which is connected to the operator’s eRG.

[PCR 5.7.6-005] The 5G system shall support service continuity for a UE accessing and moving from one Premises Radio Access Station to another Premises Radio Access Station which are both connected to the operator’s eRG.

*** End of Change ***