3GPP TSG-SA WG1 Meeting #93e	S1-210367

Electronic Meeting, 22 February - 4 March 2021 	(revision of S1-210037)

# Source:	China Unicom, Spreadtrum Communications, Samsung, KPN

# Title:	New WID on Data Integrity in 5GS

# Document for:	Approval

Agenda Item:	4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Data Integrity in 5GS

Acronym: DI_5G

Unique identifier:

Potential target Release: Rel-18

Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan.

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  |  |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

3	Justification

Verticals, such as supply chain finance and insurance, have strict requirements for IoT data integrity. This is especially true for many services that collect data from IoT edge devices. Examples include environment data such as temperature and windspeed data which can be important to fulfill insurance policy stipulations, for agriculture and other industries. Warehouse data (e.g. number and weight of goods) is essential for inventory financing. Those services require data integrity from UE to a third-party service provider.

In such contexts, integrity protection service for the data collected by IoT UEs for certain vertical services is indispensible. However in practice, all kinds of IoT UEs serving enterprises connect to MNOs’ networks in practice. Enterprises and derivative businesses would benefit from the data exchanged with integrity protection. This includes the communication of any of the many kinds of IoT UEs that enterprises deploy, including devices are diverse and hard to configure in a uniform manner, and some of these devices do not support end to end data integrity capabilities themselves.

Therefore, an additional ability to provide data integrity protection service between a UE and an Application Server offered by a third-party service provider for IoT services (especially for the services that collect data from IoT edge devices) would be very valuable to support for 3GPP system. MNOs should be able to offer a standard approach to service exchange with data integrity to their customers.

4	Objective

The objective of this work is to specify requirements on Data Integrity in 5GS for IoT UEs, specifically:

-	Data integrity protection service for data exchange between 5GS and an Application Server, to protect communication between a UE and an Application Server, offered by a third-party service provider.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 22.261 | Addition of requirements on Data Integrity in 5GS | TSG#91e（March, 18th） |  |

6	Work item Rapporteur(s)

Qun wei, China Unicom.

Weiqun5 <at> chinaunicom.cn

7	Work item leadership

SA1

8	Aspects that involve other WGs

None

9	Supporting Individual Members

Supporting IM name

China Unicom

Spreadtrum Communications

China Mobile

China Telecom

CATT

Lenovo

Motorola Mobility

Convidawireless

ZTE

Caltta

LG Electronics

Tencent

KPN

Samsung

Telefonica

AsiaInfo