3GPP TSG-SA WG1#93 e-Meeting	S1-210444

22 February – 4 March 2021	(revision of S1-210215)

Title:	Comments on “deterministic communication” in TR 22.867 on 5G Smart Energy and Infrastructure

Agenda Item:	7.9.1

Source:	Qualcomm

Contact:	Jack Nasielski (jackn at qti.qualcomm.com)

Abstract: This document provides changes to TR22.867 v0.2.0.

Introduction

Current text or TR22.867 uses the term “deterministic communication” in several places where it seems uneccessary and innaproppriate given the uses cases and required KPIs.  Generally these use cases require delays of 10-1000 ms and various data rates for monitoring and video streaming. Furthermore, as emerged in recent discussions, the term “deterministic” maybe quite confusing from a stage-2/3 point of view. It is thus proposed to remove the term in the TR.

-------------------------- First Change -------------------------

### 5.1.6	Potential requirements

[PR. 001] The 5G system shall be able to provide required communication service for distributed energy storage according to the KPIs given in table 5.1.6-1 and table 5.1.6-2 .

Table 5.1.6-1 periodic communication service performance requirements --- data for distributed energy storage

| scenario | use case | transfer interval target value (ms) | message size (byte) | data rate per storage location (bps) (note1) | communication latency (ms) | reliability | storage node density # /km2 (note2) | active factor/ km2 (note3) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dense Urban | Virtual energy storage monitoring | UL: 10 | UL:>20k | UL: >16M DL: >100k | DL:10 UL:10 | DL: >99.90% | >[x]*1k | 10% |
|  | Virtual energy storage : other data collection | UL: 1000 | UL: 16k DL: >100k | UL: >128k DL: >100k | DL:<10 UL:<1000 | DL: >99.90% | >[x]*1k | 10% |
| Rural | Power storage station monitoring | UL: 10 | UL: 16k*50 | UL: > 640M DL: > 100k | DL:<10 UL:<10 | DL: >99.90% | >[x]*100 | 10% |
|  | Energy storage station operation data collection: other data | UL: 1000 | UL: 26k*50 DL: >100k | UL: > 10.4M DL: > 100k | DL:<10 UL:<1000 | DL: >99.90% | >[x]*100 | 10% |
| NOTE 1: this KPI is to require data rate in one Energy storage station which may via one or more 5G connections and via one or more 3GPP UE(s) at the same time. NOTE 2: It is used to deduce data volume in an area which has multiple energy storage stations. The data volume can be deduced through follow formula: (Current + other data) data rate per storage station * (Storage node density /km2) * (Active factor/km2) + video data rate per storage station * (Storage node density /km2) NOTE 3: active factor means the proportion that the number of active DED which are delivering its collected data during one second time window compared with whole number of DED in the area which has multiple energy storage station Editor’s Note: the "x" in the table with real value will FFS. |  |  |  |  |  |  |  |  |

Table 5.1.6-2   Aperiodic communication service performance requirements -- - video for distributed energy storage

| scenario | use case | data rate per storage station (bps) (note1) | communication latency (ms) | reliability | storage node density # / km2 |
| --- | --- | --- | --- | --- | --- |
| Rural | Energy storage station: video | UL: >5G DL: >100k | DL:<10 UL:<1000 | DL: >99.90% | >[x]*100 |
| NOTE1: the data rate is to require data rate in one Energy storage station which may via one or more 5G connections and one or more 3GPP UE(s) at the same time. It can be calculated with following formula: 12.5M(bytes)/s * 50(containers) * 8 = 5Gbps Editor Note: the "x" in the table with real value will FFS. |  |  |  |  |  |

-------------------------- End First Change -------------------------