3GPP TSG-SA WG1 Meeting #93e 	S1-210481

E-Meeting    22 Feb - 04 Mar 2021	(revision of S1-210236r6)

Title:	Enable support for (off-the-shelf) PRAS in Residential

Agenda Item:	7.11.1

Source:	Intel

Contact:	Ellen Liao, ellen.c.liao at Intel.com

Abstract: this PCR proposed to enable support for (off-the-shelf) PRAS in Residential environment.

*** 1st Change ***

### 5.X 	Identification, Authentication, and Authorization for (off-the-shelf) Premises Radio Access Stations

### 5.X.1	Description

To ensure providing the secure connectivity for UEs connected to 5G network via (off-the-shelf) PRAS behind eRG, this use case illustrates the need to have 5G system support for identification, authentication, and authorization of (off-the-shelf) Premises Radio Access Stations.

The use case is to ensure that there is 3GPP mechanism to protect the weak-link between (off-the-shelf) PRAS (assuming untrusted) and the eRG.

### 5.7.2	Pre-conditions

Alicia has installed an eRG with 5G subscription provided by Network Operator Wallowa.

Alicia purchased two Premises Radio Access Stations (PRAS) off-the-shelf, i.e. these devices were not provided by the operator and thus considered as untrusted devices for the operator’s network.

When returning home, Alicia logs on to her account on Operator Wallowa's portal to add both Premises Radio Access Stations to configure these two devices and associated them to a trusted 3GPP device, eRG.

Alicia connects two PRAS to operator’s eRG (one PRAS is placed in the first floor and connected to eRG via non-3GPP access; another PRAS is placed in the second floor and connected to eRG via wireline).

### 5.7.3	Service Flows

1. Operator Wallowa's 5G network creates identities for both devices and stores profiles of both Premises Radio Access Stations.

2.	Alicia turns on both PRAS. The eRG connects them to 5G network. Based on the request from authenticated eRG and profiles of the PRAS, Operator Wallowa’s 5G network identifies, authenticates   and authorizes the PRAS, and provisions configurations, e.g. PRAS operation authorization, operator’s settings, etc., to PRAS via eRG.

3. 	When each PRAS completes installation, it connects to Operator Wallowa's 5G network via eRG.

4.  The Operator Wallowa's 5G network can identify both PRAS and authenticate the identity of each Premises Radio Access Station and authorize the PRAS operation based on profile of the PRAS.

5. Once this process is complete, Premises Radio Access Stations are successfully authenticated, authorized, and connected to Operator Wallowa's network via eRG and are now fully operational.

### 5.7.4	Post-conditions

The 5G network ensures that the E2E connection from the 5G core network to the UE connected to PRAS behind eRG are secure because both (off-the-shelf) PRAS connected via operator’s eRG are authenticated, authorized, and managed by the operator.

Alicia can now connect her UEs to the Premises Radio Access Stations. She is happy that she can speak to the phone when walking around the house with good 5G service coverage.

### 5.7.5	Existing features partly or fully covering the use case functionality

The following service requirement in TS22.101 clause 26a provide the principle for user centric identifiers and authentication and authorization by the 3GPP system:

The 3GPP System shall support operators to act as User Identity provider and to authenticate users for accessing operator and non-operator deployed (i.e. external non-3GPP) services.

### 5.7.6	Potential New Requirements needed to support the use case

NOTE 1:	it is assumed the evolved residential gateway is a trusted 3GPP device with subscriptions provided by the network operator.

Editor's Note: whether a distinction for 'off-the-shelf' PRAS is needed is FFS. If needed, then a suitable term for 'off-the-shelf' should be defined.

Editor’s Note: it is FFS for addressing the service requirements that allow 5G network to securely provide credential and identification to the PRAS and eRG.

[PCR 5.7.6-001] The 5G system shall provide mechanisms to identify, authenticate, and authorize a (off-the-shelf) Premises Radio Access Stations connected to an eRG.

[PCR 5.7.6-002] The 5G system shall provide mechanisms to provision configuration settings for PRAS operation to authenticated and authorized (off-the-shelf) Premises Radio Access Stations connected to an eRG.

*** End of Change ***