3GPP TSG-SA WG1 Meeting #93e	S1-210018

Online, 22 February - 04 March 2021

Title:	 FS_Ranging Consolidated Requirements

Agenda Item:	7.10.1

Source:	Xiaomi

Contact:	Xiaowei Jiang (jiangxiaowei@xiaomi.com)

Abstract: consolidated requirements section of FS_Ranging TR22.855  and will be updated based on the email discussion results in this meeting.

Discussion

- Functional requirements

| PR number | Description | Action |
| --- | --- | --- |
| [PR 5.1.6-1] | The 5G system shall be able to support Ranging enabled UEs to discover other Ranging enabled UEs. | Create a new requirement: [PR 8.1-1] The 5G system shall be able to support for a UE to discover other UEs supporting Ranging. |
| [PR 5.2.6-1] | The 5G system shall be able to authorize Ranging for each individual UE. | Merged with [PR 5.18.6-1] & [PR 5.19.6-1] Create a new requirement: [PR 8.1-2] The 5G system shall be able to authorize Ranging for a UE or a group of UE when using licensed spectrum. |
| [PR 5.3.6-1] | The 5G system shall be able to protect privacy of a UE and its user, ensuring that no identifiable information can be tracked by undesired entities during Ranging. | Create a new requirement: [PR 8.1-3] The 5G system shall be able to protect privacy of a UE and its user, ensuring that no identifiable information can be tracked by undesired entities during Ranging. |
| [PR 5.5.6-1] | The 5G system shall support identification of other ranging capable UE/UEs with which a ranging capable UE performs Ranging. | This is a duplicate of [PR 8.1-1]. |
| [PR 5.6.6-1] | The 5G system shall be able to enable or disable the Ranging service. | Create a new requirement: [PR 8.1-4] The 5G system shall be able to enable or disable Ranging. |
| [PR 5.7.6-1] | The 5G system shall be able to support for a UE to discover other UEs supporting Ranging. | Included by [PR 8.1-1] |
| [PR 5.8.6-1] | The 5G system shall be able to support for a UE to discover other UEs supporting Ranging. | Included by [PR 8.1-1] |
| [PR 5.9.6-1] | The 5G system shall be able to support for a UE to discover other UEs supporting Ranging. | Included by [PR 8.1-1] |
| [PR 5.11.6-1] | The 5G system shall support mutual ranging, i.e. the two UE’s involved shall be able to range each other. | Create a new requirement: [PR 8.1-5] The 5G system shall support mutual Ranging, i.e. two UEs shall be able to initiate Ranging to each other. |
| [PR 5.2.6-2] | The 5G system shall be able to ensure that the use of ranging, if in licensed spectrum, is only permitted in network coverage under the full control of the operator who provides the coverage. | Create a new requirement: [PR 8.1-6] The 5G system shall be able to ensure that the use of Ranging, if in licensed spectrum, is only permitted in network coverage under the full control of the operator who provides the coverage. |
| [PR 5.12.6-1] | The 5G system shall support energy efficient UE ranging services and operation. | Create a new requirement: [PR 8.1-7] The 5G system shall support energy efficient UE Ranging operation. |
| [PR 5.13.6-1] | The 5G system shall be able to start ranging and stop ranging according to the application layer’s demand. | Create a new requirement: [PR 8.1-8] The 5G system shall be able to start ranging and stop ranging according to the application layer’s demand. |
| [PR 5.13.6-2] | The 5G system shall be able to provide mechanisms for a MNO, or authorized 3rd party, to provision and manage ranging operation and configurations. | Create a new requirement: [PR 8.1-9] The 5G system shall be able to provide mechanisms for a MNO, or authorized 3rd party, to provision and manage ranging operation and configurations. |
| [PR 5.15.6-1] | The 5G system shall be able to support mechanisms for a UE to assist another UE to perform ranging of a third UE (if all UEs are in LOS conditions). NOTE 1: Providing assistance to another UE can e.g. be used to support identifying a cluster of ranging enabled UEs (i.e. a set of UEs within a configured distance from each other or from a UE determined to be the center of the cluster). NOTE 2: It cannot be assumed that all ranging UEs support the same application for exchange of information. | Merged with [PR 5.17.6-1] Create a new requirement: [PR 8.1-10] The 5G system shall be able to support mechanisms for a UE to assist another UE to perform ranging of a third UE (if the requesting UE is LOS with the assisting UE and the assisting UE is LOS with the third UE). NOTE: It cannot be assumed that all ranging UEs support the same application for exchange of information. |
| [PR 5.16.6-1] | The 5G system shall be able to support ranging enabled UEs to determine the ranging capabilities (e.g. capabilities to perform distance and/or angle measurement) of other ranging enabled UEs. | Create a new requirement: [PR 8.1-11] The 5G system shall be able to support ranging enabled UEs to determine the ranging capabilities (e.g. capabilities to perform distance and/or angle measurement) of other ranging enabled UEs. |
| [PR 5.16.6-2] | The 5G system shall be able to allow a ranging enable UE to determine if another ranging enabled UE is stationary or mobile, before and/or during ranging. NOTE: this may require assistance from other ranging enabled UEs. | Create a new requirement: [PR 8.1-12] The 5G system shall be able to allow a ranging enable UE to determine if another ranging enabled UE is stationary or mobile, before and/or during ranging. NOTE: this may require assistance from other ranging enabled UEs. |
| [PR 5.17.6-1] | The 5G system shall be able to support mechanisms for a UE to assist another UE to perform ranging of a third UE (if the requesting UE is LOS with the assisting UE and the assisting UE is LOS with the third UE). | Included by [PR 8.1-10] |
| [PR 5.18.6-1] | The 5G system shall be able to support ranging service authorization for a UE or a group of UE. | Merged with [PR 5.2.6-1] |
| [PR 5.18.6-2] | The 5G system shall be able to support ranging service policy/parameter provisioning to UE. | Create a new requirement: [PR 8.1-13] The 5G system shall be able to support ranging service policy/parameter provisioning to UE. |
| [PR 5.18.6-3] | The 5G system shall allow ranging service between 2 UEs trigged by and exposed to a third UE. | Create a new requirement: [PR 8.1-14] The 5G system shall allow ranging service between 2 UEs trigged by and exposed to a third UE. |
| [PR 5.18.6-4] | The 5G system shall allow ranging service between 2 UEs trigged by and exposed to the application server. | Create a new requirement: [PR 8.1-15] The 5G system shall allow ranging service between 2 UEs trigged by and exposed to the application server. |
| [PR 5.19.6-1] | The 5G system shall be able to support ranging service authorization for a UE or a group of UEs when a UE or a group of UEs will use ranging in licenced band. | Merged with [PR 5.2.6-1] |
| [PR 5.19.6-2] | The 5G system shall be able to support ranging service policy/parameter provisioning to UE. | Create a new requirement: [PR 8.1-16] The 5G system shall be able to support ranging service policy/parameter provisioning to UE. |
| [PR 5.19.6-3] | The 5G system shall be able to support one UE initiates ranging service to the other UE. | Create a new requirement: [PR 8.1-17] The 5G system shall be able to support one UE initiating Ranging to the other UE. |
| [PR 5.19.6-4] | The 5G system shall be able to support ranging service between UEs which subscribe to different operators. | Create a new requirement: [PR 8.1-18] The 5G system shall be able to support ranging service between UEs which subscribe to different operators. |
| [PR 5.19.6-5] | The 5G system shall be able to provide ranging service to roaming UEs. | Create a new requirement: [PR 8.1-19] The 5G system shall be able to provide Ranging to roaming UEs. |
| [PR 5.19.6-6] | The 5G system shall be able to authenticate the UE supporting Ranging service. | Create a new requirement: [PR 8.1-20] The 5G system shall be able to authenticate the UE supporting Ranging service. |
| [PR 5.19.6-7] | The 5G system shall be able to authorize the UE for enabling Ranging service. | Included by [PR 5.2.6-1] |
| [PR 5.19.6-8] | The 5G system shall be able to ensure the integrity and confidentiality of Ranging information used by Ranging-enabled UEs. | Create a new requirement: [PR 8.1-21] The 5G system shall be able to ensure the integrity and confidentiality of Ranging information used by Ranging-enabled UEs. |
| [PR 5.19.6-9] | The 5G system shall be able to ensure that user privacy is not violated during Ranging service, e.g., based on regulatory requirements. | Create a new requirement: [PR 8.1-22] The 5G system shall be able to ensure that user privacy is not violated during Ranging service, e.g., based on regulatory requirements. |
| [PR 5.19.6-10] | The 5G system shall be able to ensure security protection (e.g., interworking security) when the Ranging service concerns UEs subscribed with different operators. | Create a new requirement: [PR 8.1-23] The 5G system shall be able to ensure security protection (e.g., interworking security) when the Ranging service concerns UEs subscribed with different operators. |
| [PR 5.19.6-11] | The level of security provided by the existing 5G system shall not be adversely affected when Ranging service is enabled. | Create a new requirement: [PR 8.1-24] The level of security provided by the existing 5G system shall not be adversely affected when Ranging service is enabled. |

- Performance requirements

Except for museum tour use case (as there is only one KPI value), all other use cases are merged into one KPI table.

**************** First Change ******************

6	Consolidated requirements

6.1	Functional requirements

[PR 8.1-1] The 5G system shall be able to support for a UE to discover other UEs supporting Ranging.

[PR 8.1-2] The 5G system shall be able to authorize Ranging for a UE or a group of UE when using licensed spectrum.

[PR 8.1-3] The 5G system shall be able to protect privacy of a UE and its user, ensuring that no identifiable information can be tracked by undesired entities during Ranging.

[PR 8.1-4] The 5G system shall be able to enable or disable Ranging.

[PR 8.1-5] The 5G system shall support mutual Ranging, i.e. two UEs shall be able to initiate Ranging to each other.

[PR 8.1-6] The 5G system shall be able to ensure that the use of Ranging, if in licensed spectrum, is only permitted in network coverage under the full control of the operator who provides the coverage.

[PR 8.1-7] The 5G system shall support energy efficient UE Ranging operation.

[PR 8.1-8] The 5G system shall be able to start ranging and stop ranging according to the application layer’s demand.

[PR 8.1-9] The 5G system shall be able to provide mechanisms for a MNO, or authorized 3rd party, to provision and manage ranging operation and configurations.

[PR 8.1-10] The 5G system shall be able to support mechanisms for a UE to assist another UE to perform ranging of a third UE (if the requesting UE is LOS with the assisting UE and the assisting UE is LOS with the third UE).

NOTE: It cannot be assumed that all ranging UEs support the same application for exchange of information.

[PR 8.1-11] The 5G system shall be able to support ranging enabled UEs to determine the ranging capabilities (e.g. capabilities to perform distance and/or angle measurement) of other ranging enabled UEs.

[PR 8.1-12] The 5G system shall be able to allow a ranging enable UE to determine if another ranging enabled UE is stationary or mobile, before and/or during ranging.

NOTE: This may require assistance from other ranging enabled UEs.

[PR 8.1-13] The 5G system shall be able to support ranging service policy/parameter provisioning to UE.

[PR 8.1-14] The 5G system shall allow ranging service between 2 UEs trigged by and exposed to a third UE.

[PR 8.1-15] The 5G system shall allow ranging service between 2 UEs trigged by and exposed to the application server.

[PR 8.1-16] The 5G system shall be able to support ranging service policy/parameter provisioning to UE.

[PR 8.1-17] The 5G system shall be able to support one UE initiating Ranging to the other UE.

[PR 8.1-18] The 5G system shall be able to support ranging service between UEs which subscribe to different operators.

[PR 8.1-19] The 5G system shall be able to provide Ranging to roaming UEs.

[PR 8.1-20] The 5G system shall be able to authenticate the UE supporting Ranging service.

[PR 8.1-21] The 5G system shall be able to ensure the integrity and confidentiality of Ranging information used by Ranging-enabled UEs.

[PR 8.1-22] The 5G system shall be able to ensure that user privacy is not violated during Ranging service, e.g., based on regulatory requirements.

[PR 8.1-23] The 5G system shall be able to ensure security protection (e.g., interworking security) when the Ranging service concerns UEs subscribed with different operators.

[PR 8.1-24] The level of security provided by the existing 5G system shall not be adversely affected when Ranging service is enabled.

6.2	Performance requirements

| Ranging scenario | Ranging Accuracy (95 % confidence level) |  | Availability | Latency | Effective Ranging distance, Coverage, NLOS/LOS, Relative UE velocity, Ranging interval, Number of concurrent ranging operation for a UE, Number of concurrent ranging operation in an area |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Distance Accuracy | Direction Accuracy |  |  | Effective Ranging distance | Coverage | NLOS/LOS | Relative UE velocity | Ranging interval | Number of concurrent ranging operation for a UE | Number of concurrent ranging operation in an area |
| Distance based smart device control | 10cm | - | 99 % | 50ms | 20m | IC/PC/OOC | LOS | Static/ Moving (<1m/s) | 50ms | 20 | - |
| Smart TV remoter | 10cm up to 3 meter separation | ±2° horizontal direction accuracy at 0.1 to 3 meter separation and AoA coverage of (-60°) to (+60°); ±2° Elevation direction accuracy at 0.1 to 3 meter separation and AoA coverage of (-45°) to (+45°) | 99 % | 10ms | 10m | IC/PC/OOC | LOS | Static/ Moving (<1m/s) | 50ms | - | - |
| Smart Vehicle Key | 10 cm | - | 99 % | 50ms | 30m | IC/PC/OOC | LOS | Static/ Moving (<2m/s) | 25 | - | 50UEs/ (104m2) |
| Finding Items in a supermarket | 50 cm | 5 degree | 95 % | 500ms | 100m | IC/PC/OOC | LOS | Static/ Moving (<1m/s) | 250 | - | 100 UEs/ (3.14*104m2) |
| Touchless Self-checkout Machine Control | 10cm | - | 99% | 50ms | 1m | IC/PC/OOC | LOS | Static/ Moving (<1m/s) | 100ms | - | = |
| Hands Free Access | 10cm | - | [99 %] | 100ms | 10 m | IC/PC/OOC | LOS | Static/ Moving (1 m/s) | 50ms | - | 20 UEs/3.14*100m2 |
| Smart Transportation Metro/Bus Validation | 10cm | - | 99 % | 50ms | 2m | IC/PC/OOC | LOS | Static/ Moving (3km/h) | 50ms | 20 | 100 in the area of 8 m2 |
| distance based intelligent perception for public safety | 50cm | - | 99 % | - | 20m | IC/PC/OOC | LOS | Static/ Moving (<20km/h) | - | 100 | - |
| Picture and video sharing based on ranging results | 10cm | 2° | 99 % | 50ms | 10m | IC/PC/OOC | LOS | Static/ Moving (<1m/s) | 50ms | - | - |
| Ranging of UE’s in front of vending machine | 20cm | 10° | - | 150ms | 5m | IC/PC/OOC | LOS | Static/ Moving (<1m/s) | 50ms | - | 10 |
| Search and rescue | 20m | 5° | 99 % | 1s | 100m-TBD | IC/PC/OOC | LOS | Static/ Moving (up to 10m/s) | 5s | - | - |
| Long range approximate location | [10m] | ±[12.5°] | 99 % | - | 500m | IC/PC/OOC | NLOS | Static/ Moving (<10m/s) | - | 1 | [50]UEs/ (104m2) |

************* End of First Change ***************