3GPP TSG-SA WG1 Meeting #93-e	S1-210024r1

Electronic Meeting, 22 February – 4 March 2021	(revision of S1-204012)

Title:	FS_AMMT – New use-case on local AI/ML model split on factory robots

Agenda Item:	7.4.1

Source:	InterDigital

Contact:	Atle Monrad <Atle.Monrad@InterDigital.com>

Abstract

This contribution is a follow up of the contribution S1-204012r6 from the meeting #92e.

In factory 4.0, AI-driven robots are more and more commonly used to accomplish fast, complex and repetitive tasks. Some mobile robots will operate in open environments with human operators and other robots, and they shall adapt to new situations and be pro-active. This will require the acquisition and treatment in real-time of a huge amount of sensing data. The robots will rely on their own computing abilities to handle some of these data locally, but in some circumstances, some offloading mechanisms shall be put in place with edge servers (i.e. service hosting environment) and/or proximity robots. In that perspective, the autonomous robots shall be able to communicate via both a 5GC path and a D2D/sidelink path.

Proposal

The proposed Use Case is aligned with the SID of FS_AMMT (SP-191040) in which both communication interfaces, sidelink and 5G Uu, are identified to share data between intelligent agents.

It is proposed to include the below new use case on local AI/ML model split on factory robots in 3GPP TR 22.874 version 0.2.0.

* * * *   First Change   * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 22.891, Feasibility Study on New Services and Markets Technology Enablers

[3]	3GPP TR 22.863, Feasibility study on new services and markets technology enablers for enhanced mobile broadband

[4]	3GPP TS 22.261, Service requirements for the 5G system

[5]	3GPP TS 22.104, Service requirements for cyber-physical control applications in vertical domains

[6]	3GPP TR 37.910, Study on self evaluation towards IMT-2020 submission (Release 16)

[7]	A. Krizhevsky, I. Sutskever, and G. E. Hinton, “ImageNet classification with deep convolutional neural networks”, in Proc. NIPS, 2012, pp. 1097–1105.

[8]	K. Simonyan and A. Zisserman, “Very deep convolutional networks for large-scale image recognition,” 2014, arXiv:1409.1556. [Online]. Available: https://arxiv.org/abs/1409.1556

[9] 	C. Szegedy, et al., “Going deeper with convolutions”, in Proc. CVPR, 2015, pp. 1-9.

[10]	Zhi Zhou, Xu Chen, En Li, Liekang Zeng, Ke Luo, Junshan Zhang, “Edge intelligence: Paving the last mile of artificial intelligence with edge computing”, Proceeding of the IEEE, 2019, Volume 107, Issue 8.

[11]	Jiasi Chen, Xukan Ran, “Deep learning with edge computing: A review”, Proceeding of the IEEE, 2019, Volume 107, Issue 8.

[12]	I. Stoica et al., “A Berkeley view of systems challenges for AI”, 2017, arXiv:1712.05855. [Online]. Available: https://arxiv.org/abs/1712.05855

[13] 	Y. Kang et al., “Neurosurgeon: Collaborative intelligence between the cloud and mobile edge”, ACM SIGPLAN Notices, vol. 52, no. 4, pp. 615–629, 2017.

[14]	E. Li, Z. Zhou, and X. Chen, “Edge intelligence: On-demand deep learning model co-inference with device-edge synergy”, in Proc. Workshop Mobile Edge Commun. (MECOMM), 2018, pp. 31–36.

[15]	3GPP TR 38.913, Study on Scenarios and Requirements for Next Generation Access Technologies (Release 15)

[16]	B. Kehoe, S. Patil, P. Abbeel, and K. Goldberg, “A survey of research on cloud robotics and automation,” IEEE Transactions on automation science and engineering, vol. 12, no. 2, pp. 398–409, 2015.

[17]	Huaijiang Zhu, Manali Sharma, Kai Pfeiffer, Marco Mezzavilla, Jia Shen, Sundeep Rangan, and Ludovic Righetti, “Enabling Remote Whole-body Control with 5G Edge Computing”, to appear, in Proc. 2020 IEEE/RSJ International Conference on Intelligent Robots and Systems. Available at: https://arxiv.org/pdf/2008.08243.pdf

[18] 	K. He, X. Zhang, S. Ren, and J. Sun, “Deep residual learning for image recognition,” in Proc. IEEE CVPR, Jun. 2016, pp. 770-778.

[19] 	A. G. Howard et al., “MobileNets: Efficient convolutional neural networks for mobile vision applications,” 2017, arXiv:1704.04861. [Online]. Available: https://arxiv.org/abs/1704.04861

[20] 	B. Taylor, V. S.Marco, W. Wolff, Y. Elkhatib, and Z. Wang, “Adaptive deep learning model selection on embedded systems,” in Proc. ACM LCTES, 2018, pp. 31–43.

[21] 	G. Shu, W. Liu, X. Zheng, and J. Li, “IF-CNN: Image-aware inference framework for CNN with the collaboration of mobile devices and cloud”, IEEE Access, vol. 6, pp. 621–633, 2018.

[22] 	D. Stamoulis et al., “Designing adaptive neural networks for energy-constrained image classification”, in Proc. ACM ICCAD, 2018, Art. no. 23.

[23] 	Sergey Ioffe and Christian Szegedy. “Batch normalization: Accelerating deep network training by reducing internal covariate shift”, In ICML., 2015.

[24]	C.-J. Wu et al., “Machine learning at facebook: Understanding inference at the edge,” in Proc. IEEE Int. Symp. High Perform. Comput. Archit. (HPCA), Feb. 2019, pp. 331–344.

[25]	Vivienne Sze, Yu-Hsin Chen, Tien-Ju Yang, Joel S. Emer, “Efficient processing of deep neural networks: A tutorial and survey”, Proceeding of the IEEE, 2017, Volume 105, Issue 12.

[26]	Y. LeCun, Y. Bengio, and G. Hinton, “Deep learning,” Nature, vol. 521, no. 7553, pp. 436-444, May 2015.

[27]	“An All-Neural On-Device Speech Recognizer”, March 12, 2019, Posted by Johan Schalkwyk, https://ai.googleblog.com/2019/03/an-all-neural-on-device-speech.html

[28]	Yanzhang He, etc., “Streaming End-to-end Speech Recognition for Mobile Devices”, 2019 IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP 2019)

[29]	3GPP TS 22.243: "Speech recognition framework for automated voice services; Stage 1".

[30]	H. B. McMahan, E. Moore, D. Ramage, S. Hampson, and B. A. Y. Arcas, “Communication-efficient learning of deep networks from decentralized data”, Proc. of the International Confe rence on Artificial Intelligence and Statistics, Apr. 20 17. [Online]. Available: https://arxiv.org/abs/1602.05629

[31] 	“Federated Learning”, https://justmachinelearning.com/2019/03/10/federated-learning/

[32]	T. Nishio and R. Yonetani, “Client selection for federated learning with heterogeneous resources in mobile edge”, 2018, arXiv:1804.08333. [Online]. Available: https://arxiv.org/abs/1804.08333

[33]	E. Park et al., “Big/little deep neural network for ultra low power inference”, in Proc. 10th Int. Conf. Hardw./Softw. Codesign Syst. Synth., 2015, pp. 124–132.

[34]	Nguyen H. Tran ; Wei Bao ; Albert Zomaya ; Minh N. H. Nguyen ; Choong Seon Hong, “Federated Learning over Wireless Networks: Optimization Model Design and Analysis”, In proc. IEEE INFOCOM 2019 - IEEE Conference on Computer Communications

[35] 	Song Han, Xingyu Liu, Huizi Mao, Jing Pu, Ardavan Pedram, Mark A Horowitz, and William J Dally. “EIE: efficient inference engine on compressed deep neural network”, In 43rd International Symposium on Computer Architecture, IEEE Press, 243–254.

[36] 	V. Sze, “Efficient Computing for Deep Learning, AI and Robotics,” Dept EECS, MIT, Available online at https://lexfridman.com/files/slides/2020_01_15_vivienne_sze_efficient_computing.pdf

[37] 	V. Sze, Y. Chen, “Efficient Processing of Deep Neural Networks: A Tutorial and Survey” Proc. of IEEE, 2017, Available online at: https://www.semanticscholar.org/paper/Efficient-Processing-of-Deep-Neural-Networks%3A-A-and-Sze-Chen/3f116042f50a499ab794bcc1255915bee507413c

[38] 	Stanford University, CS231n – Lecture 5-7: CNN, Training NNs, Available at YouTube.com

[39] 	S. Han, J. Pool, J. Tran, and W, J. Dally, "Learning both weights and connections for efficient neural networks", NIPS, May 2015

[40]	P. A. Merolla, et al., “A million spikingneuron integrated circuit with a scalable communication network and interface”,Science, vol. 345, no. 6197, pp. 668–673, Aug. 2014.

[41]	R. Collobert, J. Weston, L. Bottou, M. Karlen, K. Kavukcuoglu, and P. Kuksa, “Natural language processing (almost) from scratch,” J. Mach. Learn. Res., vol. 12 pp. 2493–2537, Aug. 2011.

[42]	T. N. Sainath, A.-R. Mohamed, B. Kingsbury, and B. Ramabhadran, “Deep convolutionalneural networks for LVCSR”, in Proc. ICASSP, 2013, pp. 8614–8618.

[43]	L. P. Kaelbling, M. L. Littman, and A. W. Moore, “Reinforcement learning: A survey”, J. Artif. Intell. Res., vol. 4, no. 1, pp. 237–285, Jan. 1996.

[44] 	3 AI Trends for Enterprise Computing. [Online]. Available: https://www.gartner.com/smarterwithgartner/3-ai-trends-for-enterprise-computing/

[45] 	Shiming Ge ; Zhao Luo ; Shengwei Zhao ; Xin Jin ; Xiao-Yu Zhang, “Compressing deep neural networks for efficient visual inference”, In proc. 2017 IEEE International Conference on Multimedia and Expo (ICME)

[46]	3GPP TS 22.186, Enhancement of 3GPP support for V2X scenarios; Stage 1 (Release 16) v16.2.0

[47]	“Develop Smaller Speech Recognition Models with NVIDIA’s NeMo Framework”, https://developer.nvidia.com/blog/develop-smaller-speech-recognition-models-with-nvidias-nemo-framework/

[48]	Yusuf Aytar, Carl Vondrick, Antonio Torralba: "SoundNet: Learning Sound Representations from Unlabeled Video", 27 Oct 2016

[49]	Iacovos Ioannou et al.: "Distributed Artificial Intelligence Solution for D2D Communication in 5G Networks", 20 Jan 2020

[50]	Pimmy Gandotra et al.: "Device-to-Device Communication in Cellular Networks: A Survey"

[51]	Davide Villa et al.: "Internet of Robotic Things: Current Technologies, Applications, Challenges and Future Directions", 15 Jan 2021

[52]	Charles R. Qi et al.: "PointNet: Deep Learning on Point Sets for 3D Classification and Segmentation", 10 Apr 2017

[53]	3GPP TS 22.842: "Study on Network Controlled Interactive Services, NCIS"

[54]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2"

[55]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains; Stage 1"

[56]	https://www.robots.ox.ac.uk/~vgg/software/vgg_face/

[57]	Bee Lim et al.: "Enhanced Deep Residual Networks for Single Image Super-Resolution" 10 Jul 2017

[58]	Gao Huang et al., MULTI-SCALE DENSE NETWORKS FOR RESOURCE EFFICIENT IMAGE CLASSIFICATION, 7 Jun 2018

* * * *   Second Change   * * * *

## 5.x	Local AI/ML model split on factory robots

### 5.x.1	Description

In a modern factory, a team on a workstation comprises two human operators, two mobile robots and a fixed robot. Everyone has his own pre-defined task. Robots assist the human operators by accomplishing painful tasks in a fluid and precise manner; they also monitor that the workstation environment remains safe for the human operators. The mobile robots shall not interfere with humans and between them.

The robot control is not executed on a distant server in the cloud because reliability and confidentiality were not ensured at a sufficient level. Furthermore, as stated in [51], the overall end-to-end latency is not always guaranteed which can cause production loss. Communications between robots can rely on private wireless networks in the factory that enable expected QoS (reliability, throughput, and latency) as well as confidentiality.

The new robots are autonomous robots that can react to human voices or learn in real-time what operators do. They can perceive their environment and transmit information to other robots. They can communicate, learn from each other, assist each other and do self-monitoring.

The autonomous robot’s skills rely on several AI/ML models running on the robot itself which has the inconvenience that the mobile robot’s battery drain quicker. To overcome this issue, when the battery level reaches a certain value, a part of the AI/ML model can be transferred to a service hosting environment and / or to another robot by splitting the AI/ML model as defined in [13]. The split model approach of [13] is applicable to a UE-to-UE (or robot-to-robot) architecture. Thus, the AI/ML model M is split and shared between (e.g.) 2 robots, say an assisted robot and an assistant robot. Intermediate data generated by the assisted robot are transferred to the assisting robot which finalizes the inference and transmits the results back to the assisted robot. This intermediate data transfer shall be extremely efficient in terms of latency and throughput. When many models are at stake, the split model method is an additional challenge for the 5GS in terms of throughput, latency and synchronization.

Because they are more autonomous, mobile and smart, the industry 4.0 robots embed a large variety of sensors that generate huge amount of data to process. Table 5.x.1-1 reflects that variety.

Each type of sensing data requires a different AI/ML model. Each of these models shall produce predictions in a certain delay and with a certain accuracy.

Thus, as an offloading strategy, we may imagine that a model is split between 2 robots because it has been established that for a particular AI/ML model, the latency with the sidelink communication was better (smaller) than with the regular 5G communication path, as stated in [49] and [50].

At the same time, other AI/ML models are split between the robot and the service hosting environment because from an energy standpoint this configuration is the best.

Table 5.x.1-2 is an example of this offloading strategy where four AI/ML models are split between a robot and the service hosting environment and four other AI/ML models are split between two robots.

Sidelink and 5G communication paths are complementary from an AI/ML model split policy standpoint.

Table 5.x.1-1 shows some typical and diverse AI/ML models that can be used on robots. For each model, all the split point candidates have been considered and only the split points that generate the minimum and the maximum amount of intermediate data have been noted.

Table 5.x.1-1: Intermediate AI/ML data size per AI/ML model

| Model Name | Model type | Intermediate data size (MB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 8 bits data format |  | 32 bits data format |  |
|  |  | Min | Max | Min | Max |
| AlexNet [7] | Image recognition | 0.02 | 0.06 | 0.08 | 0.27 |
| MSDNet [58] | Image recognition | 0.056 | 2.15 | 0.224 | 8.6 |
| ResNet50 [18] | Image recognition | 0.002 | 1.6 | 0.008 | 6.4 |
| SoundNet [48] | Sound recognition | 0.0017 | 0.22 | 0.0068 | 0.88 |
| PointNet [52] | Point Cloud | 0.262 | 1.04 | 0.0068 | 4.19 |
| VGGFace [56] | Face recognition | 0.000016 | 0.8 | 0.000064 | 3.2 |
| Inception resnet | Face recognition | 0.0017 | 0.37 | 0.0068 | 1.51 |
| EDSR [57] | Super resolution | 0.027 | 9.4 | 0.11 | 37.7 |

In Table 5.x.1-2 the AI/ML models are distributed between the service hosting environment and the proximity robot. The way the models are distributed is out of scope of this use case and may depend on various criteria as said previously. Therefore, the next table is an example that illustrates the distribution. The intermediate data size are presented with the range [Min – Max], where Min and Max are respectively the sum of the Min values and the sum of the Max values of the selected models as defined in Table 5.x.1-1 (figures in bold).

Table 5.x.1-2: Example of models distribution and data rate for intermediate AI/ML data

| Model Name | Offloading target | Intermediate data size (MB) | Transfer time (ms) | Data rate (Gb/s) |
| --- | --- | --- | --- | --- |
| AlexNet [7] | Service Hosting Environment | [0.049 – 47.5] | 10 | [ 0.04 - 38] |
| ResNet50 [18] |  |  |  |  |
| VGGFace [56] |  |  |  |  |
| EDSR [57] |  |  |  |  |
| MSDNet [58] | Proximity robot | [0.25 – 15.18] | 10 | [ 0.248 – 12.14] |
| SoundNet [48] |  |  |  |  |
| PointNet [52] |  |  |  |  |
| Inception resnet |  |  |  |  |

As previously said latency is a critical requirement. Figure 5.x.1-3 summarizes the latency cost in three scenarios:

- The inference of model M is done locally. Latency is denoted LLI.

- The inference process is fully offloaded on a second device (Robot/UE). Latency is denoted LFO.

- The inference process is partially offloaded on a second device (Robot/UE). Latency is denoted LPO.

Figure 5.x.1-3 Latency summary

The current Use Case promotes the scenario (C) where a model M is split in two sub-models Ma and Mb. If both robots (UEs) have a similar computing power, the assumption is that the latency due to the inference of model M is almost equal to the latency of model Ma plus the latency of model Mb.

Hence, once the split model is deployed on the two robots (UEs), the aim is to minimize the E2E latency and to be as close as possible to the non-split case. This is done with a transfer delay of both the intermediate data and the inference results as small as possible. We can note that if the computing power on the assistant robot is more important, then scenario (C) would be the preferred scenario.

In scenario (B), the inference process is fully offloaded on the assistant robot (UE). The major inconvenience is the strong and negative impact on latency of the raw data transfer towards the assistant robot.

### 5.x.2	Pre-conditions

Two human operators are working.

Two mobile AI-driven robots (Arobot and Brobot) and one static AI-driven robot (Crobot) assist them.

The three robots (Arobot , Brobot and Crobot ) belong to the same service area, embed the same two powerful AI/ML models M1 and M2, sensors (e.g. LIDAR, microphone) and cameras (e.g. 8K videos stream).

Arobot and Brobot are powered with a battery and Crobot with fixed ground power.

The three robots (Arobot , Brobot and Crobot) are connected.

The three robots (Arobot , Brobot and Crobot) are D2D-enabled and they authorize the resource sharing (energy and computing power).

The workstation is equipped with camera and sensors.

The service area is 30 m x 30 m and the robot speed is at maximum 10 km/h.

The service area is covered by a small cell and a service hosting environment is connected and can support AI/ML model processes.

### 5.x.3	Service Flows

Figure 5.x.3-1 Factory service flow

- Brobot battery is rather low but it can still work for a while if a part of its machine learning process is offloaded.

- Brobot broadcasts a request message to get assistance. Crobot and the service hosting environment response positively.

- Brobot negotiates with Crobot and the service hosting environment what parts of M1 and M2 for the inference process they may have the charge of, knowing that the quality of the prediction shall not be under a certain level and that the end-to-end latency shall not be above a certain value. M1 model is split between Brobot and the service hosting environment. M2 model is split between Brobot and Crobot.

- Brobot, Crobot and the service hosting environment agree on split points for both M1 and M2 models and Brobot starts sending the intermediate data to Crobot and the service hosting environment.

- Crobot infers and transmits in unicast the predictions back to Brobot in a very short delay. The service hosting environment infers and transmits in unicast the predictions back to Brobot in a very short delay.

- In the meanwhile, Arobot is carrying a load to the operator Aoperator.

- Aoperator bends down to pick up a screw that has fallen on the floor. At the same time Boperator is passing between Aoperator and Arobot. Arobot can’t see Aoperator anymore.

- Brobot is busy with another task but it can see the scene. It reports the scene as intermediate data to Crobot and the service hosting environment.

- Crobot and the service hosting environment infer and then transmit in unicast the prediction back to Brobot The safety application on the service hosting environment collects the inference results.

### 5.x.4	Post-conditions

Intermediate data can be exchanged between two robots (UEs) and / or service hosting environment, and the robot with a low battery level can continue working for a while.

The offloading effort between the two robots is transient.

All the robots in the group receive the alert message and react:

- They all stop working; or

- Arobot changes its trajectory.

Aoperator and Boperator can work safely.

The huge amount of data that is required for inferring is kept local in the factory.

### 5.x.5	Existing features partly or fully covering the use case functionality

The Use Case can rely on the Proximity Service (ProSe) services as defined in 3GPP TS 23.303 [54].

Cyber-Physical Control Applications, see 3GPP TS 22.104 [55], already proposes to rely on a ProSe communication path. The proposed requirements are limited in terms of data transfer as shown in Table 5.2-1, where the message size does not exceed a few hundred of Bytes (250 kB at maximum).

In the study on Network Controlled Interactive Services, see 3GPP TR 22.842 [53], the maximum bitrate is 10 Gbps with a latency of 20 ms:

[P.R.5.2-003] The 5G system shall be able to determine exchanging interactive service data via the ProSe Communication path (direct) or via the 5GC path in order to guarantee the user experienced data rate and latency.

### [P.R.5.2-008] The 5G system shall support packets transmission between UEs with 0.1-[10] Gbps data rates for the ProSe Communication path (direct), and the end-to-end latency plus processing delay should satisfy the upper limit of motion-to-photon latency (including tracking, pose delivery, viewport rendering, encoding, delivery, decoding and display), i.e., 20 ms.

### These requirements could partially cover the current Use Case needs.

### 5.x.6	Potential New Requirements needed to support the use case

[P.R.5.x-001] The 5G system shall support the transfer of AI/ML model intermediate data from UE to UE via the D2D/sidelink communication path.

[P.R.5.x-002] The 5G system shall support a 15 Gb/s data rate from UE to UE via the D2D/sidelink communication path.

[P.R.5.x-003] The 5G system shall guarantee a latency of 10 ms from UE to UE via the D2D/sidelink communication path.

NOTE 1:	Requirements [P.R.5.x-002] and [P.R.5.x-003] are related to Table 5.x.1-2

[P.R.5.x-004] The 5G system shall guarantee a communication service availability higher than 99.9999 % from UE to UE via the D2D/sidelink communication path within a service area of 30 m x 30 m when robots are moving with a speed less than 10 km/h.

[P.R.5.x-005] The 5G system shall maintain theQoS (latency, reliability, data rate as defined above) of the D2D/sidelink communication path.

[P.R.5.x-006] The 5G system shall have the means to modify the QoS of the D2D/sidelink communication path.

NOTE 2:	The split point selection is dynamic. In consequence, the amount of intermediate data will vary. To maintain the QoS, the bandwidth is adjusted.

* * * *   End-of Changes   * * * *