3GPP TSG-SA WG1 Meeting #92e	S1-210044

Electronic Meeting, 2 February – 4 March 2021

Title:	FS_5TRS Charging aspects

Agenda Item:	7.8.1

Source:	Nokia, Nokia Shanghai Bell

Contact:	Betsy Covell	betsy.covell@nokia.com

Abstract: This contribution provides several new potential requirements for charging data collection based on the new functionality provided with the 5G timing resiliency service.

First Proposed Modified Text:

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	National Institute of Standards and Technology (NIST), "Timing Challenges in the Smart Grid", January 2017.

[3 ]	61850-9-3-2016 - IEC/IEEE International Standard - Communication networks and systems for power utility automation – Part 9-3: Precision time protocol profile for power utility automation.

[4]	IEEE 1588-2019, IEEE Standard for a Precision Clock Synchronization Protocol for Networked Measurement and Control Systems.

[5] 	Used with permission from ABB Distribution Solutions.

[6]		3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains".

[7]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN"

[8]	ATIS-0900005: "Technical Report on GPS Vulnerability", https://access.atis.org/apps/group_public/download.php/36304/ATIS-0900005.pdf

[9]	3GPP TR 22.867: "Study on 5G Smart Energy and Infrastructure"

[10]	NPL: "Time Traceability for the Finance Sector, Fact Sheet", version 2.0.

[11]	BIPM Time Department, https://www.bipm.org/en/bipm/tai/

[12]	Markets in Financial Instruments (MiFID II) - Directive 2014/65/EU
https://ec.europa.eu/info/law/markets-financial-instruments-mifid-ii-directive-2014-65-eu_en

[13]	Regulatory Technical Standard 25. Level of accuracy of business clocks
https://ec.europa.eu/finance/securities/docs/isd/mifid/rts/160607-rts-25_en.pdf

[14]	Annex to Regulatory Technical Standard 25, https://ec.europa.eu/finance/securities/docs/isd/mifid/rts/160607-rts-25-annex_en.pdf

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

holdover: A clock A, previously synchronized/syntonized to another clock B (normally a primary reference or a Master Clock) but whose frequency is determined in part using data acquired while it was synchronized/syntonized to B, is said to be in holdover or in the holdover mode as long as it is within its accuracy requirements.

### NOTE:	holdover is defined in [4]

### 5.2.6	Potential new requirements

[PR 5.1.6-1] The 5G system shall be able to receive accurate timing signals from one or more independent timing source(s), which can offer a timing alternative to GNSS, e.g. TBS/MBS [7] [8]], Sync over Fiber [7].

[PR 5. 1.6-2] The 5G system shall be able to detect when GNSS reference timing signals are no longer viable for network time synchronization.

[PR 5.1.6-3] The 5G system shall be able to maintain accurate time synchronization as appropriate for the supported applications in the event of degradation or loss of GNSS timing signals.

[PR 5.1.6-4] The 5G system shall be able to collect charging information based on the timing source (e.g., the source in use, start and stop of source usage).

Second Proposed Modified Text:

#### 5.2.6.1	Potential Requirements

[PR 5.2.6.1-1] The 5G system shall monitor for timing service failure (e.g., detect QoS degradation).

[PR 5.2.6.1-2] The 5G system shall be able to indicate to devices (e.g., UEs, applications) using a timing service that they need to use an alternate source (e.g., use 5G system with internal holdover capability or an alternate source, e.g. atomic clock, Sync over Fiber, TBS).

[PR 5.2.6.1-3] The 5G system shall support a holdover capability (e.g., maintaining required accuracy to UTC) of up to 24h.

[PR 5.2.6.1-4] A time synchronization device connected to the 5G system shall support a holdover capability of >5s.

[PR 5.2.6.1-5] The 5G system shall be able to detect and indicate to devices (e.g., UEs, applications) when GNSS reference timing signals are restored for network time synchronization.

[PR 5.2.6.1-6] The 5G system shall be able to collect charging information per UE for use of a timing resiliency service (e.g., start/stop time and source used by a UE, timing service used by UE, holdover capability of the service).

[PR 5.2.6.1-7] The 5G system shall be able to collect charging information on 5G system timing resiliency service (e.g., service KPIs, holdover capability, number of UEs using the service).

Third Proposed Modified Text:

#### 5.4.6.1	Potential Requirements

[PR 5.4.6.1-1] The 5G system shall support a mechanism to monitor time distribution at time equipment within the network for reporting timestamp measurement uncertainty.

[PR 5.4.6.1-2] The 5G system shall support mechanisms to calibrate and report time information for network entities involved in the timing distribution chain.

[PR 5.4.6.1-3] The 5G system shall support a mechanism to report UTC traceability information to 3rd party applications.

[PR 5.4.6.1-4] The 5G system shall support a mechanism to manage and correlate (e.g., compare, prioritize, correct) clock information from multiple time sources in the network.

[PR 5.4.6.1-5] The 5G system shall support a mechanism to configure granularity of the timestamp for cases where UE/DS-TT performs the timestamping of the financial transaction.

[PR 5.4.6.1-6] The 5G system shall be able to collect charging information per application, including 3rd party application (e.g., timing resiliency service KPIs, holdover capability, number of UEs using the service, timing source).