3GPP TSG-SA WG1 Meeting #93e 	S1-210229r2

E-Meeting    22 Feb - 04 Mar  2021	(revision of S1-21xxxx)

Title:	Enable support for user centric identifiers and authentication in PIN and Residential

Agenda Item:	7.12.1

Source:	Intel

Contact:	Ellen Liao, ellen.c.liao at Intel.com

Abstract: this PCR proposed clarification on the support of user centric identifiers and authentication in PIN and Residential.

Proposal:

This PCR proposes clarification to address editor’s note of SR002.

In the TS22.101 clause 26a, the 5G network operator can act as an identity provider for users to be identified could be:

- an individual human user, using a UE with a certain subscription, or

- an application running on or connecting via a UE, or

- a device (“thing”) behind a gateway UE.

These users are associated to the 3GPP devices (UE or gateway UE) which are subscribers of the 5G network. The 5G network can trust and identify a user based on the authenticated 3GPP device that is associated to the user.

The following service requirement in TS22.101 clause 26a provide the principle for user centric identifiers and authentication by the 3GPP system:

The 3GPP System shall support operators to act as User Identity provider and to authenticate users for accessing operator and non-operator deployed (i.e. external non-3GPP) services

As such, in the context of PIN and residential environment, this PCR proposed that the 5G network can trust and identify a user based on the authenticated “non-3GPP device” that is associated to the user for supporting UEs to access non-operator deployed services (external non-3GPP services). That is, it is proposed that the application running on or connected to a non-3GPP device (“thing”) can be a user (application associated to a non-3GPP device) for being managed by the 5G network operator. The trust between non-3GPP device connected to an eRG and an application connected to the non-3GPP device can be established when the application and non-3GPP device are both authenticated by the 5G network operator, as shown in Figure X.

This PCR proposed to enable support of the user centric identifiers and authentication in the PIN and Residential network

Figure X: overview of network operator supports acting as User Identity provider for user centric identifiers and authentication in Residential environment or PIN

Editor’s Note: this use case considers only PIN. The related eRG description and requirements are applicable for connecting non-3GPP devise in Customer Premises Network (CPN), which needs to move to TR22.858.

*** 1st Change ***

## 5.5	Use case: UE accessing Services provided by PIN Elements behind 5G enabled gateway(s)

### 5.5.1	Description

There are more and more PIN Elements, e.g. smart watch, smart pet collar, earbuds, VR goggle headset, etc., or non-3GPP devices, e.g. media server, printer, smart thermostat/sprinkler/blinds, NAS server, etc., that can provide services for users at home or out of home. These PIN Elements or non-3GPP devices are usually behind a wireless gateway. In recent years, there are some security risks found in such settings due to port forwarding and unsecure connectivity provided by the wireless gateway for in home devices.

When considering the gateway with 5G capability for accessing 5G services, e.g. gateway UE (aka PIN Element with Gateway Capabilities) or evolved Residential Gateway  (eRG), it is important to enable the support of the secure connectivity for allowing authorized users from anywhere in the world to access authorized services provided by these PIN Elements or non-3GPP devices in terms of user authentication and authorization.

Editor's Note: evolved residential gateway (eRG) is defined in TR 22.858 [6].

Editor’s Note: this use case considers only PIN. The related descriptions and requirements for PIN Element with Gateway Capabilities are also applied to evolved residential gateway (eRG) defined in 3GPP TR 22.858 [6] for supporting non-3GPP devices connected to eRG in Customer Premises Network (CPN). A new use case in TR22.858 [6] is needed to accommodate these eRG related cases and requirements.

Figure 5.5.1-1 shows the scenarios of the 5G network enabling connectivity service support for the UE using 3GPP indirect (case a) or direct (case b) communication or non-3GPP access (case c) to access services provided by PIN Elements in PIN or non-3GPP devices in CPN. Each PIN Element or non-3GPP device may provide one or more services. For example, in PIN, the PIN Element is a smart watch, smart pet collar, etc. For example, in CPN, the non-3GPP device is a media server, smart TV, smart video doorbell, etc., which provide one media service. For another example, the non-3GPP device is a NAS server which can provide multiple services, e.g. media service, web server service, live security cams services, etc.

Figure 5.5.1-1: 5G network support for a User/UE accessing services provided by in Home Devices

Editor’s Note: In this figure, it is assumed that PIN Elements connecting to PIN Element with Gateway Capabilities in PIN are also applied to non-3GPP devices connecting to evolved residential gateway (eRG) defined in 3GPP TR 22.858 [6] in CPN. A new use case in TR22.858 [6] is needed to accommodate these cases for non-3GPP devices connecting to eRG in CPN.

In Figure 5.5.1-1, a user using an authorized UE, e.g. smartphone or tablet, accesses the service A provided by an PIN Element which has connection with eRG via a non-3GPP access technologies, e.g. WiFi, Bluetooth, fiber, etc., or 3GPP direct communication.

Case (a): the user/UE is out of Home and uses service A provided by a PIN Element behind a PIN Element with Gateway Capabilities  which relays traffic to and from 5G network.

Case (b): the user/UE operating as a PIN Element is at home and uses service A via 3GPP direct communication or non-3GPP access, e.g. WiFi, with PIN Element with Gateway Capabilities  which supports communication between two PINs.

Case (c): the user/UE operating as a relay UE is at home and uses service A directly provided by the PIN Element via a non-3GPP access technology, e.g. Bluetooth, WiFi, or 3GPP direct communication.

One or more IoT devices can connect to these PIN Element. For example, the UE and the IoT devices, e.g. earbuds, VR goggle headset, etc., form a personal IoT network, e.g. PIN#2 in Case (b). Also, one or more services can run on or connected to these PIN Elements, e.g. a NAS server, which forms another PIN, e.g. PIN#1, in Case (a)(b)(c).

To avoid the potential security/privacy risks that invade the PIN Elements and services, it is important that the 5G network can enable supports of secure access to the PIN Elements  and their services for authenticated and authorized users. According to 3GPP TS 22.101 [3] clause 26a, an "User" to be identified could be

-	an individual human user, using a UE with a certain subscription, or

-	an application running on or connecting via a UE, or

-	a device ("thing") behind a gateway UE.

In the context of PIN (personal IoT network), a "User" includes a service ("application”) running on or connected via a PIN Element behind a PIN Element with Gateway Capabilities.

The following service aspects for PIN Element as non-3GPP device connected to the network via a UE as PIN Element with Gateway Capabilities needs to be considered:

-	User Identifiers and user authentication;

1.	User Identifiers of services provided by PIN Element;

2.	The authentication for these services provided by PIN Element;

-	Access to services provided by PIN Elements;

-	User Profiles and User Identifiers for services provided by an PIN Element;

-	UE policies in the home settings.

### 5.5.2	Pre-conditions

The Incredible family adopts civilian identities and lives at suburbs for a normal life to hide their superheroes identities. To ensure secure communication for the Incredible family accessing services of PIN Elements from anywhere in the world, Mr. Incredible sets up many non-3GPP IoT devices, e.g. smart watch, smart pet collar, earbuds, VR goggle headset, etc. in PIN; personal data storage, smart devices for home automation, home security cameras, etc. in CRN (Customer Premises Network), and subscribes to a reliable network operator’s services, which can act as User Identity provider and provide secure access to his PIN Elements and the applications running on or connected to the PIN Element behind the PIN Element with Gateway Capabilities, for all UEs of his family, including smartphones, tablets, and one PIN Element with Gateway Capabilities.

### 5.5.3	Service Flows

Step1: [Users configuration]

Mr. Incredible, signs in his account at operator’s network that provides 5G connectivity services for all his UE devices. In his account, there are two listed UE devices/subscriptions with gateway UE capabilities, including one smartphone and one eRG. In Mr. Incredibles account, he can create user accounts for all his family members, and indicates Users with User Identities for family members, PIN Elements, and services provided by PIN Elements.

Further, for each service of the PIN Elements  behind PIN Element with Gateway Capabilities, Mr. Incredible configures User Profiles, e.g. via scanning the QR code of the device to get some information and editing details manually. For each service identified by a User Identity, it can have one or more User Profile(s) and each User Profile contains the following information:

-	User Identifier

-	Specific service settings and parameters, e.g. active/inactive time, number of accesses, etc.

-	Authentication/authorization policy and access restriction policy required for the service, which are going to be used to authenticate/authorize a User for accessing to the service of the PIN Element.

-	Credential information, e.g. password for the authorized service, security keys for encryption/decryption, and hash algorithm for message digital signing, etc.

For an authorized human user(s), its User Profile can indicate the authorized service identified by User Identity and allowed User Identifiers.

Step2: [Registration of PIN Element and Update of User Profiles for services provided by the PIN Element]

(2a): When an PIN Element is turned on, the eRG discovers and connects to the PIN Element at the first time, the eRG determines if the PIN Element is an authorized User identified by a User Identity indicated in its UE configuration.

(2b): The serving network of the eRG authenticates User Identity of the PIN Element based on its credentials, and then updates User Profiles of the services provided by the PIN Element. In return, the network responds the  PIN Element with Gateway Capabilities with the authentication result and updated User Profiles of the registered services.

(2c): The serving network of the eRG further provides updated User Profiles of the services to 5G subscriber’s HPLMN. The HPLMN of  PIN Element with Gateway Capabilities updates its stored User Profiles of all impacted Users.

(2d): Based on serving network’s policies, the serving network can update User Profiles of impacted Users and UE configuration towards eRG.

Step3: [User/UE at home or out of home accessing services provided by PIN Elements]

Violeta, Mr. Incredibles daughter as an authorized user (User) of the service of a PIN Element, would like to use the authorized UE to access a registered application-A of a PIN Element behind a PIN Element with Gateway Capabilities.

-	Case (a): When Violet (User) using authorized UE is out of home, the  PIN Element with Gateway Capabilities uses connections with 5G network for the UE to request application-A of the PIN Element behind the PIN Element with Gateway Capabilities.

-	Case (b): When Violet (User) using authorized UE is at home, the  PIN Element with Gateway Capabilities discovers and connects the UE acting as an PIN Element and using 3GPP direct communication or non-3GPP accesses, based on stored UE policies of the UE or user preferences.

-	Case (c): When  Violet (User) using authorized UE is at home, the UE acting as a gateway UE discovers and connects with PIN Element directly via a non-3GPP access technologies, e.g. Bluetooth, WiFi, or via 3GPP direct communication, instead of via indirect communication over PIN Element with Gateway Capabilities, based on stored UE policies of the UE or user preferences.

(3b): The User/UE requests to access application-A of the PIN Element. Based on stored User Profiles of the PIN Element with allowed Users, the PIN Element with Gateway Capabilities can determine whether to accept access request of the PIN Element from the User.

(3c): The PIN Element with Gateway Capabilities can further perform user authentication of the application-A based on the security polices and credentials in stored User Profiles of the application-A.

(3d): The PIN Element with Gateway Capabilities forwards the service access request to the PIN Element only if the user authentication is successful. Otherwise, the PIN Element with Gateway Capabilities rejects the request for service access.

(3e): The PIN Element with Gateway Capabilities starts to forward the traffic between the PIN Element and the UE.

-	For Case (a), the  PIN Element with Gateway Capabilities supports indirect communication for forwarding the traffic over 5G network between the PIN Element and the UE.

-	For Case (b), the  PIN Element with Gateway Capabilities supports communication at home for forwarding the traffic between two PINs, i.e. the UE and the PIN Element.

-	For Case (c), the gateway UE supports communication at home for forwarding traffic between two PINs, e.g. connected earbuds via Bluetooth and connected PIN Element, e.g. smart watch.

Step4: [UE policies in the home settings]

When the authorized User/UE moves from out of home, i.e. case (a), to in home, i.e. case (b) or case (c), the User can manually determine how the used UE adopts case(a)/case (b)/case(c), or UE can automatically adapt to case(a)/ case(b)/case (c) based on the UE policies, including the following information provisioned by the 5G network:

-	one or more operation modes (PIN Element, UE, gateway UE);

-	communication methods (3GPP indirect communication, 3GPP direct communication, or non-3GPP access);

-	location information.

### 5.5.4	Post-conditions

The Incredible family can safely live with hidden superheroes identities by securely accessing application of the PIN Elements from anywhere in the world without compromising the security of the PIN Elements/services at home.

### 5.5.5	Existing features partly or fully covering the use case functionality

Referring to 3GPP TS22.101 [3] clause 26a, the user to be identified could be an individual human user, using a UE with a certain subscription, or an application running on or connecting via a UE, or a device (“thing”) behind a gateway UE. The following service requirements have been supported:

-	The 3GPP network shall be able to provide a User Identifier for a non-3GPP device that is connected to the network via a UE that acts as a gateway.

-	The 3GPP network shall support to perform authentication of a User Identity used by devices that are connected via a UE that acts as a gateway.

-	The User Identifier may be provided by some entity within the operator’s network or by a 3rd party.

-	The 3GPP system shall be able to take User Identity specific service settings and parameters into account when delivering a service.

NOTE: 	The requirement applies to 3GPP services and non-3GPP services that are accessed via the 3GPP System

-	The 3GPP system shall be able to store and update a User Profile for a user.

-	The 3GPP System shall support to authenticate a User Identity to a service with a User Identifier.

-	When a user requests to access a service, the 3GPP System shall support authentication of the User Identity with a User Identifier towards the service if the level of confidence for the correct association of a User Identity with a User Identifier complies to specified policies of the service.

-	A service shall be able to request the 3GPP network to only authenticate users to the service for which the association of the user with a User Identifier has been established according to specified authentication policies of the service.

-	Subject to operator policy, the 3GPP system shall be able to update User Profile related to a User Identifier, according to the information shared by a trusted 3rd party.

-	The User Profile may include one or more pieces of the following information:

-	additional User Identifiers of the user's User Identities and potentially linked 3GPP subscriptions,

-	used UEs (identified by their subscription and device identifiers),

-	capabilities the used UEs support for authentication,

-	information regarding authentication policies required by different services and slices to authenticate a user for access to these services or slices.

-	User Identity specific service settings and parameters. 
Those shall include network parameters (e.g. QoS parameters), IMS service (e.g. MMTEL supplementary services) and operator deployed service chain settings.

-	User Identity specific network resources (e.g., network slice).

The following service requirement in TS22.101 clause 26a provide the principle for user centric identifiers and authentication by the 3GPP system:

-  	The 3GPP System shall support operators to act as User Identity provider and to authenticate users for accessing operator and non-operator deployed (i.e. external non-3GPP) services.

### 5.5.6	Potential New Requirements needed to support the use case

Editor's Note: it is assumed the service requirements related to PIN Element with Gateway Capabilities are also applied to evolved residential gateway (eRG) defined in 3GPP TR 22.858 [6] for supporting non-3GPP devices connected to eRG in Customer Premises Network (CPN). A new use case in TR22.858 [6] is needed to accommodate these eRG related requirements.

[PR-5.5.6-1]: The 5G network shall enable support for an authorized UE to securely access the authenticated and authorized application running on or connected to a PIN Element behind a PIN Element with Gateway Capabilities.

[PR-5.5.6-2]: The 5G system shall be able to store and update a User Profile associated to PIN Element or an application running on or connected to a PIN Element behind the PIN Element with Gateway Capability and the User Profile shall include one or more pieces of the following information related to this application:

-	User Identifier

-	Specific service settings and parameters, e.g. active/inactive time, number of accesses, etc.

-	Authentication/authorization policy and access restriction policy required for the application, which are going to be used to authenticate/authorize a user for accessing to the application of the PIN Element.

-	Credential information, e.g. password for the authorized service, security keys for encryption/decryption, and hash algorithm for message digital signing, etc.

[PR-5.5.6-3]: The 5G system shall support operator to act as User Identity provider and to authenticate user identity associated to a PIN Element or an application running on or connected to it.

*** End of Change ***