3GPP TSG-SA WG1 Meeting #93e 	S1-210272r1

Electronic Meeting, 22 February – 4 March	 (revision of S1-20xxxx)

Title:	New use case on tracking of device position

Agenda Item:	7.10.1 - FS_RANGING

Source:	Philips International B.V.

Contact:	Walter Dees <walter.dees@philips.com>

Abstract: This contribution proposes a new use case for TR 22.855 on emergency response solution. All text is new.

* * * * Start of Change 1 * * * *

5.X     Tracking of devices

### 5.X.1 	Description

A mass casualty incident (MCI) describes an event in which the emergency medical services may get overwhelmed by the number and severity of the casualties. An MCI may include  chemical, biological, radiological emergencies, mass shootings, major road accidents, buildings being on fire,  natural disasters such as earthquakes, etc., which can cause result in loss of human lives in the MCI area. Digital technologies can help to improve the response time of the first responders and reduce the amount of damage and/or loss of life by improving the logistics of clinical and non-clinical operations in an MCI event and real-time victim monitoring in the MCI area. Triaging is a process applied when there are more casualties requiring aid than there are medical personal available in the MCI area. Real-time positioning and grouping of the victims in the triage area and health monitoring can aid the first responders to save a lot of precious time for the first responders to make swift yet accurate and group-level decisions to reduce loss of life in the MCI area. A triage generally consists of a quick check for immediate life-threatening concerns for the victims present in the triage area. An example of the triaging process is the START system (simple triage and rapid treatment) in which the first responder assigns each patient to one of four color-coded triage levels, based on their breathing, circulation, and mental status. The triage levels as described in START system are:

- Immediate (Red color): Victims with major life-threatening injuries, but are salvageable given the resources available

- Delayed (Yellow color): Victims with non-life-threatening injuries, but are unable to walk or exhibit an altered mental status

- Walking wounded (Green color): Victims who are able to move themselves out of the incident area to a treatment area

- Deceased or expectant (Black color): Victims who are dead, or whose injuries make survival unlikely.

In this use case, a massive terrorist attack has taken place in an airport at the arrivals hall located in the second floor of the airport building. More than 100 victims are injured, classifying the event as a mass casualty incident (MCI). The public telecommunication services are majorly interrupted, with no positioning system accessible within the airport building. A first responder crew of 3 members have arrived at the scene to identify, triage, and transport the injured victims prioritized based on triage status to the nearest hospitals. Tom, a first responder, has arrived at the scene with a first responding vehicle which can serve as a temporary non-public mobile network AmbCel. Tom have started to triage the victims based on the severity of the injuries using color coding scheme similar to the START system. After being triaged, every victim receives a body-worn ranging-enabled victim-tags (e.g. wearable patient monitor) from Tom. Once the logistics team has arrived, they started to deploy anchor nodes in the airport floor to enable accurate tracking of victims tags in the airport. This allows later arriving teams (such as ambulance personnel) to be able to quickly find the respective victims and bring them to the hospital.

### 5.X.2	Pre-conditions

Tom has a 5G enabled mobile phone UE with a USIM and a valid 5G subscription and registered as a first responder device which has ranging service.

The victim-tags are 5G enabled wearable monitoring devices such as smartwatches that can capture vitals and uses ranging services to measure distances.

Anchor nodes have higher power rating therefore, can range over longer distances.

### 5.X.3	Service Flows

- Tom triaged Victor as a red victim and attached a victim tag digitally marked as red onto his wrist

- Logistics team have arrived and started to deploy anchor nodes in the airport floor to enable accurate tracking of victims tags in the airport

- A treatment team has arrived at the scene, which is notified about the number of red victims and their priority to receive treatment based on triage score

- While arriving at the scene, the treatment team finds that there are multiple red victims that are scattered across the airport floors

- A treatment officer, who has a treatment device and a mobile UE, picks Jerry from the list of victims. The mobile UE of the treatment officer shows that Jerry is located in bay two, row four of the departure halls in the first floor of the airport building.

- The treatment officer is guided t to the exact location of Jerry in the airport building.

- Within seconds, Jerry is provided the first-aid treatment for his injuries. His bleeding is temporarily stopped and he is immediately transported to the nearest hospital in an helicopter ambulance.

- Thanks to the ranging services of the AmbCel 5G system, many lives are saved at an unfortunate time of this tragic incident.

### 5.X.4	Post-conditions

Tom and his crew can easily track the location of all the victims and time for re-triaging is significantly reduced.

### 5.X.5	Existing features partly or fully covering the use case functionality

### 5.X.6	Potential New Requirements needed to support the use case

- The 5G system shall be able to support ranging enabled UEs to discover the capabilities (e.g. distance and/or angle measurement) of other ranging enabled UEs.

- The 5G system shall be able to support ranging measurements of a ranging enabled UE to a set of stationary UEs.

- The 5G system shall be able to allow a ranging enable UE to determine if another ranging enabled UE is stationary or mobile, before and/or during ranging.

- The 5G system shall be able to support a ranging enabled UE to determine a local coordinate system, based on ranging measurements between multiple ranging enabled UEs.

* * * * End of Change 1 * * * *