3GPP TSG-SA1 Meeting #94-bis-e	S1-212027

# Online, , 4th Jul 2021 - 12th Jul 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.859 | CR | 0005 | rev | - | Current version: | 18.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Requirements consolidation - part 1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo Mobile Com. (Chongqing) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | 3GPP SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_PIN |  |  |  |  |  | Date: |  |  | 2021-06-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addition of consolidated requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of consolidated requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No consolidated requirements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.0 (new), 7.1, 7.2, 7.3, 7.4, 7.5, 7.6, 7.7, 7.9 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****FIRST CHANGE****

# 7	Potential Consolidated Requirements

This section provides Consolidated Potential Requirements for consideration to include in the normative specifications. The CPR’s have been grouped into different functional categories, each category contains a table that lists the original PR and any relationship to 3GPP TR 22.858 [6].

## 7.0	PIN Element Requirements

Table 7.0-1 – PIN Element Consolidated Requirements

| CPR No. | Potential Requirement |  |
| --- | --- | --- |
|  | Original Potential requirement No. | Potential requirement text |
| 7.0-X |  | A PIN Element shall have a unique identifier. |
| 7.0-X+1 | PR.5.1.5-7 | The 5G system shall allow PIN Elements to communicate when there is no connectivity between a PIN Element with Gateway Capability and a 5G network. For a Public Safety PIN licensed spectrum may be used for PIN direct communications otherwise unlicensed spectrum shall be used. |
| 7.0-X+2 | PR 5.2.6-1, PR 5.7.6-1 PR 5.10.6-1, | The 5G system shall support a mechanism for a PIN Element to be member of more than one PIN. When authorised for more than one PIN, the PIN Element shall be capable of concurrent communications in more than one PIN. NOTE: Concurrent operations is subject to PIN Element capabilities. |
| 7.0-X+3 | PR 5.3.6-1 | The 5G system shall support a PIN Element that can: - simultaneous unicast to multiple PIN Elements; and - multicast. NOTE: PIN Elements maybe in more than one PIN. |

## 7.1	Gateway

Table 7.1-1 – PIN Gateway Consolidated Requirements

| CPR No. | Potential Requirement |  |
| --- | --- | --- |
|  | Original Potential requirement No. | Potential requirement text |
| 7.1-X | PR 5.3.6-3 | A PIN Network may include at least one PIN Element with Gateway Capability. |
| 7.1-X+1 | PR 5.9.6-1, PR 5.13.6-3, PR 5.10.6-4 | The 5G system shall support access to the 5G network and its services for an authorized PIN Element (linked to a 3GPP subscription and provisioned with credentials) via one or more PIN Elements with gateway capability or directly. |
| 7.1-X+2 | PR 5.8.6-2 | When a PIN Element is using direct network connection for communications with other PIN Elements, the 5G system shall support an efficient way the PIN Element to discover other PIN Elements and their capabilities when those other PIN Elements use PIN direct connections for communications. |

## 7.2	Service Discovery

Table 7.2-1– PIN Service Discovery Consolidated Requirements

| CPR No. | Potential Requirement |  |
| --- | --- | --- |
|  | Original Potential requirement No. | Potential requirement text |
| 7.2-X | PR.5.1.5-10, PR 5.6.6-1, PR 5.8.6-1, PR 5.10.6-2, PR-5.10.6-6, PR 5.11.6-2, PR 5.13.6-1 | The 5G system shall support a service discovery mechanism for a PIN Element to discover capabilities of other authenticated/authorized PIN Elements: - PIN Element capabilities and status (e.g., relay, PEMC, PEGC); - Whether a PEGC has external data network connectivity; NOTE 1: External data network connectivity could be local break out or via the 5G core network. - Expected availability of Power (e.g., how long is the remaining battery life); - Supported applications/service (e.g., UPNP); - Device manufacturer; - Security/encryption mechanisms available; - if a relay allows a PIN Element to reach the desired endpoint PIN Element based on the applications/service being used; - connection types support by other PIN Elements (e.g. licensed spectrum PIN direct connection, non-licensed spectrum PIN direct connection); - Location of the PIN Element. NOTE 2: Any of the above may be done directly (depending on the discovery capabilities of the PIN elements) or via a PIN element with gateway capability or via a PIN with management capability e.g. when a PIN Element acting as a UE in the 5GS needs to discover a PIN Element or service behind a PEGC. |

## 7.3	Service Hosting

Table 7.3-1 – PIN Service Hosting Consolidated Requirements

| CPR No. | Potential Requirement |  |
| --- | --- | --- |
|  | Original Potential requirement No. | Potential requirement text |
| None | None | NOTE: The PRs 5.6.6-1, 5.6.6-2, 5.6.6-3, 5.6.6-4 are integrated with other potential requirements, on aspects related to discovery of, and connectivity to, application/services on PIN elements. Whether service hosting within PIN requires additional requirements will be determined during the normative work. |

## 7.4	Privacy and Security

Table 7.4-1 – PIN Privacy Consolidated Requirements

| CPR No. | Potential Requirement |  |
| --- | --- | --- |
|  | Original Potential requirement No. | Potential requirement text |
| TBD | TBD | TBD |
| 7.4-X | PR 5.6.6-4, PR 5.7.6-7, PR 5.11.6-4, PR 5.5.6-1 | The 5G system shall be able to provide secure communications between two or more PIN Elements in a PIN. |
| 7.4-X+1 | PR 5.8.6-3, PR 5.8.6-4 | The 5G system shall support a mechanism to mitigate repeated and unauthorized attempts to access PIN Elements (e.g. mitigate a malicious flood of messages). |
| 7.4-X+2 | PR 5.1.5.-6 | The 5G system shall support a PIN Element using non operator managed credentials (e.g. provided by a third party) for performing communications within the PIN when those communications use PIN direct connections. |
| 7.4.X+4 | PR 5.10.6-3, PR 5.5.6-2 | These requirements will be captured in descriptive test in the normative phase. |
| 7.4-X+5 | PR-5.11.6-3 | The 5G system shall be able to support PINs with PIN Elements subscribed to more than 1 network operator. |

## 7.5	Direct Communications

Table 7.5-1 – PIN Direct Communications Consolidated Requirements

| CPR No. | Potential Requirement |  |
| --- | --- | --- |
|  | Original Potential requirement No. | Potential requirement text |
| 7.5-X | PR 5.2.6-3, PR 5.7.6-3, PR 5.1.5-4 | The 5G system shall support mechanisms to provision a PIN Element to use either licensed or unlicensed spectrum (e.g., when it has no connectivity to the 5G system). |
| 7.5-X+1 | PR 5.4.6-1 | The 5G system shall support mechanisms to aggregate, switch or split the service between non-3GPP RAT and PIN direct connections using licensed spectrum. |
| 7.5- X+2 | PR 5.7.6-5 | The 5G system shall be able to support concurrent use of both licenced spectrum and non-licensed spectrum PIN direct connectivity between PIN Elements. |
| 7.5- X+3 | PR 5.3.6-2 | The 5G system shall support service continuity when a PIN Element changes the communication path from one PIN Element to another PIN Element. The communication path between PIN devices may include both 3GPP and non-3GPP access. |

## 7.6	Connectivity - QoS - charging

Table 7.6-1 – PIN Connectivity, QoS and Charging Consolidated Requirements

| CPR No. | Potential Requirement |  |
| --- | --- | --- |
|  | Original Potential requirement No. | Potential requirement text |
| 7.6-1 | PR 5.6.6-2, PR 5.6.6-3 | The 5G system shall support a mechanism to manage QoS for communications between PIN Elements when supported by the access technology. |

## 7.7	PIN Management

Table 7.7-1 – PIN Provisioning Consolidated Requirements

| CPR No. | Potential Requirement |  |
| --- | --- | --- |
|  | Original Potential requirement No. | Potential requirement text |
| TBD | TBD | TBD |
| 7.7-X | PR.5.1.5-8 | The 5G system shall support a PIN to contain one or more PIN Elements with Management Capability. |
| 7.7-X+1 | PR 5.1.5-1, PR 5.2.6-2, PR 5.7.6-2, PR 5.13.5-2, PR 5.8.6-1a, 5.11.6-1, PR 5.11.6-7, PR 5.x.6-1 | The 5G system shall support mechanisms for a PIN User, network operator or authorized 3rd party to create and manage a PIN, including: - Authorizing/deauthorizing and adding and removing PIN Elements - Authorizing/deauthorizing and adding and removing PIN Elements with Management Capability - Authorizing/deauthorizing and adding and removing PIN Elements with Gateway Capability - Establishing duration of the PIN - Configure PIN Elements to enable service discovery of other PIN Elements - Authorize/deauthorise if a PIN Element can use a PEGC to communicate with the 5GS. - Authorize/deauthorise which PIN Element another PIN Element may communicate with or use as relay PIN Element. - Authorize/deauthorise which PIN Element when using the 5GS can perform service discovery of other PIN Elements in the PIN. - Configure a PIN Element for external connectivity e.g., Local Break Out (LBO) or via 5GS. NOTE: The authorization may include the consideration of the location and time of the PIN and its PIN elements. |
| 7.7-X+2 |  | The 5G system shall support mechanisms for a network operator to configure the following policies in a PIN: - Configure the connectivity type (e.g. licensed or unlicensed PIN direct connection) a PIN Element can use. |

## 7.8	Void

## 7.9	Charging

Table 7.8A-1 – Charging Consolidated Requirements

| CPR No. | Potential Requirement |  |
| --- | --- | --- |
|  | Original Potential requirement No. | Potential requirement text |
| 7.9-1 | PR.5.1.5-5 PR 5.4.6-2 | The 5G system shall support mechanisms to collect charging information on PIN Element communications (e.g., start and stop of communications, amount of data transmitted, radio resources used) when 3GPP access is used. |