3GPP TSG-SA1 Meeting #94e-bis	S1-212047

Electronic Meeting, July 04 – 12, 2021	(revision of S1-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.858 | CR | 0011 | rev | - | Current version: | 18.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove Editor’s Note and add Note on requirements E2E QoS Monitoring. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KPN |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_RESIDENT |  |  |  |  |  | Date: |  |  | 29-6-2021 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | An Editor’s Note on further study of a requirement for the use case on E2E QoS Monitoring, may be removed. Additionally, an extra note may be added to clarify E2E QoS in context of the requirement, stating that end-to-end QoS includes the customer premises network between PRAS and eRG and the (fixed) access network between the eRG and the 5G core network. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes include: - Remove Editor’s Note - Add note on end-to-end QoS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TR contains redundant Editor’s Note and there may be confusion about end-to-end QoS in the context of the requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 5.6	Use case on E2E QoS monitoring

### 5.6.1	Description

This use case assumes that multiple PRASs were already deployed in individual rooms behind an eRG, in order to provide better cellular coverage at home. This use case is to enable E2E QoS monitoring for the whole communication path, i.e., from/to a UE to/from the 5GC via a PRAS and an eRG.

Figure 5.6.1-1. E2E QoS monitoring

### 5.6.2	Pre-Conditions

The following pre-conditions and assumptions apply to this use case:

- Multiple Premises Radio Access Stations were deployed in individual rooms inside a residential home.

- The Premises Radio Access Station provides cellular access to UEs

-	The Premises Radio Access Station is connected to an Evolved Residential Gateway via wireline or wireless link

-	The Premises Radio Access Station is connected to the same 5G system (and has a subscription to the same 5G system) as the Premises Radio Access Station belongs to.

In particular, a Premises Radio Access Station is deployed in the living room.

### 5.6.3	Service Flows

Alice sits in the living room and her smartphone connects to the Premises Radio Access Station.

Alice is watching an interesting video on her smartphone. The smartphone downloads the video content in real time from the video application server in the cloud.

After a while, the download speed becomes slow and the video is stuck.

Alice makes a call to the network operator and reports the downgrade of the network performance.

The network operator checks the E2E QoS status and identifies there is heavy interference on the Premises Radio Access Station. The network operator reconfigures the Premises Radio Access Station and recovers the E2E QoS.

### 5.6.4	Post-Conditions

The download speed recovers normal and Alice could continue enjoy the video.

### 5.6.5	Existing features partly or fully covering the use case functionality

3GPP TS 22.261 [2] Section 6.23.2 "QoS monitoring".

The 5G system shall be able to provide information that identifies the type and the location of a communication error for the Premises Radio Access Station (e.g. the ID of this Premises Radio Access Station).

### 5.6.6	Potential New Requirements needed to support the use case

[PR. 5.6.6-001] The 5G system shall provide a mechanism for supporting real time E2E QoS monitoring for the data traffic path (i.e., from/to a UE to/from the 5GC via a Premises Radio Access Station and a Evolved Residential Gateway).

NOTE:	End-to-end QoS includes the customer premises network between PRAS and eRG and the (fixed) access network between the eRG and the 5G core network.