3GPP TSG-SA1 Meeting #94e-bis	S1-212063

Electronic Meeting, 5 – 12 July 2021	(revision of S1-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.858 | CR | 0019 | rev | - | Current version: | 18.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Use case on supporting inter-connection between CPNs in different networks |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | 3GPP SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Resident |  |  |  |  |  | Date: |  |  | 28th June 2021 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This new use case describes a scenario where a customer has two customer premises networks (CPNs) in different geographical areas. Each CPN has its own set of services provided by a given local application server which can be under the control of a 5G network operator, an authorized third party or an end-user. The local application servers interconnect through a 5G system to provide seamless access to devices and applications both inside one CPN and between the CPNs. It is important that the QoS is ensured in such a scenario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Interconnection of “islands” of local connectivity through a 5G network. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Control of QoS at interconnection of multiple CPNs through a 5G network is not possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X (new), 5.x.1 (new), 5.x.2 (new), 5.x.3 (new), 5.x.4 (new), 5.x.5 (new), 5.x.6 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * *   First Change   * * * *

## 5.x	Use case: Supporting inter-connection between CPNs in different networks

### 5.x.1	Description

This use case describes a scenario where a customer has two customer premises networks (CPNs) in different geographical areas (e.g. different countries). Each CPN connects through a separate 5G core network. Each CPN also has its own set of services provided by a given local application server which can be under the control of a 5G network operator, an authorized third party or an end-user. The devices and local application servers in each CPN interconnect through a 5G system to provide seamless access between the devices and applications both inside one CPN and between the CPNs. Figure 5.x.1-1 describes this use case where:

- a device (UE 2) in CPN 2 wants to utilize a service available in a local application server (Application Server 1) accessible via the eRG of CPN 1 (eRG 1); and

- a device (UE 1) in CPN 1 can access to another device (UE 2) in CPN 2.

Figure 5.x.1-1. Remote access between two customer premises networks.

The use case describes a scenario where the two CPNs are in in different countries, but different geographical areas served by different 5G core networks within a country is equally applicable.

### 5.x.2	Pre-conditions

The following pre-conditions and assumptions apply to this use case:

- A first local application server is attached to the 5G core network 1 via an evolved residential gateway (eRG 1).

- A Premises Radio Access Station (PRAS 1) is deployed in the first CPN and it is connected to the 5G core network 1 via eRG 1.

- A second local application server exists in another CPN that is deployed in another country.

- The second local application server is attached to the 5G core network 2 via eRG 2.

e)	A PRAS 2 is deployed in the second CPN and it is connected to the 5G core network 2 via eRG 2.

### 5.x.3	Service Flows

We consider a scenario where Tom has two houses, one in the UK and another house in Portugal. Tom established a CPN for each house and utilizes different services from different local application servers in those CPNs. Tom has merged the two CPNs in a way that the devices in one CPN is authorized to access devices/applications in the other CPN. The local application server in Tom’s house in the UK hosts certain augmented reality and video/image processing applications. Tom has interest in astronomical events and night sky photography. While he is in the UK, an important astronomical event will occur in Portugal. Thus, Tom wants to watch the event live in Portugal out of his remote location in the UK and take photos when this transient event occurs. He has already connected his camera to his CPN in Portugal prior to the event. As the astronomical event occurs, he starts receiving live camera streaming, and uses certain applications from his local application server in the UK to augment his experience and control in real-time the configuration of the camera back in Portugal.

As the camera is connected to the 5G core network 2 via the eRG 2, and that the video processing as well as the remote control of the camera configuration are done using applications in a local application server that is connected to 5G core network 1 via the eRG 1, the 5G system shall make sure that the eRG 1 via 5G core network 1 and eRG 2 via 5G core network 2 are connected in order to ensure low latency transport for the remote services going across eRG 1 and eRG 2.

### 5.x.4	Post-conditions

The eRG 1 and eRG 2 have established a connection via 5G core network 1 and 5G core network 2 to ensure seamless low latency transport between the two CPNs. Tom is enjoying services seamlessly across his two CPNs in the UK and in Portugal.

### 5.x.5	Existing features partly or fully covering the use case functionality

3GPP TS 22.261 [2] clause 6.5.2 Efficient user plane: Requirements

The 5G network shall maintain user experience (e.g. QoS, QoE) when a UE in an active communication moves from a location served by a Service Hosting Environment to:

-	another location served by a different Service Hosting Environment; or

-	another location served by an application server located outside the operator’s network, and vice versa.

The 5G network shall maintain user experience (e.g. QoS, QoE) when an application for a UE moves as follows:

-	within a Service Hosting Environment; or

-	from a Service Hosting Environment to another Service Hosting Environment; or

-	from a Service Hosting Environment to an application server located place outside the operator’s network, and vice versa.

The 5G network shall support configurations of the Service Hosting Environment in the network (e.g. access network, core network), that provide application access close to the UE's point of attachment to the access network.

The 5G system shall support mechanisms to enable a UE to access the closest Service Hosting Environment for a specific hosted application or service.

The 5G network shall enable instantiation of applications for a UE in a Service Hosting Environment close to the UE's point of attachment to the access network.

3GPP TS 22.261 [2] clause 6.7.2: Priority, QoS, and policy control: Requirements

The 5G system shall be able to support end-to-end (e.g. UE to UE) QoS for a service.

NOTE:	End-to-end QoS needs to consider QoS in the access networks, backhaul, core network, and network to network interconnect. The QoS between UE, PRAS and eRG is not covered yet.

The 5G system shall be able to support QoS for applications in a Service Hosting Environment.

### 5.x.6	Potential New Requirements needed to support the use case

[PR. 5.x.6-001] The 5G system shall support QoS for a UE connected via an eRG and an application in a local application server connected via another eRG in another CPN.

* * * *   End of Changes   * * * *