3GPP TSG-SA WG1 Meeting #94bis-e 	S1-212083

Electronic Meeting, 5 – 12 July 2021	(revision of S1-21xxxx)

Title:	Adding use case for remote UAV control through HD video in First Person View

Agenda Item:	2.6.1 FS_TACMM

Source:	China Mobile

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com)

Abstract: This paper proposes a use case for remote UAV control through HD video in First Person View. In this case, people who control the UAV is wearing a head-mounted display, and can send flight command i.e. C2 message, through control handle to UAV based on the real-time video. During UAV mobility, especially handover case, the target RAN may do not have enough resource, for C2 message flow, the target RAN is expected to send notification to AF(existing feature in R17 for GBR),but since the video(usually non-GBR) is quite important for UAV control, the video traffic flow is expected to be kept in the target RAN. This is a coordination processing for the two traffic flows.

5.X	Remote UAV control through HD video in First Person View

### 5.X.1	Description

In first person view mode, the flight control is further simplified and can allow more people to experience the fun of immersive flight. There are UAV products which can support the first person view UAV control, e.g. DJI FPV[x]

[X]https://store.dji.com/cn/guides/dji-fpv-unboxing/

In this use case, the UAV, control handle, head-mounted display with HD video are used together. People who control the UAV should put on the head-mounted display and based on the HD video to control the UAV flight. The head-mounted has ultra wide angle of view (e.g.150 degrees) and provide clear HD video with 120 fps.

### 5.X.2	Pre-conditions

People who control the UAV should put on the head-mounted display and put the control handle in hands.

### 5.X.3	Service Flows

1. The UAV access to the 5G network.

2. The UAV taking off and people can control the UAV around 10km in NLOS. People can see the UAV flight environment thought the head-mounted display.

3. The UAV flies from one 5G RAN to another 5G RAN (target RAN), but the target RAN radio source is limited.

4. Since the C2 message is guaranteed by 5G system with GBR (one possible solution to guarantee the C2 message), once the target RAN cannot support the GBR flow, the target RAN is still expected to continue the handover procedure in order to keep the UAV control continuity. And after the handover procedure the target RAN should evaluate whether current radio condition can support the GBR, if not, the target RAN should notify this to AF (existing solution in 3GPP TS 23.501).

At the same time, since the HD video in head-mounted display is quite important to the UAV control, even this video is carried on non-GBR flow, the target RAN should also keep this traffic flow to help the UAV control.

The relationship between C2 message and HD video in head-mounted display should be configured by UAV AF, therefore in the handover case, once the radio resource is limited, these two traffic flows can be guaranteed together.

### 5.X.4	Post-conditions

The UAV still under people’s control and can fly in the target RAN coverage. People can continue to experience the immersive flying.

### 5.X.5	Existing features partly or fully covering the use case functionality

Remote UAV controller through HD video requirements found in Unmanned Aerial System (UAS) in TS 22.125.

### 5.X.6	Potential New Requirements needed to support the use case

#### 5.1.6.1	KPIs for Remote UAV control through HD video in First Person View

| Traffic direction | Traffic types | Altitude AGL | Latency (ms) | Average data rate | Service area |
| --- | --- | --- | --- | --- | --- |
| ControllerUAV | Command and control signal | <300m | 20ms | 300Kbps UAV terminated | <10km |
| UAV Head-mounted display | Video | <300m | <40ms(810P/60fps) in high-definition mode <28ms (810P/120fps) in low latency mode | <50Mbps | <10km |

#### 5.1.6.2	Service requirements for Remote UAV control through HD video in First Person View

[PR 5.X.6-X] The 3rd party requirement about the co-ordinately processing for related multiple traffic flows should inform 5G system through network exposure capability.

[PR 5.X.6-X] 5G system shall be able to identify the traffic flows which should be processing co-ordinately.

[PR 5.X.6-X] 5G system shall be able to process the multiple traffic flows together during mobility, if these traffic flows are identified for processing co-ordinately, even if they have different QoS requirement.

- Use Case template ----------