3GPP TSG-SA WG1 Meeting #94-bis-e 	S1-212041

Electronic Meeting, 5 – 12 July 2021	(revision of S1-21xxxx)

Title:	RESIDENT – Consolidated requirements

Agenda Item:	7.11.1

Source:	KPN, Vivo Mobile Communications Ltd

Contact:	Toon(dot)Norp(at)tno(dot)nl

Abstract: This document proposes a list of consolidated requirements.

Discussion

Document contains consolidated requirements from the usecases.  For ease of referencing a table has been created, the table labels each consolidated requirement and provides a reference to with PR it was.  There is also a column with comments.

****Changes****

# 7	Consolidated requirements

## 7.1	Introduction

This section provides Consolidated Potential Requirements for consideration to include in the normative specifications.  The CPR’s have been grouped into different functional categories, each category contains a table that lists the original PR.

## 7.2	General

Table 7.2-1 – RESIDENT General Consolidated Requirements

| Potential Requirement |  |  |  |
| --- | --- | --- | --- |
| CPR No. | Original Potential requirement No. | Potential requirement text | Comments |
| [CPR.7.2-1] |  | Subject to local regulations, the 5G system shall support regulatory requirements for emergency calls, PWS and eCall for UEs connected to a CPN. |  |

Editor’s Note: the consolidated requirements above are only an indication of grouping of potential requirements in the different sections and as a basis for further work on consolidation. Further consolidation of these requirements is needed. This can imply that requirements are changed, merged or not accepted as consolidated requirement.

## 7.3	Gateway

Table 7.3-1 – RESIDENT Gateway Consolidated Requirements

| Potential Requirement |  |  |  |
| --- | --- | --- | --- |
| CPR No. | Original Potential requirement No. | Potential requirement text | Comments |
|  |  |  | Add to Comments: the evolution of eRG from 5G-RG will be covered in descriptive text. |
|  | PR 5.1.6-002 |  | Moved to QoS |
|  | PR 5.3.6-002 |  | Moved to QoS |
|  | PR 5.5.6-003 |  | Moved to security |
|  | PR 5.5.6-004 |  | Moved to security |
|  | PR 5.5.6-005 |  | Moved to security |
|  |  |  | Moved to privacy and security |
|  | PR 5.13.6-001 | The 5G system shall be able to support IP traffic offload within the CPN. NOTE: The priority of offload can be from default configuration, network or user. |  |
|  | PR 5.19.6-001 |  | This aspect is to be covered in descriptive text. |
|  | PR 5.19.6-002 | The 5G system shall support a mechanism for the network operator to provide policies to the eRG on which transport (e.g. wireless, cable, etc.) is best suited for different negotiated QoS levels. |  |
|  | PR 5.17.6-001 |  |  |
|  | PR. 5.17.6-002 | When the eRG has lost connection with the 5GC, the eRG shall allow existing communication using non-3GPP access within the CPN to continue. NOTE1: Setting up new communication sessions without eRG connection to the 5GC is only possible with non-3GPP provided credentials. | E/// should be rephrased, not talking about detection, but about no connection with the 5GC. |
|  | PR. 5.17.6-003 | When the CPN has lost connection with the 5GC and the PRAS is serving UEs under its coverage using: - licensed spectrum, the CPN may allow the PRAS to continue transmission using the licensed spectrum for an operator-defined maximum time period. - During this time period, if the PRAS continues transmission using licensed spectrum, the CPN shall allow existing communication using licensed spectrum within the CPN to continue. - At the end of this time period, the CPN shall deactivate the PRAS air interface. - If the time period is undefined, the CPN shall deactivate the PRAS air interface. - unlicensed spectrum, the CPN shall allow communication using unlicensed spectrum within the CPN to continue. NOTE1: In the requirement above, only data traffic exchange within the CPN is allowed to continue. NOTE2: Setting up new communication sessions without PRAS connection to the 5GC is only possible with non-3GPP provided credentials. Editor’s Note: It is FFS to add more descriptions to clarify in which assumptions this use case and requirements [PR. 5.17.6-002] and [PR. 5.17.6-003] apply. | E/// rephrase to what the PRAS does when not connected, not about who detects and what happens at that point HW replace timer with an operator specified mechanism KPN: new communications are not possible (as there is no core network to authenticate/authorize). Protocol time-outs make it impossible to have indefinite continuation of communication without additional signalling with the 5GC. Also this implies communication may end before the operator defined time period. |

Editor’s Note: the consolidated requirements above are only an indication of grouping of potential requirements in the different sections and as a basis for further work on consolidation. Further consolidation of these requirements is needed. This can imply that requirements are changed, merged or not accepted as consolidated requirement.

## 7.4	Service Discovery

Table 7.4-1– RESIDENT Service Discovery Consolidated Requirements

| Potential Requirement |  |  |  |
| --- | --- | --- | --- |
| CPR No. | Original Potential requirement No. | Potential requirement text | Comments |
|  | PR 5.2.6-008 |  | Nokia: tweaking needed HW what is discovered? HW: we need to have a requirement to support existing UPNP |
|  |  | The 5G system shall support a service discovery mechanism for a UE or device in a CPN to discover capabilities of other UEs or authenticated/authorized devices in the CPN, for example: - Capabilities and status (e.g., relay, eRG) - Whether an eRG has external data network connectivity - Expected availability of power (e.g., how long is remaining battery life) - Supported applications/services (e.g. similar to what is provided by UPNP) - Device manufacturer - Security/encryption mechanisms available - Connection types support by other UEs or devices (e.g. licensed spectrum direct connection, non-licensed spectrum direct connection) NOTE: service discovery can also use existing service discovery mechanism, e.g. UPNP. | Requirements from PIN applied to Resident China Unicom: rethink last requirement is this 3GPP |

Editor’s Note: the consolidated requirements above are only an indication of grouping of potential requirements in the different sections and as a basis for further work on consolidation. Further consolidation of these requirements is needed. This can imply that requirements are changed, merged or not accepted as consolidated requirement.

## 7.5	Application servers

Table 7.5-1 – RESIDENT Application servers Consolidated Requirements

| Potential Requirement |  |  |  |
| --- | --- | --- | --- |
| CPR No. | Original Potential requirement No. | Potential requirement text | Comments |
|  | PR 5.11.6-001 | The 5G system shall support applications on an Application Server connected to the Customer Premises Network. | HW: who provides the application KPN: Generalised the requirement Interdigital: assumption of discriptive text |
|  | PR 5.11.6-002 |  |  |
|  | PR 5.11.6-002 | Based on operator policy, application needs, or both, the 5G system shall support an efficient user plane path, modifying the path as needed when the UE moves or application changes location, between a UE in an active communication and: - an application in a Service Hosting Environment; or - an application server located in a customer premises network. | Replacing PR 5.11.6-002 with three requirements inspired by existing requirements in 22.261 for easy integration (basically only adding the yellow part) HW+IDC: modify figures in 5.10 and 5.11 |
|  | PR 5.11.6-002 | The 5G network shall maintain user experience (e.g. QoS, QoE) when a UE in an active communication moves from a location served by a Service Hosting Environment to: - another location served by a different Service Hosting Environment; or - another location served by an application server located in a customer premises network, and vice versa. | Replacing PR 5.11.6-002 with three requirements inspired by existing requirements in 22.261 for easy integration |
|  | PR 5.11.6-002 | The 5G network shall maintain user experience (e.g. QoS, QoE) when an application for a UE moves as follows: - from a Service Hosting Environment to an application server located in a customer premises network, and vice versa | Replacing PR 5.11.6-002 with three requirements inspired by existing requirements in 22.261 for easy integration |

Editor’s Note: the consolidated requirements above are only an indication of grouping of potential requirements in the different sections and as a basis for further work on consolidation. Further consolidation of these requirements is needed. This can imply that requirements are changed, merged or not accepted as consolidated requirement.

## 7.6	Identification, Privacy, and security

Table 7.6-1 – RESIDENT Privacy Consolidated Requirements

| Potential Requirement |  |  |  |
| --- | --- | --- | --- |
| CPR No. | Original Potential requirement No. | Potential requirement text | Comments |
|  | PR 5.2.6-009 | The 5G system shall ensure that communications to/from individual UEs in a CPN can be associated to a subscription by the 5G network. | Nokia: this requirement is vague, what is it trying to do? KPN: proposed rewording |
|  | PR 5.22.6-001 |  | This aspect is treated in descriptive text |
|  | PR 5.22.6-002 |  | Descriptive text will cover this. |
|  | PR 5.5.6-006 | The 5G system shall ensure an Evolved Residential Gateway provides user privacy protection for UEs that are using the Evolved Residential Gateway, including communication confidentiality, location privacy and identity protection. NOTE: Privacy protection should not block differentiated routing and QoS at the eRG for different destinations and services for the UE(s). | Nokia: this seems to fall into the same category as the User Authentication – it’s an existing capability that should be carried forward in a CPN and can be noted in text. KPN: Disagree with Nokia. Issue is that eRG is a customer premise entity and is not under full operator control |
|  | PR 5.2.6-006 | The 5G system shall ensure the Premises Radio Access Station (PRAS) does not compromise user privacy for UEs that are using the PRAS, including communication confidentiality, location privacy and identity protection. NOTE: Privacy protection should not block differentiated routing, QoS, and services for the UE(s). | Nokia: this seems to fall into the same category as the User Authentication – it’s an existing capability that should be carried forward in a CPN and can be noted in text. KPN: Disagree with Nokia. Issue is that PRAS is a customer premise entity and is not under full operator control |
|  | PR 5.5.6-003 | The 5G system shall support a mechanism to minimize security impact of communications using an eRG. | Moved from gateway section Combined with 5.5.6-004 |
|  | PR 5.5.6-004 |  | Moved from gateway section Combined with 5.5.6-003 |
|  | PR 5.5.6-005 | The 5G system shall enable the network operator associated with an Evolved Residential Gateway to control the security policy of an Evolved Residential Gateway. | Moved from gateway section Nokia: BBF responsibility based on working agreement with SA2 KPN: SA1 can write requirements that are implemented by other SDOs |
|  | PR 5.2.6-003 | The 5G system shall support a mechanism to minimize the security impact of communications via a PRAS. | Moved from PRAS section Merged with 5.2.6-004 |
|  | PR 5.2.6-004 |  | Moved from PRAS section Merged with 5.2.6-003 |
|  | PR 5.2.6-005 | The 5G system shall enable the network operator associated with the Premises Radio Access Station (PRAS) to control the security policy of the PRAS. | Moved from PRAS section Nokia: BBF responsibility based on working agreement with SA2 KPN: SA1 can write requirements that are implemented by other SDOs. PRAS security most likely not part of BBF because of radio interface |

Editor’s Note: the consolidated requirements above are only an indication of grouping of potential requirements in the different sections and as a basis for further work on consolidation. Further consolidation of these requirements is needed. This can imply that requirements are changed, merged or not accepted as consolidated requirement.

## 7.7	Direct Communications

Table 7.7-1 – RESIDENT Direct Communications Consolidated Requirements

| Potential Requirement |  |  |  |
| --- | --- | --- | --- |
| CPR No. | Original Potential requirement No. | Potential requirement text | Comments |
|  | PR 5.10.6-001 | The 5G system shall be able to minimize service disruption when a direct communication path between UEs is switched to an indirect communication path going through a PRAS and an eRG that are connected to the 5G network. | KPN: this PR is a modification of what was proposed by InterDigital. KPN: existing text in 22.278 (not 22.261): “According to operator policy a UE's communication path can be switched between an EPC Path and a ProSe Communication path and a UE can also have concurrent EPC and ProSe Communication paths.” Not the best requirement. However, including something specific for CPN only in 22.261 is confusing. Probably best covered in descriptive text Nokia: may be in descriptive text HW: split in two parts E/// is this needed? |

Editor’s Note: the consolidated requirements above are only an indication of grouping of potential requirements in the different sections and as a basis for further work on consolidation. Further consolidation of these requirements is needed. This can imply that requirements are changed, merged or not accepted as consolidated requirement.

## 7.8	Connectivity - QoS - charging

Table 7.8-1 – RESIDENT Connectivity, QoS and Charging Consolidated Requirements

| Potential Requirement |  |  |  |
| --- | --- | --- | --- |
| CPR No. | Original Potential requirement No. | Potential requirement text | Comments |
|  | PR 5.5.6-002 | The 5G system shall support routing efficiency for data traffic between two UEs through an Evolved Residential Gateway. Editor’s Note: This requirement can be merged during potential requirement consolidation. | Nokia: make clear that there is no operator network involved in user plane HW: will have a look at it |
|  | PR 5.4.6-001 | The 5G system shall support routing efficiency for data traffic between a UE and a non-3GPP device through an Evolved Residential Gateway. Editor’s Note: This requirement can be merged during potential requirement consolidation. | Nokia: make clear that there is no Core network involved |
|  | PR 5.6.6-001 PR 5.1.6-002 | The 5G system shall support end-to-end QoS for UEs connected to the eRG via a PRAS. NOTE3: End-to-end QoS includes the customer premises network between PRAS and eRG and the (fixed) access network between the eRG and the 5G core network. | Combine with PR5.1.6-002, PR5.3.6-001, PR5.10.6-002 in two simplified requirements. |
|  | PR 5.3.6-002 PR 5.10.6-002 | The 5G system shall support end-to-end QoS for UEs directly connected to the eRG via either 3GPP RAT or non-3GPP (R)AT. NOTE: End-to-end QoS includes the customer premises network between UE and eRG and the (fixed) access network between the eRG and the 5G core network. | Combine with PR5.1.6-001, PR5.6.6-001, PR5.10.6-002 in two simplified requirements. Moved from eRG E///: what can 3GPP do for non-3GPP access. May be only provide info on QoS Interdigital: this is the right (non solution) way to describe |
|  | PR 5.5.6-001 | The 5G system shall be able to provide QoS control for the communication path between a UE and an Evolved Residential Gateway via a Premises Radio Access Station. | Moved from PRAS |
|  |  |  | Already mentioned under gateways |
|  |  |  | Philips: is there a requirement for QoS between two UEs in a CPN |
|  | PR 5.2.6-007 | The 5G system shall be able to generate charging information that can differentiate between UEs connected to the Premises Radio Access Station (PRAS) and between backhaul for the PRAS and other data traffic over the same access. | Moved from PRAS |

Editor’s Note: the consolidated requirements above are only an indication of grouping of potential requirements in the different sections and as a basis for further work on consolidation. Further consolidation of these requirements is needed. This can imply that requirements are changed, merged or not accepted as consolidated requirement.

## 7.9	Provisioning

Table 7.9-1 – RESIDENT Provisioning Consolidated Requirements

| Potential Requirement |  |  |  |
| --- | --- | --- | --- |
| CPR No. | Original Potential requirement No. | Potential requirement text | Comments |
|  | PR 5.18.6-001 | The 5G system shall enable configuration and management of an eRG by both the network operator of the public (mobile) network the eRG is connected to and, within the boundaries defined by the network operator, by an Authorised Administrator. | Nokia; per the working agreement between SA2/BBF, provisioning of the eRG is a BBF responsibility KPN: SA1 can provide requirements that are implemented by other SDOs |
|  | PR 5.18.6-003 | The 5G system shall enable configuration and management of a PRAS by a network operator and, within the boundaries defined by the network operator, by an Authorised Administrator. Specifically the network operator shall be able to configure: radio settings pertaining to licensed spectrum. Specifically the Authorised Administrator shall be able to configurevisitor access (allowing all UEs, or allowing specific UEs only) Editor’s Note: it is FFS whether we need to specify other aspects that can be managed and whether we need to specify how the Authorised Administrator can do secure configuration and management. | Nokia: modified to include 5.7.6-002 and 5.12.6-002 KPN: some rewording to still mention visitor access or the option to allow all UEs. |
|  | PR 5.7.6-001 | The 5G system shall provide mechanisms for the network operator or an Authorised Administrator (e.g. a homeowner) to trigger remote provisioning of evolved residential gateways and for the network operator to perform remote provisioning of eRGs, which includes verification and configuration of evolved residential gateway identity and initial OA&M provisioning. | Nokia; per the working agreement between SA2/BBF, provisioning of the eRG is a BBF responsibility KPN: SA1 can provide requirements that are implemented by other SDOs. KPN: initial OA&M provisioning is a specific eRG requirement as the eRG is not operator owned |
|  | PR 5.7.6-002 | The 5G system shall provide mechanisms for the network operator or an Authorised Administrator (e.g. a homeowner) to trigger remote provisioning of Premises Radio Access Stations and for the network operator to perform remote provisioning of PRASs, which includes verification and configuration of Premises Radio Access Station identity and initial OA&M provisioning. | Nokia: covered by 5.18.6-003 above KPN: initial OA&M provisioning is a specific eRG requirement as the eRG is not operator owned |
|  | PR 5.12.6-002 |  | Nokia: covered by 5.18.6-003 above |

Editor’s Note: the consolidated requirements above are only an indication of grouping of potential requirements in the different sections and as a basis for further work on consolidation. Further consolidation of these requirements is needed. This can imply that requirements are changed, merged or not accepted as consolidated requirement.

## 7.10	Premises Radio Access Station

Table 7.10-1 – RESIDENT Premises Radio Access Station Consolidated Requirements

| Potential Requirement |  |  |  |
| --- | --- | --- | --- |
| CPR No. | Original Potential requirement No. | Potential requirement text | Comments |
|  | PR 5.1.6-001 | The 5G system shall enable the network operator to provide any 5G services to any 5G UE via a Premises Radio Access Station (PRAS) connected via an evolved Residential Gateway (eRG). NOTE1: The PRAS may also be co-located with the eRG | Nokia: note 1 should already be covered by 5.19.6-001, which is noted as existing capability. |
|  | PR 5.2.6-001 | The 5G system shall enable the network operator to provide any 5G services via a Premises Radio Access Station (PRAS) to any 5G UE with a valid subscription to the HPLMN associated with the PRAS. |  |
|  | PR 5.2.6-002 | The 5G system shall enable the network operator to provide any 5G services via a Premises Radio Access Station (PRAS) to any 5G UE with a valid subscription to any VPLMN that has a roaming agreement with the HPLMN. NOTE: Whether 5G UEs from VPLMNs in the same country as the HPLMN can use the PRAS is subject to regulatory policy on national roaming. |  |
|  |  |  | Moved to security |
|  |  |  | Moved to security |
|  |  |  | Moved to security |
|  |  |  | Does not fully combine with e.g. PR 5.12.6-001 and other requirements. Moved to security |
|  |  |  | Moved to charging |
|  |  |  | Moved to QoS |
|  | PR 5.12.6-001 | The 5G system shall provide means to provision which PRAS in a CPN a UE can connect to . | Nokia suggested simplification. |
|  | PR 5.12.6-003 | The 5G system shall minimize service disruption when a CPN communication path changes between two PRASes. |  |
|  | PR 5.14.6-001 | The 5G system shall be able to support PRAS sharing between multiple PLMNs. |  |
|  | PR 5.18.6-002 | The 5G System shall support PRAS that use licensed spectrum, use unlicensed spectrum, or can use both unlicensed and licensed spectrum. | Nokia: the 5G system can already support both licensed and unlicensed spectrum, this is not a new requirement. This could be indicated as descriptive text. KPN: This is specifically relevant for a PRAS, as what the Authorised Administrator versus the operator can do depends on whether licensed or unlicensed spectrum is used. Prefer to keep it as requirement. |
|  | PR 5.21.6-001 | The 5G system shall provide support for a network operator to authenticate a PRAS. Editor’s Note: Authentication and authorization in these requirements need to be clarified. | Intel: solving EN Nokia: this requirement needs clarification |
|  | PR 5.21.6-002 | The 5G system shall provide support for a network operator to authorize a PRAS for its use in a CPN. Editor’s Note: Authentication and authorization in these requirements need to be clarified. | Intel: solving EN Nokia: this requirement needs clarification |
|  | PR 5.21.6-003 | The 5G system shall support a secure mechanism to provide credentials to a PRAS. Editor’s Note: Authentication and authorization in these requirements need to be clarified. | Intel: solving EN Nokia: this requirement needs clarification |
|  | PR 5.21.6-004 |  | Intel: solving EN Nokia: this is covered by 5.18.6-003 in the priovision clause KPN: and by PR 5.7.6-002 |

Editor’s Note: the consolidated requirements above are only an indication of grouping of potential requirements in the different sections and as a basis for further work on consolidation. Further consolidation of these requirements is needed. This can imply that requirements are changed, merged or not accepted as consolidated requirement.

## 7.11	5G-LAN

Table 7.11-1 – RESIDENT 5G-LAN Consolidated Requirements

| Potential Requirement |  |  |  |
| --- | --- | --- | --- |
| CPR No. | Original Potential requirement No. | Potential requirement text | Comments |
|  | PR 5.8.6-001 | The 5G system shall be able to support large amounts of small 5G LAN-VNs targeting residential deployments. NOTE: Targeting residential requirements translate into millions of 5GLAN-VN per operator per country. These 5G LAN-VNs may contain between 10-50 devices per LAN |  |
|  | PR 5.8.6-002 | The 5G system shall support authorized 3rd parties to authorize/deauthorize UEs to be able to access a 5G LAN-VN. |  |
|  | PR 5.9.6-001 | The 5G system shall support the use of an evolved Residential Gateway to connect 5G devices from the 5G LAN VN it belongs to with non-3GPP devices on an in-home LAN. |  |
|  | PR 5.16.6-001 | The 5G system shall support interconnection of a 5G LAN-Virtual Network (5G LAN-VN) with a fixed IP VPN. Editor’s Note: This requirement may need to be further studied. Editor’s Note: Further potential new requirements to support the use case may be identified. | Nokia: interconnection is already supported by existing 5G system. KPN: checked 22.261, but not covered there |

Editor’s Note: the consolidated requirements above are only an indication of grouping of potential requirements in the different sections and as a basis for further work on consolidation. Further consolidation of these requirements is needed. This can imply that requirements are changed, merged or not accepted as consolidated requirement.

## 7.12	Broadcast Multicast

Table 7.12-1 – RESIDENT Broadcast Multicast Consolidated Requirements

| Potential Requirement |  |  |  |
| --- | --- | --- | --- |
| CPR No. | Original Potential requirement No. | Potential requirement text | Comments |
|  | PR.5.20.6-001 |  | Nokia: Covered by 5.1.6-001 and 5.2.6-001 KPN: 5.1.6-001 and 5.2.6-002 are related to PRAS, these are eRG requirements |
|  | PR.5.20.6-002 |  | Nokia: Covered by 5.1.6-001 and 5.2.6-001 KPN: 5.1.6-001 and 5.2.6-002 are related to PRAS, these are eRG requirements |
|  | PR.5.20.6-003 | Under operator control, an eRG, shall be able to efficiently deliver 5G multicast/broadcast services to authorized UEs and non-3GPP devices in the CPN. | Nokia: Covered by 5.1.6-001 and 5.2.6-001 KPN: 5.1.6-001 and 5.2.6-002 are related to PRAS, these are eRG requirements KPN: Propose that 5.20.6-003 is generic enough to cover also 5.20.6-001, 5.20.6-002, and 5.15.6-001. |
|  | PR. 5.15.6-001 |  | Nokia: Covered by 5.1.6-001 and 5.2.6-001 KPN: 5.1.6-001 and 5.2.6-002 are related to PRAS, these are eRG requirements |

Editor’s Note: the consolidated requirements above are only an indication of grouping of potential requirements in the different sections and as a basis for further work on consolidation. Further consolidation of these requirements is needed. This can imply that requirements are changed, merged or not accepted as consolidated requirement.