3GPP TSG-SA WG1 Meeting #94bis	    																S1-212034r5
Electronic Meeting, July 5-12, 2021                                       					 	(Revision of S1-21xxxx)

Title:	Updated proposal on consolidated requirements for FS_VMR

Agenda Item:	2.5.1

Source:	Qualcomm

Contact:	Francesco Pica
	fpica <at> qti.qualcomm.com

# Abstract: This document includes an updated proposal on consolidated potential requirements for FS_VMR, based on S1-94e progress and current TR 22.839 (v1.0.0).

# Introduction

This document includes an updated proposal of consolidated requirements, based on the initial discussion and proposal in S1-211502, postponed from S1-94e (same as S1-211248r5, noted). It captures the following parts and updates:

Rev0 (submitted to S-94bis):

1) - as baseline text, a copy of the proposed consolidated requirements from S1-211502 sec. 7;

2) - new text (see rev1 for track changes/highlights):

a - some of the requirements left TBDs in S1-211502 (Annex), proposed to be moved/added to the consolidated part; others are merged or left out (see annex below for details).

b - suggested clarifications/changes to the baseline requirements

c - minor edits (e.g. to the introduction part)

------------ Revisions during S1-94bis ----------------

Rev1 changes (see rev1 for track changes/highlights) - based on comments before/during call#1

- Add descriptive text in the introduction to clarify assumptions and aspects related to spectrum, single-hop, legacy UEs, and general gaps vs IAB

- Add one new requirement on network management (in the “other aspects”, see PR 6-6)

- Small update to PR.5.15.6-1

------------
Rev2&3 updates (with track changes) – based on comments after call#1

- r2: proposed removals and simplification from Huawei (see comments in 2034r1_Huawei)

- r3: add edits proposed by Ericsson (in 2034r2_Ericsson)

-----------

Rev4 updates (see green highlights) – based on comments during call#2

- way forward proposal on some controversial changes/removals in r3, i.e. proposing text/requirements to be kept/reworded.

-----------

Rev5 changes: (see highlights, plus QC_r5 marked changes)

- addressing (most) comments from 2034r4_Ericsson, and other comments on the reflector

# Proposal

A text proposal is provided below, to be included in Section 7 of TR 22.839. 
As reminder, all content represents NEW TEXT; as indicated above, current revision marks show only incremental changes compared to earlier versions of the doc (i.e. on top of rev1), to facilitate readability and focus review/discussion.

# 7	Consolidated Potential Requirements

------------------ START of TR text proposal (ALL NEW TEXT) ------------------

Introduction

The requirements below refer to a “mobile base station relay” (or mobile BS relay), which is a mobile base station acting as a relay between a UE and the 5G network, i.e. providing a NR access link to UEs and connected wirelessly (using NR) through a donor NG-RAN to the 5G Core. Such mobile BS relay is assumed to be mounted on a moving vehicle and serve UEs that can be located inside or outside the vehicle (or entering/leaving the vehicle).

NOTE: the radio link used between a mobile BS relay and served UEs, as well as between mobile BS relay and donor RAN,  is assumed to be NR-Uu; in that regard, it should be clear that a mobile BS relay is different than a UE relay (which uses instead a PC5-based link to provide indirect connection to remote UEs).

Few main underlying assumptions and notes (from the TR overview and gap analysis) are:

- use cases and requirements cover single-hop relay scenarios as baseline (multi-hop is not precluded);

- legacy UEs are supported;

- current stage-1 requirements (e.g. on wireless self-backhaul, in TS 22.261), as well as existing stage-2/3 functionalities and architecture options (e.g. IAB) do not assume or address full relay mobility (e.g. BS relays on board of moving vehicles), thus may not cover the identified new potential requirements, which are intended to be specific to mobile BS relays;

- The identified requirements do not intend to imply or exclude specific network/relay architectures and topology solutions (e.g. could be IAB based, or others).

General requirements

[PR 5.1-1] The 5G system shall support efficient operation of mobile base station relays.

[PR 5.1-2] The 5G system shall be able to support means, for a mobile network operator, to configure, provision and control the operation of a mobile base station relay, e.g. authorization, activation/deactivation, frequency bands, permitted geographic areas/locations or time periods.

[PR 5.4-1] The 5G system shall be able to support provisioning and configuration mechanisms to control UEs’ selection and access to a mobile base station relay, e.g. based on UE authorization, geographic or temporary restrictions, user preference (for manual selection)

[PR5.16.6-1] The 5G System shall be able to support mobile base station relays using a NR satellite access link to connect to RAN.

[PR-5.17-1] The 5G system shall be able to support RAN sharing between multiple PLMNs for UEs connected to the 5G network via mobile base station relays.

NOTE 1: the above requirement assumes both relay and (donor) RAN resources, including UE access link and relay backhaul link, are shared among operators.

[PR 5.10-1] The 5G system shall be able to configure and provision specific required QoS for traffic relayed via a mobile base station relay.

NOTE 2: QoS is intended as end-to-end QoS, i.e. covering both UE access and relay backhaul QoS.

[PR 5.18-1] The 5G system shall support means to expose monitoring information of a mobile base station relay (e.g. number of UEs connected, QoS) to an MNO’s authorized third-party, based on monitoring configuration requested by the third-party.

[PR.5.15.6-1] The 5G System shall be able to support configuration and connectivity of mobile base station relays for providing ad-hoc coverage and capacity extension in specific locations, e,g, for public safety scenarios.

[PR 5.21-1] The 5G system shall be able to provide means to optimize network behaviour to efficiently deliver data based on the mobility information (e.g., itinerary), known or predicted, of mobile base station relays.

[PR 5.20-1] The 5G system shall be able to support communication from/to users of one MNO (MNO-A) via mobile base station relays, where the traffic between the relay and the MNO-A network is transported using 5G connectivity (RAN and 5GC) provided by a different MNO (MNO-B).

NOTE 4: The 5G connectivity provided to the MNO-A relays by the different MNO (MNO-B) assumes a generic wireless backhaul transport, independent from the mobile BS relay functionalities.

Multi-link connectivity

[PR.5.13.6-2] The 5G system shall be able to support UEs connectivity to RAN using simultaneously, a link without mobile BS relay and a link via a mobile BS relay, or simultaneous links via different mobile BS relays.

NOTE 5: the above requirements cover scenarios were the two links (to the RAN) could be connected to the same or different RAN node(s), and assuming both relay and RAN belong to the same PLMN.

[PR 5.22.6-1] The 5G System shall be able to support mobile base station relays capable of using both NR satellite access and NR terrestrial access to connect to RAN, can be one access link at a time or simultaneously.

Mobility and Service Continuity

[PR 5.7-1] The 5G system shall be able to provide means to support efficient UE cell selection and cell reselection (between mobile BS relays or between relays and RAN) in the presence of mobile base station relays.

[PR 5.7-2] The 5G system shall be able to ensure end-to-end service continuity, in the presence of mobile base station relays

NOTE 9: the above requirement intends to cover different scenarios of UE mobility (e.g. UE moving between two mobile BS relays, or between macro RAN and BS relay) and BS relay mobility (e.g. BS relay moving between different donor RAN nodes).

Charging

[PR 5.10-1] The 5G system shall be able to identify and differentiate UEs’ traffic carried via a mobile base station relay (e.g. mounted on a vehicle) and collect charging information, including specific relay information (e.g. geographic location served by the relay).

Other aspects

[PR 6-1a] The 5G System shall support means for a mobile base station relay to have a certain subscription with a HPLMN, used to get access and connectivity to the HPLMN network (via a donor RAN).

[PR 6-1b] The 5G System shall support the ability of a base station relay to roam from its HPLMN into a VPLMN.

[PR 6-1c] The 5G system shall support mechanisms, for the HPLMN controlling a mobile base station relay, to enable/disable mobile relay operation if the relay (e.g. on a mobile vehicle) is roaming in a VPLMN.

[PR 6-1d] The 5G system shall support mechanisms to disable mobile relay operation by a VPLMN where a mobile base station relay is roaming to.

[PR 6-2] The 5G system shall be able to fulfil necessary regulatory requirements (e.g. for support of emergency services) when UEs access the 3GPP network via a mobile base station relay (e.g. mounted on a vehicle).

[PR 6-3] The 5G system shall be able to support priority services (e.g. MPS) when UEs access the 3GPP network via a mobile base station relay (e.g. mounted on a vehicle).

[PR 5.9-1]: The 5G system shall be able to support location services for the UEs accessing 5GS via a mobile base station relay (e.g. mounted on a vehicle).

[PR 6-4] The 5G system shall ensure that existing end-to-end 5G security between the UE and 3GPP network is unaffected when the UE accesses the 3GPP network via a mobile base station relay (e.g. mounted on a vehicle).

[PR 6-5] The 5G system shall be able to minimize radio interference possibly caused by mobile base station relays (e.g. when mounted on moving vehicles).

[PR 6-6] The 5G system shall minimize the impact of the presence of mobile base station relays (e.g. mounted on vehicles) on radio network management (e.g. through automatic neighbor cell list configuration)

-------------------------------- END of text proposal ------------------------------------

Annex [Other FFS requirements, from S1-211502 - Annex]

FFS Requirements proposed to be included in the consolidated part

Note: requirements below are proposed to be moved to the consolidated part – see above.

[PR.5.15.6-1] The 5G System shall be able to support dynamic deployment and configuration of one or more mobile base station relays (e.g. mounted on vehicles) for providing ad-hoc indoor coverage extension in specific locations.

[PR 5.8-1] The 5G system shall be able to provide a means to perform load balancing among mobile base station relays (e.g. mounted on vehicles).

NOTE: This requirement is intended to provide the capability for the 5G system (UEs/ mobile base station relays) to be able to optimize the load of network resources whenever possible

[PR 5.9-1]: The 5G system shall support providing location service for the UEs accessing to the 5GS network via a mobile base station relay (e.g. mounted on a vehicle).

[PR 5.21-1] The 5G system shall be able to provide a means to optimize network behaviour to efficiently deliver data based on the mobility information (e.g., itinerary), known or predicted, of the vehicle-mounted relays.

[PR 5.10-1] The 5G system shall be able to configure and provision specific required QoS for traffic relayed via a mobile base station relay, e.g. may be based on user’s subscription, geographical area, day/time of operation. relay type, itinerary/mobility, etc.

NOTE: QoS is intended as end-to-end QoS, i.e. covering both UE access and relay backhaul QoS.

[PR 5.18-1] The 5G system shall support means to expose monitoring information of a mobile base station relay (e.g. number of UEs connected, QoS, geographical location, time of data monitoring/collection, etc.) to an authorized third-party based on monitoring parameters configured by the authorized third-party (e.g. data to monitor/collect, periodicity/schedule of monitoring, etc.).

[PR 5.20-1] The 5G system shall be able to support communication from/to users of one MNO (MNO-A) via mobile vehicle base station relays, where the relayed traffic is transported to/from the MNO-A network using 5G connectivity (RAN and 5GC) provided by a different MNO (MNO-B).

NOTE 1: the above requirement includes support of necessary policies and provisioning for end-to-end user’s QoS, e.g. based on MNO-A user/UE’s subscription.

NOTE 2: The 5G transport and connectivity provided by the different MNO (MNO-B) assumes a generic wireless BS backhaul, independent from the BS relay functionalities.

[PR 5.22.6-1] The 5G System shall be able to support mobile base station relays using NR satellite access and using NR terrestrial access to connect to 5GC.

Note 1: The simultaneous satellite and terrestrial access links (from the relay) are assumed to carry traffic pertaining to different UEs.

[PR 5.22.6-2] The 5G System shall be able to support a mobile base station relay capable using both NR satellite access and NR terrestrial access with service continuity in the scenario when this mobile base station relay switch from using NR satellite access to using NR terrestrial access, and vice versa.

Remaining FFS Requirements, not included in the consolidated part

Note: Requirements below are proposed to bes merged with others or left out [see rapporteur’s suggestion/comment].

[PR-5.17-2] The 3GPP System shall support end-to-end service continuity for a UE connected to a shared RAN via a mobile base station relay (e.g. mounted on a vehicle), i.e. when there is a change between a shared RAN (e.g. inside the vehicle) and a non-shared RAN (e.g. outside a vehicle).

[Rapporteur’s comment/suggestion]: covered by general requirements on service continuity

[PR 5.19-2] The 5G system shall be able to support, for a UE simultaneously connected to RAN via different mobile base station relays (e.g. mounted on vehicles), service continuity during various mobility events, e.g. when the connected mobile BS relays and/or the macro donor node(s) change. 
[Rapporteur’s comment/suggestion]: covered by general requirements on service continuity

[PR 5.9-2]: The 5G system shall support providing location information to a requesting UE or other location entity, for UEs accessing the 5GS network via a mobile base station relay (e.g. mounted on a vehicle), considering e.g. specific location granularity, and efficient UE power consumption.
[Rapporteur’s comment/suggestion]: covered by general requirement on support of location services (5.9-1)

[PR.5.13.6-1] The 5G system shall be able to efficiently control UE idle mode mobility when mobile base station relays are used, e.g. to facilitate UEs connecting to the macro RAN (as opposed to a vehicle relay).
[Rapporteur’s comment/suggestion]: covered by 5.7-1

[PR 5.23.6-1] The 5G System shall be able to support mobile base station relays capable of connecting via both NR satellite access and NR terrestrial access, to select one access as a relay link between the mobile base station relays and the 5GC.
[Rapporteur’s comment/suggestion]: merged with 5.22.6-1

[PR 5.23.6-2] The 5G System shall be able to support mobile base station relays capable of connecting simultaneously via NR satellite access and NR terrestrial access as relay link to connect via both to 5GC at the same time.
[Rapporteur’s comment/suggestion]: merged with 5.22.6-1

[PR 5.23.6-3] The 5G System shall be able to support mobile base station relays capable of connecting in parallel via NR satellite and NR terrestrial access to add a second relay link to allow more UEs to access the network and e.g. to adapt to increased bandwidths demand for access via this mobile base station relay.
[Rapporteur’s comment/suggestion]: merged with 5.22.6-1

[PR 5.23.6-4] The 5G System shall be able to support mobile base station relays connected via both NR satellite and NR terrestrial access to remove one of two relay links to adapt to decreased bandwidths demand of UEs accessing via this mobile base station relay.
[Rapporteur’s comment/suggestion]: merged with 5.22.6-1

[PR 5.23.6-5] The 5G System shall be able to support mobile relay traffic load balancing across satellite and terrestrial access links, for mobile base station relays that use both NR satellite access and NR terrestrial access as relay link at the same time.
[Rapporteur’s comment/suggestion]: merged with 5.22.6-1