3GPP TSG-SA WG1 Meeting #94bis	    																S1-212034r7
Electronic Meeting, July 5-12, 2021                                       					 	(Revision of S1-21xxxx)

Title:	Updated proposal on consolidated requirements for FS_VMR

Agenda Item:	2.5.1

Source:	Qualcomm

Contact:	Francesco Pica
	fpica <at> qti.qualcomm.com

# Abstract: This document includes an updated proposal on consolidated potential requirements for FS_VMR, based current TR 22.839 (v1.0.0).

# Introduction

This document includes:

- A text proposal for the consolidated requirements (CPRs), to be included in Section 7 of TR 22.839, for SA1 discussion and approval.

# Proposal

A text proposal is provided below, to be included in Section 7 of TR 22.839.

------------------ START of TR text proposal --------------------

# 7	Consolidated Potential Requirements

Introduction

The requirements below refer to a “mobile base station relay” (or mobile BS relay), which is a mobile base station acting as a relay between a UE and the 5G network, i.e. providing a NR access link to UEs and connected wirelessly (using NR) through a donor NG-RAN to the 5G Core. Such mobile BS relay is assumed to be mounted on a moving vehicle and serve UEs that can be located inside or outside the vehicle (or entering/leaving the vehicle).

NOTE: the radio link used between a mobile BS relay and served UEs, as well as between mobile BS relay and donor RAN,  is assumed to be NR-Uu; in that regard, it should be clear that a mobile BS relay is different than a UE relay (which uses instead a PC5-based link to provide indirect connection to remote UEs).

Few main underlying assumptions and notes (from the TR overview and gap analysis) are:

- use cases and requirements cover single-hop relay scenarios as baseline (multi-hop is not precluded);

- legacy UEs are supported;

- current stage-1 requirements (e.g. on wireless self-backhaul, in TS 22.261), as well as existing stage-2/3 functionalities and architecture options (e.g. IAB) do not assume or address full relay mobility (e.g. BS relays on board of moving vehicles), thus may not cover the identified new potential requirements, which are intended to be specific to mobile BS relays;

- The identified requirements do not intend to imply or exclude specific network/relay architectures and topology solutions (e.g. could be IAB based, or others).

General requirements

CPR 001: The 5G system shall support efficient operation of mobile base station relays.

Editor’s Note: this requirement is for FFS

CPR 002: The 5G system shall be able to support means, for a mobile network operator, to configure, provision and control the operation of a mobile base station relay,.

CPR 003: The 5G system shall be able to support provisioning and configuration mechanisms to control UEs’ selection and access to a mobile base station relay,.

CPR 005: The 5G system shall be able to support RAN sharing between multiple PLMNs for UEs connected to the 5G network via mobile base station relays.

NOTE 1: the above requirement assumes both relay and (donor) RAN resources, including UE access link and relay backhaul link, are shared among operators.

CPR 006: The 5G system shall be able to configure and provision specific required QoS for traffic relayed via a mobile base station relay.

NOTE 2: QoS is end-to-end QoS, i.e. from UE to 5GC.

CPR 007: The 5G system shall support means to expose monitoring information of a mobile base station relay (e.g. number of UEs connected, QoS) to an MNO’s authorized third-party, based on monitoring configuration requested by the third-party.

Editor’s Note: this requirement is for FFS

CPR 009: The 5G system shall be able to provide means to optimize network behaviour to efficiently deliver data based on the mobility information (e.g., itinerary), known or predicted, of mobile base station relays.

CPR 010: The 5G system shall be able to support communication from/to users of one MNO (MNO-A) via mobile base station relays, where the traffic between the relay and the MNO-A network is transported using 5G connectivity (RAN and 5GC) provided by a different MNO (MNO-B).

NOTE 3: The 5G connectivity provided to the MNO-A relays by the different MNO (MNO-B) assumes a generic wireless backhaul transport, independent from the mobile BS relay functionalities.

Multi-link connectivity

CPR 011: The 5G system shall be able to support UEs connectivity to RAN using simultaneously, a link without mobile BS relay and a link via a mobile BS relay, or simultaneous links via different mobile BS relays.

NOTE 4: the above requirements cover scenarios were the two links (to the RAN) could be connected to the same or different RAN node(s), and assuming both relay and RAN belong to the same PLMN.

Mobility

CPR 013: The 5G system shall be able to provide means to support efficient UE cell selection and cell reselection (between mobile BS relays or between relays and RAN) in the presence of mobile base station relays.

Editor’s Note: this requirement is FFS.

CPR 014: The 5G system shall be able to ensure end-to-end service continuity, in the presence of mobile base station relays

NOTE 5: the above requirement intends to cover different scenarios of UE mobility (e.g. UE moving between two mobile BS relays, or between macro RAN and BS relay) and BS relay mobility (e.g. BS relay moving between different donor RAN nodes).

Charging

CPR 015: The 5G system shall be able to identify and differentiate UEs’ traffic carried via a mobile base station relay and collect charging information, including specific relay information (e.g. geographic location served by the relay).

Other aspects

CPR 016: The 5G System shall support means for a mobile base station relay to have a certain subscription with a HPLMN, used to get access and connectivity to the HPLMN network (via a donor RAN).

CPR 017: The 5G System shall support the ability of a base station relay to roam from its HPLMN into a VPLMN.

CPR 018: The 5G system shall support mechanisms, for the HPLMN controlling a mobile base station relay, to enable/disable mobile relay operation if the relay is roaming in a VPLMN.

CPR 019: The 5G system shall support mechanisms to disable mobile relay operation by a VPLMN where a mobile base station relay is roaming to.

CPR 020: The 5G system shall be able to fulfil necessary regulatory requirements (e.g. for support of emergency services) when UEs access the 3GPP network via a mobile base station relay.

CPR 021: The 5G system shall be able to support priority services (e.g. MPS) when UEs access the 3GPP network via a mobile base station relay.

CPR 022: The 5G system shall be able to support location services for the UEs accessing 5GS via a mobile base station relay.

Editor’s Note: this requirement is FFS.

CPR 023: The 5G system shall ensure that existing end-to-end 5G security between the UE and 3GPP network is unaffected when the UE accesses the 3GPP network via a mobile base station relay.

CPR 024: The 5G system shall be able to minimize radio interference possibly caused by mobile base station relays.

CPR 025: The 5G system shall minimize the impact of the presence of mobile base station relays on radio network management (e.g. through automatic neighbor cell list configuration).

-------------------------------- END of text proposal ------------------------------------