3GPP TSG-SA WG1 #94-e 	         S1-211152r2

Electronic Meeting, 10th – 20th May 2021

Title:	Proposal for 5GSEI consolidated requirements – KPI

Agenda Item:	7.9.1 (FS_5GSEI)

Source:	China Telecom, CEPRI-China

Contact:	 xiaxu@chinatelecom.cn

Abstract:  This paper proposes the consolidated KPI requirements for 5GSEI TR22.867 V0.3.0.

# Introduction

The following potential requirements about KPIs for 5GSEI have been identified in the TR 22.867 v0.3.0. The corresponding consolidated potential requirements regarding KPI performance are proposed.

# Text proposal for TR 22.867 v0.3.0

************************Change Starts************************

…

# 7	Consolidated potential requirements and KPIs

## 7.2	Consolidated potential KPIs

Editor’s Note: 	This section summarizes the available KPI values from the various approved use cases based on v0.3.0. The KPI values in the tables may be corrected/modified aligned with the newly-added use-cases and the updates to the approved use-cases.

# Table 7.2-1 KPI Table of Periodic Communication Services

| # Use case | # Latency jitter | # Experienced data rate | # availability | # Transfer interval: target value | # Message size | # Service area | # Communication latency | # density |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # 5.1 # Distributed Energy Storage -monitoring | - | UL: > 16 M (urban), 640 M (rural) DL: > 100 kbit/s (see note 1) | # DL: >99.90% | # UL: 10 # ms | UL: 50 x 16 kbyte | # - | # DL:<10 ms # UL:<10 # ms | # >[x] x 10 # /km2 (urban), >[x] x 100 # /km2 (rural) # (storage node density, see note 2) |
| # 5.1 # Distributed Energy -Storage # Data collection | - | UL: > 128 k (urban), 10.4 M (rural) DL: > 100 kbit/s (see note 1) | # DL: >99.90% | # UL: 1000 ms | UL: 50 x 26 k DL: >100 kbyte | # - | # DL:<10 ms # UL:<1000 # ms | # >[x] x 10 # /km2 (urban), >[x] x 100 # /km2 (rural) # (storage node density, see note 2) |
| # 5.2 # advanced metering | - | UL:<2 Mbit/s DL:<1 M bit/s | # >99.99% | # - | - | # - | Accuracy fee control: < 100 ms (NOTE 3); # General information data collection: <3000 ms | # <10000/km2 (connection density, NOTE 4) |
| # 5.3 # Distributed Feeder Automation | <50 µs (see NOTE 5) | 2 Mbit/s to 10 M bit/s | # 99.999% | # - | - | # - | <10 ms # (see NOTE 6)- | # 78/km2 (connection density, NOTE 7,8) |
| # 5.12 # Distribution Intelligence – FLISR # High speed current differential protection (see NOTE9 ) | - | 2,5 Mbit/s | # > 99,999 % | # ≤ 1 ms | < 245 byte | # several km2 | # 5 ms end to end | # - |
|  |  | 1,2 Mbit/s |  | # ≤ 2 ms |  |  |  |  |
|  |  | 2,5 Mbit/s |  | # ≤ 1 ms |  |  |  |  |
|  |  | 1,2 Mbit/s |  | # ≤ 2 ms |  |  |  |  |
|  |  | 2,5 Mbit/s |  | # ≤ 1 ms |  |  |  |  |
|  |  | 1,2 Mbit/s |  | # ≤ 2 ms |  |  |  |  |
| # 5.15 # Distributed Energy Resources and Micro-Grids(see NOTE9 ) | - | 4,5 Mbit/s | # 99,9999 % | # ≤ 1 ms | 140 byte | # - | < 3 ms # end to end | # - |
|  |  | 5,4 Mbit/s | # 99,9999 % |  |  |  |  |  |
| # 5.22 # ensuring uninterrupted MTC service availability during emergencies | - | < 1 kbit/s per DER | # 99.9999 % | # 100 ms # (see NOTE10-11 ) | - | # - | # - | # - |

NOTE 1: This KPI is to require data rate in one Energy storage station which may via one or more 5G connections and via one or more 3GPP UE(s) at the same time. NOTE 2: It is used to deduce data volume in an area which has multiple energy storage stations. The data volume can be deduced through follow formula ( Current + other data) data rate per storage station * (Storage node density /km2) * (Active factor/km2) + video data rate per storage station * (Storage node density /km2). In general, the Active factor is 10% NOTE 3: The accuracy fee control latency here is for communication one way latency from 5G IoT device to backend system while the distance between them is no more than 40 km i.e. city range. The command implementation need 100 ms. NOTE 4: It is the typical connection density in today city environment. With the evolution from meter centralization collection to sockets in home directly collection, the connection density is expected to increase 5-10 times. NOTE 5: The latency jitter is required for the switch off between the active and standby communication links NOTE 6: It is the one way delay from a distributed terminal to 5G network. NOTE 7: When the distributed terminals are deployed along overhead line, about 54 terminals will be distributed along overhead lines in one square kilometre with the power load density is 20MW/km2. NOTE 8: When the distributed terminals are deployed in power distribution cabinets, and considering the power load density is 20 MW/km2, there are about 78 terminals in one square kilometre. NOTE 9: UE to UE communication is assumed. NOTE 10: Unless otherwise specified, all communication includes 1 wireless link (UE to network node or network node to UE) rather than two wireless links (UE to UE). NOTE 11: It applies to both UL and DL unless stated otherwise.

# Table 7.2-2 KPI Table of Aperiodic Communication Services

| # Use case | # Scenario | # Max allowed latency | # Experienced data rate | # End-to-end latency: maximum | # Communication service availability | # Message size | # Service Area | # Reliability | # Communication latency | # Storage node density # /km2 (Note2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # 5.1 # Distributed Energy Storage # Energy storage station: video | Rural | - | UL: >5 G DL: >100 k bit/s (see NOTE 1) | # - | # - | # - | # - | # DL: >99.90% | # DL:<10 ms # UL:<1000 ms | # >[x]*100 |
| # 5.12 # Distribution Intelligence – FLISR # Feeder automation # (NOTE 2) | - | 20 ms | - | - | # > 99,999 % | < 100 byte | # several km2 | # - | # - | # - |
| # 5.15 # Distributed Energy Resources and Micro-Grids(NOTE 2) | - | - | - | < 3 ms | # > 99,9999 % | 160byte | # - | # - | # - | # - |
| NOTE 1: The require data rate in one Energy storage station which may via one or more 5G connections and one or more 3GPP UE(s) at the same time. It can be calculated with following formula:12.5 Mbytes/s * 50(containers) * 8 = 5 Gbit/s NOTE 2: UE to UE communication is assumed. |  |  |  |  |  |  |  |  |  |  |

# Table 7.2-3 Clock Synchronization Service Performance Requirements

| # Use case | # User-specific clock synchronicity accuracy level [65] | # Number of devices in one Communication group for clock synchronisation | # Clock synchronicity requirement | # Service area | # Number of devices in one Communication group for clock synchronisation | # 5GS synchronicity budget requirement |
| --- | --- | --- | --- | --- | --- | --- |
| # 5.3 # Distributed Feeder Automation | - | - | - | - | - | <10 µs |
| # 5.12 # Distribution Intelligence – FLISR # feeder automation aperiodic deterministic communication | - | - | - | several km2 | ≤ 100/km2 | ≤ 10 µs |
| # 5.12 # Distribution Intelligence – FLISR # High speed current differential protection periodic deterministic communication | - | - | - | several km2 | ≤ 100/km2 | ≤ 10 µs |
| # 5.19 # Applications Using IEC 61850-9-2 Sampled Values # Smart Grid: Synchronicity between sync master and PMUs This range covers the extreme cases where the PTP clock in the end device uses 5G sync modem as direct time-source (1 µs) # The 5G sync modem acts as PTP GM or 5G sync modem provides PPS output to PTP GM at the top of the Ethernet based synchronization chain with up to 15 transparent clocks or 3 boundary clocks (250 ns). | 4 | Up to 100 UEs | <250 ns-1 µs [7] (see Note1) | < 20 km2 | - | - |
| # 5.19 # Applications Using IEC 61850-9-2 Sampled Values # Smart Grid: Power system protection in digital substation with merging units, line differential protection and synchronization | 4a | Up to 100 UEs | <10-20 µs [64] (see NOTE 1) | < 20 km2 |  |  |
| # 5.19 # Applications Using IEC 61850-9-2 Sampled Values # Smart Grid: Event reporting and Disturbance recording use-cases | 4b | Up to 100 UEs | <1 ms [64] (see NOTE 1) | < 20 km2 |  |  |
| NOTE 1: The clock synchronicity requirement refers to the clock synchronicity budget for the 5G system, as described in Clause 5.19.6.1. |  |  |  |  |  |  |