3GPP TSG-SA WG1 Meeting #94e 	S1-211093r03

Electronic Meeting, 10 – 20 May 2021	(revision of S1-21xxxx)

Title:	PIN – requirements consolidation

Agenda Item:	7.12.1

Source:	Vivo Mobile Communications Ltd

Contact:	Adrian(dot)Buckley(at)vivo(dot)com

Abstract: This document proposes adds consolidated requirements.

Discussion

Consolidates the requirements from the usecases.  Each row identifies the existing PR number and also if there is a corresponding PR is TR 22.858

****Changes****

# 7	Potential Consolidated Requirements

This section provides Consolidated Potential Requirements for consideration to include in the normative specifications.  The CPR’s have been grouped into different functional categories, each category contains a table that lists the original PR and any relationship to TR 22.858 [6].

## 7.1	Gateway

Table 7.1-1 – PIN Gateway Consolidated Requirements

| CPR No. |  | Potential Requirement |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Original Potential requirement No. |  | Potential requirement text |  | Equivalent CPR in TR 22.858 [6] |  |
| [CPR.7.1-1] |  | PR 5.3.6-3 |  | For a PIN it shall be possible to have more than one PIN Element with Gateway Capability. |  | PR 5.19.6-001 |  |
| [CPR.7.1-2] |  | PR 5.5.6-2 |  | A PIN Element with Gateway Capabilityshall be able to store and update a User Profile associated to PIN Element or an application running on or connected to a PIN Element behind the PIN Element with Gateway Capabilityand the User Profile shall include one or more pieces of the following information related to this application: - User Identifier - Specific service settings and parameters, e.g. active/inactive time, number of accesses, etc. - Authentication/authorization policy and access restriction policy required for the application, which are going to be used to authenticate/authorize a user for accessing to the application of the PIN Element. - Credential information, e.g. password for the authorized service, security keys for encryption/decryption, and hash algorithm for message digital signing, etc. |  |  |  |
| [CPR.7.1-3] |  | PR 5.5.6-1: |  | The 5G network shall enable support for an authorized UE to securely access the authenticated and authorized application running on or connected to a PIN Element behind a PIN Element with Gateway Capability. |  |  |  |
| [CPR.7.1-4] | PR 5.9.6-1 | The 5G system shall support access to the 5G network and its services for an authorized PIN device (linked to a 3GPP subscription and provisioned with credentials) via a gateway UE or directly via non-3GPP access. |  |

## 7.2	Service Discovery

Table 7.2-1– PIN Service Discovery Consolidated Requirements

| CPR No. | Potential Requirement |  |  |
| --- | --- | --- | --- |
|  | Original Potential requirement No. | Potential requirement text | Equivalent CPR in TR 22.858 [6] |
| [CPR.7.2-1] | PR 5.8.6-1 | The 5G system shall enable service discovery of PIN Elements (e.g. based on certain device applications) in PIN by PIN Elemets in the PIN or UEs via the public network. | PR 5.2.6-008 |
| [CPR.7.2-2] | PR 5.8.6-1a | The 5G system shall enable an authorized PIN user to configure which UEs connected to the public network can perform service discovery of PIN Element in a PIN. The 5G system shall support configuration per 5GLAN VN, per group of UEs, or per individual UE. |  |
| [CPR.7.2-3] | PR 5.8.6-2 | The PIN Element with Gateway Capability shall support optimization of service discovery of PIN Elements in a PIN by UEs on the public network, e.g. by reducing the amount and frequency of service discovery messages sent from PIN Elements. |  |

## 7.3	Service Hosting

Table 7.3-1 – PIN Service Hosting Consolidated Requirements

| CPR No. | Potential Requirement |  |  |
| --- | --- | --- | --- |
|  | Original Potential requirement No. | Potential requirement text | Equivalent CPR in TR 22.858 [6] |
| [CPR.7.3-1] | PR 5.6.6-1 | The 5G network shall enable a Service Hosting Environment hosted by a PIN Element with Gateway Capability. | PR 5.11.6-001 |
| [CPR.7.3-2] | PR 5.6.6-4 | The 5G system shall support a secure mechanism to access a service or an application in an operator's Service Hosting Environment hosted by a PIN Element with Gateway Capability. |  |
| [CPR.7.3-3] | PR 5.6.6-3 | The 5G system shall be able to support QoS for applications in a Service Hosting Environment hosted by a PIN Element with Gateway Capability. |  |

Editor’s Note:	CPRs in clause 7.3 are subject to email on email thread “[FS_PIN - 9] Resolving Editor's Notes on the Use Case UE accessing PIN applications hosted by gateways”

## 7.4	Privacy

Table 7.4-1 – PIN Privacy Consolidated Requirements

| CPR No. | Potential Requirement |  |  |
| --- | --- | --- | --- |
|  | Original Potential requirement No. | Potential requirement text | Equivalent CPR in TR 22.858 [6] |

## 7.5	Direct Communications

Table 7.5-1 – PIN Direct Communications Consolidated Requirements

| CPR No. | Potential Requirement |  |  |
| --- | --- | --- | --- |
|  | Original Potential requirement No. | Potential requirement text | Equivalent CPR in TR 22.858 [6] |
| [CPR.7.5-1] | PR 5.1.5-4 | The 5G system shall support mechanisms to provision a PIN to use PIN direct connection in non-operator managed spectrum when it has no connectivity to the 5G system. |  |
| [CPR.7.5-2] | PR 5.2.6-3 | The 5G system shall enable direct device communications between PIN Elements in a PIN to use licensed spectrum (under the control of a MNO) or between PIN Elements to use unlicensed spectrum (may be under the control of the MNO, or not). |  |
| [CPR.7.5-3] | PR 5.7.6-5 | The 5G system shall be able to support a PIN Element shall be able to concurrently use both operator managed and non-operator managed PIN direct connectivity with another PIN Element. |  |
| [CPR.7.5-4] | PR 5.4.6-1 | The PIN Element can act upon user and operator preferences to aggregate, switch or split the service between non-3GPP RAT and operator managed PIN direct connection services. | PR 5.10.6-001 |
| [CPR.7.5-5] | PR 5.4.6-2 | When operator managed PIN direct connections are used for inter PIN UE Element communications the 5G System shall be able to collect charging data, including data transmitted over the operator managed PIN direct connection between the PIN Elements, time, the operator managed resources used for the data transmission, e.g. operators managed spectrum and etc. |  |
| [CPR.7.5-6] | PR 5.4.6-3 | When PIN UE Element uses unlicensed spectrum for PIN direct connections for intra PIN UE device communications the 5G System may, subject to local/regional regulations and user consent, collect statistics data, including if 3GPP authentication was used. |  |

## 7.6	Connectivity - QoS - charging

Table 7.6-1 – PIN Connectivity, QoS and Charging Consolidated Requirements

| CPR No. | Potential Requirement |  |  |
| --- | --- | --- | --- |
|  | Original Potential requirement No. | Potential requirement text | Equivalent CPR in TR 22.858 [6] |
| [CPR.7.6-1] | PR 5.3.6-1 | For intra-PIN communications, a PIN Element shall be able to transmit media to one or more PIN Element at the same time. |  |
| [CPR.7.6-2] | PR 5.7.6-6 | The 5G system shall be able to support that a PIN Element can support concurrent communications with PIN Elements in more than one PIN. |  |
| [CPR.7.6-3] | PR 5.3.6-2 | A PIN Element shall support service continuity when a PIN Element changes the communication path from one PIN Element to another PIN Element. The communication path between PIN devices may include both 3GPP and non-3GPP access. |  |
| [CPR.7.6-4] | PR 5.7.6-7 | The 5G system shall be able to provide secure communications between PIN Elements in a PIN or across different PIN. |  |
| [CPR.7.6-7] | PR 5.1.5-3 | The PIN shall support fault tolerant operations. |  |

## 7.7	Provisioning

Table 7.7-1 – PIN Provisioning Consolidated Requirements

| CPR No. | Potential Requirement |  |  |
| --- | --- | --- | --- |
|  | Original Potential requirement No. | Potential requirement text | Equivalent CPR in TR 22.858 [6] |
| [CPR.7.7-1] | PR 5.1.5-1 | The 5G system shall support the ability for a network operator or authorised 3rd party to create a PIN. |  |
| [CPR.7.7-2] | PR 5.2.6-2 & PR 5.7.6-2 | The 5G system shall support a PIN Element being added or removed from a PIN. |  |
| [CPR.7.7-3] | PR 5.2.6-1 & PR 5.7.6-1 | The 5G system shall support that a PIN Element may be a member of more than one PIN. |  |
| [CPR.7.7-4] | PR 5.7.6-4 | The 5G system shall be able to provision PIN Elements that have been authorised to use that PIN with the necessary configuration parameters to use that PIN subject to MNO and local policies. |  |

## 7.8	Positioning

Table 7.8-1 – PIN Positioning Consolidated Requirements

| CPR No. |  | Potential Requirement |  |  |
| --- | --- | --- | --- | --- |
|  |  | Original Potential requirement No. | Potential requirement text | Equivalent CPR in TR 22.858 [6] |
| None identified |  |  |  |

## 7.9	KPIs

Editor’s note:	KPI still need to be completed at SA1#95e, this section will be updated then.